3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210861

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1868 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_UT_ReadAllowedNSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function, the assignment v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); ensures that the empty lines from the list are already removed. Thus, the actual string value would be available at v_ListOfLines[0], rather than v_ListOfLines[1]. However, in the current TTCN implementation of this function, the variable v_CharStringList is assigned the output of f_StringSplit(v_ListOfLines[1], {","});. This needs to be changed to f_StringSplit(v_ListOfLines[0], {","}); |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of assignment v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, assigned v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_UT_ReadAllowedNSSAI	3

# Corrections required

## function f_UT_ReadAllowedNSSAI

| Function name | f_UT_ReadAllowedNSSAI |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the assignment v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); ensures that the empty lines from the list are already removed. Thus, the actual string value would be available at v_ListOfLines[0], rather than v_ListOfLines[1]. However, in the current TTCN implementation of this function, the variable v_CharStringList is assigned the output of f_StringSplit(v_ListOfLines[1], {","});. This needs to be changed to f_StringSplit(v_ListOfLines[0], {","}); |
| Summary of change | Instead of assignment v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, assigned v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |
| TTCN module | Common\Common\UppwerTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); // @sic R5s210431 sic@ }<<SKIPPED CODE>>

After Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); // @sic R5s210431 sic@ }<<SKIPPED CODE>>