3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210862

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1869 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test case may fail during UECapability information procedure due to the problem highlighted in R5s210315 Due to the implementation of R5s210315 the default BSR configuration is used in the preamble and later reverted to use the standard L2 test case settings defined in TS 38.523-1 section 7.1.0. This is done by calling f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1). This update to BSR is not handled as the RRCSetup message is only updated for test cases that perform early contention resolution, for other cases the RRCSetup message is encoded and provided during cell configuration. Therefore the update of BSR configuration needs to be passed into f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS similar to other test cases (i.e. TC 7.1.1.3.4). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new variable v_MAC_CellGroupConfig. Updated the variable using f_NR_CellInfo_GetMAC_CellGroupConfig and passed as a parameter to f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS. Note : The issue was reported as part of R5s210484 and the implementation that resulted in 21wk24 does not solve the issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BSR configuration will not be correct and therefore results in TC failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required to test case 7.1.1.3.5	2

1.1	Correction to f_TC_7_1_1_3_5_NR5GC	2

# Corrections required to test case 7.1.1.3.5

## Correction to f_TC_7_1_1_3_5_NR5GC

| Function name | f_TC_7_1_1_3_5_NR5GC |
| --- | --- |
| Reason for change | The test case may fail during UECapability information procedure due to the problem highlighted in R5s210315. Due to the implementation of R5s210315 the default BSR configuration is used in the preamble and later reverted to use the standard L2 test case settings defined in TS 38.523-1 section 7.1.0. This is done by calling f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1). This update to BSR is not handled as the RRCSetup message is only updated for test cases that perform early contention resolution, for other cases the RRCSetup message is encoded and provided during cell configuration. Therefore the update of BSR configuration needs to be passed into f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS similar to other test cases (i.e. TC 7.1.1.3.4). |
| Summary of change | Introduced new variable v_MAC_CellGroupConfig. Updated the variable using f_NR_CellInfo_GetMAC_CellGroupConfig and passed as a parameter to f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS. Note : The issue was reported as part of R5s210484 and the implementation that resulted in 21wk24 does not solve the issue. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR var DRB_Identity v_DRBId1 ; var DRB_Identity v_DRBId2 ; //@sic R5-201519 sic@ var UInt_Type v_LCID1; var UInt_Type v_LCID2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; //@sic r5s190392 sic@ var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) DRB_ToAddModList v_DRBToAddModList; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; //@sic R5-201519 sic@ v_LCID1 := v_DRBId1 + 3; v_LCID2 := v_DRBId2 + 3; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID1,//@sic R5-201519 sic@ v_DRBId1, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(2, kBps0, 7)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID2,//@sic R5-201519 sic@ v_DRBId2, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(1, kBps0, 6)) ; v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; //@sic r5s190392 sic@ v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); //@sic r5s191054 sic@ v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config)}; //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig v_DRBToAddModList, v_SS_Drb_ConfigList ); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_1_3_5_NR_TestBody(v_DRBId1); f_NR_TestBody_Set(false);

After Change:

function f_TC_7_1_1_3_5_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR var DRB_Identity v_DRBId1 ; var DRB_Identity v_DRBId2 ; //@sic R5-201519 sic@ var UInt_Type v_LCID1; var UInt_Type v_LCID2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; //@sic r5s190392 sic@ var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; // //Init Cell parameters f_NR5GC_Init(NR_1); //f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ //removed //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; //@sic R5-201519 sic@ v_LCID1 := v_DRBId1 + 3; v_LCID2 := v_DRBId2 + 3; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID1,//@sic R5-201519 sic@ v_DRBId1, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(2, kBps0, 7)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB_LCID(v_LCID2,//@sic R5-201519 sic@ v_DRBId2, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(1, kBps0, 6)) ; v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NormalMode)}; //@sic r5s190392 sic@ v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); //@sic r5s191054 sic@ v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config)}; f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, v_MAC_CellGroupConfig, //MAC_CellGroupConfig //WA#WI=866608 v_DRBToAddModList, v_SS_Drb_ConfigList ); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId1, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_1_3_5_NR_TestBody(v_DRBId1); f_NR_TestBody_Set(false);