3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210865

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1871 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.7.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | f_TC_7_1_1_7_1_NR_TestBody needs to pass SPcell instead of SCell for next UL occasion calculation because Scell may be using different numerology and It will lead numerology mismatch in between the cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - f_TC_7_1_1_7_1_NR_TestBody needs to pass p_SPCell for next UL occasion calculation instead SCell at step 6. - f_TC_7_1_1_7_1_NR_TestBody needs to pass v_CellID_UL which is derived from the p_UL_CA==pc_UL_NR_CA_2CC for next UL occasion calculation instead SCell at step 15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will lead SS configuration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1.NR5GC, 7.1.1.7.1.2.NR5GC, 7.1.1.7.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	9

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_7_1_NR_TestBody |
| --- | --- |
| Reason for change | f_TC_7_1_1_7_1_NR_TestBody needs to pass SPcell instead of SCell for next UL occasion calculation because Scell may be using different numerology and It will lead numerology mismatch in between the cells. |
| Summary of change | f_TC_7_1_1_7_1_NR_TestBody needs to pass p_SPCell for next UL occasion calculation instead SCell at step 6. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ const integer const_UL_TBS := 384; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_ResourceAllocation_Type v_NR_ResourceAllocation2; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id); var NR_UplinkBWP_Type v_NR_UplinkBWP2 ; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); var boolean v_SI_Rcv := false; var boolean v_Data_Rcv := false; var B8_Type v_DeactivateSCell := '00000000'B; // var NR_CellId_Type v_CellID_UL := p_SPCell; timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1)); //@siclog "Steps 6" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { if (not v_SI_Rcv) { v_SI_Rcv := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } repeat; }; []DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); t_Watchdog.stop; }; }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@siclog "Steps 15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); ….. //@sic R5s210066 sic@ //disable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable)); };//End of f_TC_7_1_1_7_1_NR_TestBody

After Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ const integer const_UL_TBS := 384; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_ResourceAllocation_Type v_NR_ResourceAllocation2; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id); var NR_UplinkBWP_Type v_NR_UplinkBWP2 ; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); var boolean v_SI_Rcv := false; var boolean v_Data_Rcv := false; var B8_Type v_DeactivateSCell := '00000000'B; // var NR_CellId_Type v_CellID_UL := p_SPCell; timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1)); //@siclog "Steps 6" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SPCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { if (not v_SI_Rcv) { v_SI_Rcv := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } repeat; }; []DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); t_Watchdog.stop; }; }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@siclog "Steps 15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); ….. //@sic R5s210066 sic@ //disable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable)); }; //End of f_TC_7_1_1_7_1_NR_TestBody

## Change 2

| Function name | f_TC_7_1_1_7_1_NR_TestBody |
| --- | --- |
| Reason for change | f_TC_7_1_1_7_1_NR_TestBody needs to pass SPcell instead of SCell for next UL occasion calculation because Scell may be using different numerology and It will lead numerology mismatch in between the cells. |
| Summary of change | f_TC_7_1_1_7_1_NR_TestBody needs to pass v_CellID_UL which is derived from the p_UL_CA==pc_UL_NR_CA_2CC for next UL occasion calculation instead SCell at step 15. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ const integer const_UL_TBS := 384; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_ResourceAllocation_Type v_NR_ResourceAllocation2; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id); var NR_UplinkBWP_Type v_NR_UplinkBWP2 ; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); var boolean v_SI_Rcv := false; var boolean v_Data_Rcv := false; var B8_Type v_DeactivateSCell := '00000000'B; // var NR_CellId_Type v_CellID_UL := p_SPCell; timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1)); //@siclog "Steps 6" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { if (not v_SI_Rcv) { v_SI_Rcv := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } repeat; }; []DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); t_Watchdog.stop; }; }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@siclog "Steps 15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); ….. };//End of f_TC_7_1_1_7_1_NR_TestBody

After Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ const integer const_UL_TBS := 384; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_ResourceAllocation_Type v_NR_ResourceAllocation2; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id); var NR_UplinkBWP_Type v_NR_UplinkBWP2 ; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); var boolean v_SI_Rcv := false; var boolean v_Data_Rcv := false; var B8_Type v_DeactivateSCell := '00000000'B; // var NR_CellId_Type v_CellID_UL := p_SPCell; timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1)); //@siclog "Steps 6" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SPCell, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { if (not v_SI_Rcv) { v_SI_Rcv := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } repeat; }; []DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); t_Watchdog.stop; }; }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@siclog "Steps 15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 40); // 40 ms later f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); ….. }; //End of f_TC_7_1_1_7_1_NR_TestBody