3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210866

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1872 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The cell reselection priority is changed in SIB4 for Cell 3 at step 18. However, the value tag in SIB1 is not incremented, so the UE may not read the updated SIB contents and as a result, may not reselect to now higher priority cell 3 as intended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the function calls to increment the value tag in SIB1 after changing the SIB4 contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_2_1_TestBody |
| --- | --- |
| Reason for change | The cell reselection priority is changed in SIB4 for Cell 3 at step 18. However, the value tag in SIB1 is not incremented, so the UE may not read the updated SIB contents and as a result, may not reselect to now higher priority cell 3 as intended. |
| Summary of change | Added the function to increment the value tag in SIB1 after changing the SIB4 contents. |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { //@sic R5-197054 sic@ … … //@siclog "Step 18A" siclog f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7); //@siclog "Step 19" siclog //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell3); … … }

After Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { //@sic R5-197054 sic@ … …. //@siclog "Step 18A" siclog f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7); //increment value tags f_NR_IncrementValueTag(nr_Cell1, sibType4); f_NR_IncrementValueTag(nr_Cell3, sibType2); //@siclog "Step 19" siclog //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell3); … … }