3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210870

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1875 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR idle mode test case 6.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current TTCN implementation for steps 6A-6D uses the function f_NR5GC_RRC_ConnectedState3N_Steps5_8(), This function will include a SERVICE ACCEPT message with the RRCReconfiguration message at step 6C. This message should not include any piggybacked NAS message, according to specific message contents in 38.523-1 table 6.4.1.2.3.3-4A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement steps 6A-6D with no piggybacked NAS message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | The current TTCN implememtation for steps 6A-6D uses the function f_NR5GC_RRC_ConnectedState3N_Steps5_8(), This function will include a SERVICE ACCEPT message with the RRCReconfiguration message at step 6C. This message should not include any piggybacked NAS message, according to specific message contents in 38.523-1 table 6.4.1.2.3.3-4A. |
| Summary of change | Implement steps 6A-6D with no piggybacked NAS message |
| TTCN module | Inactive_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 R5-213163 sic@ … … SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // step 15 of the Registration procedure f_NR5GC_Registration_Complete(nr_Cell12); // @siclog "Step 6A-6D" siclog@ // Steps 5 to 8 of the generic test procedure in TS 38.508-1 Table 4.5.4.2-3. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @siclog "Step 7" siclog@ // The SS transmits an RRCRelease message with suspend. f_NR_RRC_InactiveDef(nr_Cell12, f_NR_CellInfo_GetPLMN (nr_Cell12)); … … } // end of f_TC_6_4_1_2_NR5GC_TestBod

After Change

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 R5-213163 sic@ … … SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // step 15 of the Registration procedure f_NR5GC_Registration_Complete(nr_Cell12); // @siclog "Step 6A-6D" siclog@ // Steps 5 to 8 of the generic test procedure in TS 38.508-1 Table 4.5.4.2-3. //REMOVED f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); f_NR_RRC_ActivateSecurity (nr_Cell12, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_RRCReconfigExistingDRBs(nr_Cell12, omit ); // @siclog "Step 7" siclog@ // The SS transmits an RRCRelease message with suspend. f_NR_RRC_InactiveDef(nr_Cell12, f_NR_CellInfo_GetPLMN (nr_Cell12)); … … } // end of f_TC_6_4_1_2_NR5GC_TestBod