3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210881

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1880 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If the UE configured with IMS PDN as default, then, relevant QoS rule/flow should be provided to AT command |  |  |  |  |  |  |  |  |
| Summary of change: |  | fl_TC_10_1_5_1_Body() updated to conditional at commands according to default DNN type at step1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.5.1 NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_5_1_Body() |
| --- | --- |
| Reason for change | If the UE configured with IMS as default, then, relevant QoS rule/flow should be provided to AT command |
| Summary of change | fl_TC_10_1_5_1_Body() updated to conditional at commands according to default DNN type at step1. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_10_1_5_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) QoS_Rules v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(1,6), 'FF'O, '100'B)}); // @sic R5s190673 sic@ // @sic R5-197259 sic@ //@sic R5s190987, R5-202585 sic@ v_QoS_Rules.iei :='7A'O; // @sic R5s190673 sic@ //@siclog "Steps 1" siclog@ f_UT_RequestModifyQosFlow(UT,1, f_Convert5G_QoSFlow_ForATCommand(cs_QoS_Flow('000001'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}))); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190673 sic@ if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo, -, -, -, -, v_QoS_Rules); // @sic R5s190673 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Release Complete SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { // @sic R5s190673 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } } //End of test body of TC_10_1_5_1

After Change

function fl_TC_10_1_5_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) QoS_Rules v_QoS_Rules; select (pc_APN_Default_Configuration) { case (internet){ v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(1,6), 'FF'O, '100'B)}); // @sic R5s190673 sic@ // @sic R5-197259 sic@ //@sic R5s190987, R5-202585 sic@ v_QoS_Rules.iei :='7A'O; // @sic R5s190673 sic@ //@siclog "Steps 1" siclog@ f_UT_RequestModifyQosFlow(UT,1, f_Convert5G_QoSFlow_ForATCommand(cs_QoS_Flow('000001'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}))); } case(ims) { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('02'O, int2bit(2,6), 'FF'O, '100'B)}); // @sic R5s190673 sic@ // @sic R5-197259 sic@ //@sic R5s190987, R5-202585 sic@ v_QoS_Rules.iei :='7A'O; // @sic R5s190673 sic@ //@siclog "Steps 1" siclog@ f_UT_RequestModifyQosFlow(UT,1, f_Convert5G_QoSFlow_ForATCommand(cs_QoS_Flow('000010'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}))); } } //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190673 sic@ if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo, -, -, -, -, v_QoS_Rules); // @sic R5s190673 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Release Complete SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { // @sic R5s190673 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } } //End of test body of TC_10_1_5_1