3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210883

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1881 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC MAC testcase 7.1.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the current TTCN implementation, there is a redundant piece of code where the log statement for Step 10 is repeated as well as the initialisation of the variable v_TimingInfo1. - Step 15 is used in the log statement for the preliminary pass verdict for Step 18 rather than using Step 18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the extra lines of code. - Corrected the Step number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Un-necessary initialisation of variable v_TimingInfo1 - TTCN would not be correct in following the Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.7.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_TC_7_1_1_3_7_ENDC_NR ()	3

# Corrections required

## Correction to function f_TC_7_1_1_3_7_ENDC_NR ()

| Function name | Function f_TC_7_1_1_3_7_ENDC_NR |
| --- | --- |
| Reason for change | - In the current TTCN implementation, there is a redundant piece of code where the log statement for Step 10 is repeated as well as the initialisation of the variable v_TimingInfo1. - Step 15 is used in the log statement for the preliminary pass verdict for Step 18 rather than using Step 18. |
| Summary of change | - Removed the extra lines of code. - Corrected the Step number. |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } }; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 200); // @siclog "Step 13" siclog@ // Reduce SS power level for NR Cell so as to cause a DL_Pathloss change at UE by 5dB.. f_NR_SetCellPowerList(v_CellPowerList_AtT1, cs_TimingInfo_NR(v_TimingInfo2),nr_Cell1); <<SKIPPED CODE>> //@siclog "Steps 17-18" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND; v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo1, v_TimingInfo2); if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15: TP5 failed, UE didn't send PHR after periodic PHR expiry"); } t_Watchdog.stop; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } }; /*REMOVED f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;*/ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 200); // @siclog "Step 13" siclog@ // Reduce SS power level for NR Cell so as to cause a DL_Pathloss change at UE by 5dB.. f_NR_SetCellPowerList(v_CellPowerList_AtT1, cs_TimingInfo_NR(v_TimingInfo2),nr_Cell1); <<SKIPPED CODE>> //@siclog "Steps 17-18" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND; v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo1, v_TimingInfo2); if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18: TP5 failed, UE didn't send PHR after periodic PHR expiry"); } t_Watchdog.stop; <<SKIPPED CODE>>