3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210886

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1884 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcases 7.1.2.3.5 and 7.1.2.3.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of 7.1.2.3.5 and 7.1.2.3.5a - Step 1 is transmitting AMD PDU ((SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) ) and Step 1A is transmitting one extra RLC PDU ( SN = W ) , it is proposed to merge these two steps , so that it can be done within the same loop structure - At step 1 , before sending the RLC PDUs , v_TimingDL is added with (20*p_innerLoop) ms duration which allows UL grant to start much earlier than DL data transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to merge step 1 and step 1A so that TTCN will generate and send RLC PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)) in the same loop structure - To be in align with Prose , v_TimingDL update before sending the RLC PDUs at step1 has to be removed , so that Step 2 ( UL Grant ) can be started 100ms after the first DL transmission happened in Step 1 as mentioned in the Prose |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.5.ENDC, 7.1.2.3.5.NR5GC, 7.1.2.3.5a.ENDC, 7.1.2.3.5a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR R5-215138 is submitted TTCN Change to handle Status PDU and AMD PDU is already addressed in R5s210778 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_7_1_2_3_5and5a	2

# Corrections required

## Correction to function fl_TC_7_1_2_3_5and5a

| Function name | fl_TC_7_1_2_3_5and5a |
| --- | --- |
| Reason for change | In the current TTCN implementation of 7.1.2.3.5 and 7.1.2.3.5a - Step 1 is transmitting AMD PDU ((SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) ) and Step 1A is transmitting one extra RLC PDU ( SN = W ) , it is proposed to merge these two steps , so that it can be done within the same loop structure - At step 1 , before sending the RLC PDUs , v_TimingDL is added with (20*p_innerLoop) ms duration which allows UL grant to start much earlier than DL data transmission |
| Summary of change | - It is proposed to merge step 1 and step 1A so that TTCN will generate and send RLC PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)) in the same loop structure - To be in align with Prose , v_TimingDL update before sending the RLC PDUs at step1 has to be removed , so that Step 2 ( UL Grant ) can be started 100ms after the first DL transmission happened in Step 1( as mentioned in the Prose ) |
| TTCN module | ttcn\develop\NR_TC_Common\7_1_2\RLC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { // @sic R5-203535 sic@ var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_RLC_SDUsize := 10; var integer v_NR_AM_Window_Size; var B1_Type v_Poll; var integer i; var integer j; var integer v_SlotsNumber; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotListRx; var template (value) NR_DRB_DataPerSlotList_DL_Array v_DataPerSlotArray; var integer v_VerdictFilter; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_TX; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_RX; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var NR_RLC_DataField_Type v_NR_RLC_Data; var template (value) NR_RLC_AMD_Data_Type v_NR_RLC_AMD_Data; timer t_WatchDog; if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_AM_Window_Size := tsc_NR_AM_Window_Size_18; v_RLC_SDUsize := 4; // @sic R5s190730, R5s190772, R5-197011, R5-203535 sic@ // TBS = RLC PDU data + RLC header without SO (3 octets)+ MAC header (2 octets) + MAC BSR or padding (2 octets) = 4 + 3 + 2 + 2 = 11 octets = 88 bits => TBS = 88 bits v_VerdictFilter := 4096; // @sic R5-187106 sic@ } else { //SN12 v_NR_AM_Window_Size := tsc_NR_AM_Window_Size_12; v_RLC_SDUsize := 4; // @sic R5-185083, R5s190730, R5s190772, R5-197011, R5s190906, R5-203535 sic@ // TBS = RLC PDU data + RLC header without SO (2 octets)+ MAC header (2 octets) + MAC BSR or padding (2 octets)= 4 + 2 + 2 + 2 = 10 octets = 80 bits => TBS = 88 bits v_VerdictFilter := 256; // @sic R5-187106 sic@ } v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(88, v_NR_UplinkBWP); v_SlotsNumber := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); // @siclog "Step 0" siclog@ // The SS does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // generate W RLC SDUs in lists of length iteration size // p_OuterLoop * p_InnerLoop = W for (j := 0; j < p_OuterLoop; j:= j + 1) { // generate p_InnerLoop RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, p_InnerLoop); // put those p_InnerLoop AMD PDUs into a list for transmission v_Poll := tsc_NR_P_NoPoll; // no indication of Poll bit usage in the prose for (i := 0; i < p_InnerLoop; i:= i + 1) { v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[i]); v_DataPerSlotList[i] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); } v_DataPerSlotArray [j] := v_DataPerSlotList; } // generate 1 more RLC SDU for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(0, -, v_NR_RLC_AMD_PDU_TX); // @sic R5s210093 sic@ // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); // @sic R5s210093 sic@ } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), {v_DataPerSlotList[0]})); // @sic R5s210093 sic@ // EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). // @siclog "Step 2A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotListRx := v_DataPerSlotArray [j]; for (i := 0; i < p_InnerLoop; i:= i + 1) { if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[0].AMD.SN18Bit.Data; // @sic R5s201490 sic@ } else { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[0].AMD.SN12Bit.Data; // @sic R5s201490 sic@ } if ((j == p_OuterLoop-1) and (i == p_InnerLoop-1)) {// SN = W-1 @sic R5s201490 sic@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit the Wth AMD PDU with the Poll bit set and with the contents of the SDU?. f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else {// SN < W-1 f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_NoPoll); } if ((((j*p_OuterLoop) + (i+1)) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2A" & " SN = " & int2str(((j*p_OuterLoop) + i)))}; } } // @siclog "Step 4" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell1); // @siclog "Step 5" siclog@ // Check: Does the UE transmit an AMD PDU within t-PollRetransmit/2? t_WatchDog.start(p_RLC_Rec.t_PollRetransmit/2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } [] t_WatchDog.timeout {} } // @siclog "Step 6" siclog@ // The SS transmits a STATUS PDU to acknowledge the W uplink AMD PDUs with SN=0 to SN=W-1. ACK_SN = W. f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @siclog "Step 7" siclog@ // Check: Does the UE transmit the (w+1)st AMD PDU with the Poll bit set and with the contents of the SDU? f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[0], tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 8" siclog@ // The SS transmits a STATUS PDU with ACK_SN = W+1. f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @siclog "Step 9" siclog@ // The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to ((2W+1 mod AM_Modulus) = 1) and the Polling bit set. // generate data for 1 more RLC SDU (w+2nd) v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); p_RLC_Rec.AM_TX_Next := 1; // @sic R5s190730, R5s190772, R5-197011 sic@ v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B((v_NR_AM_Window_Size + 1), substr(v_NR_RLC_DataList[0], 3, lengthof (v_NR_RLC_DataList[0]) - 3))); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_Data, tsc_NR_P_Poll); // @siclog "Step 10" siclog@ // Check: Does the UE transmit a STATUS PDU acknowledging W+1 SDUs? (ACK_SN = W+1). p_RLC_Rec.AM_RX_Next_Ack := v_NR_AM_Window_Size + 1; f_NR_RxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ // The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to W+1 and the Polling bit set. p_RLC_Rec.AM_TX_Next := v_NR_AM_Window_Size + 1; // @sic R5s190730, R5s190772 sic@ v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B((v_NR_AM_Window_Size + 1), substr(v_NR_RLC_DataList[0], 3, lengthof (v_NR_RLC_DataList[0]) - 3))); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_Data, tsc_NR_P_Poll); // EXCEPTION: Step 12 and 13 can happen in any order @sic R5s200148, R5s200164 sic@ // @siclog "Step 12" siclog@ p_RLC_Rec.AM_RX_Next_Ack := v_NR_AM_Window_Size + 2; // @sic R5s190730, R5s190772 sic@ // Check: Does the UE transmit a STATUS PDU acknowledging W+1 PDUs? (ACK_SN field = W+2)? // @siclog "Step 13" siclog@ // Check: Does the UE transmit an AMD PDU with the same data as received in the corresponding DL AMD PDU in step 11?. v_NR_RLC_AMD_PDU_RX := f_NR_RLC_Get_AMD_FullSDU_RX(p_RLC_Rec, tsc_NR_P_Poll, v_NR_RLC_Data); v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec); alt { //Expect RLC AMD PDCU and Status PDU in same or in multiple slots but in any order [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)} )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)} )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)} )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)} )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU), cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 and 13"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, { cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX), cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 and 13"); } } // @siclog "Step 14" siclog@ f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @sic R5s190730, R5s190772, R5-197011 sic@ }

After Change:

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { // @sic R5-203535 sic@ var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_RLC_SDUsize := 10; var integer v_NR_AM_Window_Size; var B1_Type v_Poll; var integer i; var integer j; var integer v_SlotsNumber; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotListRx; var template (value) NR_DRB_DataPerSlotList_DL_Array v_DataPerSlotArray; var integer v_VerdictFilter; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_TX; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_RX; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var NR_RLC_DataField_Type v_NR_RLC_Data; var template (value) NR_RLC_AMD_Data_Type v_NR_RLC_AMD_Data; timer t_WatchDog; if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_AM_Window_Size := tsc_NR_AM_Window_Size_18; v_RLC_SDUsize := 4; // @sic R5s190730, R5s190772, R5-197011, R5-203535 sic@ // TBS = RLC PDU data + RLC header without SO (3 octets)+ MAC header (2 octets) + MAC BSR or padding (2 octets) = 4 + 3 + 2 + 2 = 11 octets = 88 bits => TBS = 88 bits v_VerdictFilter := 4096; // @sic R5-187106 sic@ } else { //SN12 v_NR_AM_Window_Size := tsc_NR_AM_Window_Size_12; v_RLC_SDUsize := 4; // @sic R5-185083, R5s190730, R5s190772, R5-197011, R5s190906, R5-203535 sic@ // TBS = RLC PDU data + RLC header without SO (2 octets)+ MAC header (2 octets) + MAC BSR or padding (2 octets)= 4 + 2 + 2 + 2 = 10 octets = 80 bits => TBS = 88 bits v_VerdictFilter := 256; // @sic R5-187106 sic@ } v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(88, v_NR_UplinkBWP); v_SlotsNumber := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); // @siclog "Step 0" siclog@ // The SS does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // generate W RLC SDUs in lists of length iteration size // p_OuterLoop * p_InnerLoop = W for (j := 0; j < p_OuterLoop; j:= j + 1) { // generate p_InnerLoop RLC SDUs = PDCP PDUs for transmission // In case of 7.1.2.3.5 if block will be executed to generate 1 extra RLC packet + 2048 packets // In case of 7.1.2.3.5a if block will be executed to generate 1 extra RLC packet + 2048 packets for the last outerloop iteration if ( p_OuterLoop - j == 1) { v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, p_InnerLoop + 1); // put those p_InnerLoop AMD PDUs into a list for transmission v_Poll := tsc_NR_P_NoPoll; // no indication of Poll bit usage in the prose for (i := 0; i < (p_InnerLoop + 1); i:= i + 1) { v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[i]); v_DataPerSlotList[i] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); } v_DataPerSlotArray [j] := v_DataPerSlotList; } // In case of 7.1.2.3.5a else block will be executed to generate 2048 packets for the (Outerloop-1) iterations else { v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, p_InnerLoop); // put those p_InnerLoop AMD PDUs into a list for transmission v_Poll := tsc_NR_P_NoPoll; // no indication of Poll bit usage in the prose for (i := 0; i < (p_InnerLoop); i:= i + 1) { v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[i]); v_DataPerSlotList[i] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); } v_DataPerSlotArray [j] := v_DataPerSlotList; } } /* WA#7_1_2_3_5 REMOVED generate 1 more RLC SDU for transmission //To be removed and merged in above loop v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX);*/ // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); //To be removed as it makes DL RLC PDU Tx and UL Grant transmission not separated by 100ms DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); // @sic R5s210093 sic@ } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. //To be removed as handled in above loop /*DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotList));*/ // EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). // @siclog "Step 2A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotListRx := v_DataPerSlotArray [j]; for (i := 0; i < p_InnerLoop; i:= i + 1) { if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[0].AMD.SN18Bit.Data; // @sic R5s201490 sic@ } else { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[0].AMD.SN12Bit.Data; // @sic R5s201490 sic@ } if ((j == p_OuterLoop-1) and (i == p_InnerLoop-1)) {// SN = W-1 @sic R5s201490 sic@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit the Wth AMD PDU with the Poll bit set and with the contents of the SDU?. f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else {// SN < W-1 f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_NoPoll); } if ((((j*p_OuterLoop) + (i+1)) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2A" & " SN = " & int2str(((j*p_OuterLoop) + i)))}; } } // @siclog "Step 4" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell1); // @siclog "Step 5" siclog@ // Check: Does the UE transmit an AMD PDU within t-PollRetransmit/2? t_WatchDog.start(p_RLC_Rec.t_PollRetransmit/2.0); alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } [] t_WatchDog.timeout {} } // @siclog "Step 6" siclog@ // The SS transmits a STATUS PDU to acknowledge the W uplink AMD PDUs with SN=0 to SN=W-1. ACK_SN = W. f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @siclog "Step 7" siclog@ // Check: Does the UE transmit the (w+1)st AMD PDU with the Poll bit set and with the contents of the SDU? f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[i], tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 8" siclog@ // The SS transmits a STATUS PDU with ACK_SN = W+1. f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @siclog "Step 9" siclog@ // The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to ((2W+1 mod AM_Modulus) = 1) and the Polling bit set. // generate data for 1 more RLC SDU (w+2nd) v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); p_RLC_Rec.AM_TX_Next := 1; // @sic R5s190730, R5s190772, R5-197011 sic@ v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B((v_NR_AM_Window_Size + 1), substr(v_NR_RLC_DataList[0], 3, lengthof (v_NR_RLC_DataList[0]) - 3))); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_Data, tsc_NR_P_Poll); // @siclog "Step 10" siclog@ // Check: Does the UE transmit a STATUS PDU acknowledging W+1 SDUs? (ACK_SN = W+1). p_RLC_Rec.AM_RX_Next_Ack := v_NR_AM_Window_Size + 1; f_NR_RxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ // The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to W+1 and the Polling bit set. p_RLC_Rec.AM_TX_Next := v_NR_AM_Window_Size + 1; // @sic R5s190730, R5s190772 sic@ v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B((v_NR_AM_Window_Size + 1), substr(v_NR_RLC_DataList[0], 3, lengthof (v_NR_RLC_DataList[0]) - 3))); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_Data, tsc_NR_P_Poll); // EXCEPTION: Step 12 and 13 can happen in any order @sic R5s200148, R5s200164 sic@ // @siclog "Step 12" siclog@ p_RLC_Rec.AM_RX_Next_Ack := v_NR_AM_Window_Size + 2; // @sic R5s190730, R5s190772 sic@ // Check: Does the UE transmit a STATUS PDU acknowledging W+1 PDUs? (ACK_SN field = W+2)? // @siclog "Step 13" siclog@ // Check: Does the UE transmit an AMD PDU with the same data as received in the corresponding DL AMD PDU in step 11?. v_NR_RLC_AMD_PDU_RX := f_NR_RLC_Get_AMD_FullSDU_RX(p_RLC_Rec, tsc_NR_P_Poll, v_NR_RLC_Data); v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec); alt { //Expect RLC AMD PDCU and Status PDU in same or in multiple slots but in any order [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)} )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)} )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)} )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, {cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)} )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU), cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 and 13"); } [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, { cr_NR_RLC_PDU_AMD(v_NR_RLC_AMD_PDU_RX), cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12 and 13"); } } // @siclog "Step 14" siclog@ f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); // @sic R5s190730, R5s190772, R5-197011 sic@ }