3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210893

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1888 | rev | - | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.7.1.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS38.523-1 Table 7.1.1.7.1.1.3.2-2,TTCN needs to check the SDU data in step7(MAC PDU containing the loop back PDU corresponding to step 4) and step 16(MAC PDU containing the loop back PDU corresponding to step 14) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Check MAC PDU in step7 and step 16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will pass the test case even the SDU data transmitted does not comply with protocol. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections requireds	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk24  related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

- Corrections requireds

  - Change 1

| Function name | f_TC_7_1_1_7_1_1_NR5GC |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 7.1.1.7.1.1.3.2-2,TTCN needs to check the SDU data in step7(MAC PDU containing the loop back PDU corresponding to step 4) and step 16(MAC PDU containing the loop back PDU corresponding to step 14) |
| Summary of change | Check MAC PDU in step7 and step 16. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

SCellIndex p_SCellIndex ,

DRB_Identity p_NR_DRB_Id,

boolean p_UL_CA) runs on NR_BASE_PTC

{ //UE power headroom reporting / SCell activation / DL pathloss change reporting/

const integer const_UL_TBS := 384;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_MAC_SDU_Type v_EncodedRlcPdu2;

var NR_MAC_SDU_Type v_EncodedRlcPdu3;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4;

var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation2;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id);

var NR_UplinkBWP_Type v_NR_UplinkBWP2 ;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);

var boolean v_SI_Rcv := false;

var boolean v_Data_Rcv := false;

var B8_Type v_DeactivateSCell := '00000000'B; //

var NR_CellId_Type    v_CellID_UL :=    p_SPCell;

timer t_Watchdog := 5.0;

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep);

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) };

v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) };

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) };

v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) };

v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) };

v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit,  cs_NR_MAC_Padding_SubPDU('00000000'O)) };   // 1 byte sub headers +4byte Padding header= 40 bits

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP);

v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case

v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) };

v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) };

if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell

v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2);

v_CellID_UL := p_SCell;

}

t_Watchdog.start ;

//@siclog "Steps 3" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 4" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1));

//@siclog "Steps 6" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation);

//@siclog "Steps 8" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Steps 9" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400);  //

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@sic R5s210066 sic@

//enable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable));

//@siclog "Steps 5, 7" siclog@

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

t_Watchdog.stop;

};

};

//@siclog "Step 10" siclog@

t_Watchdog.start (1.0);

alt{

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR  ");

};

[] t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

}

};

t_Watchdog.start;

//@siclog "Steps 11" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

…

}; //End of f_TC_7_1_1_7_1_NR_TestBody

After change

functionf_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

SCellIndex p_SCellIndex ,

DRB_Identity p_NR_DRB_Id,

boolean p_UL_CA) runs on NR_BASE_PTC

{  //UE power headroom reporting / SCell activation / DL pathloss change reporting/

const integer const_UL_TBS := 384;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_MAC_SDU_Type v_EncodedRlcPdu2;

var NR_MAC_SDU_Type v_EncodedRlcPdu3;

var NR_MAC_SDU_Type v_EncodedRlcPdu4;

var NR_MAC_SDU_Type v_EncodedRlcPdu5;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL_Rec;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4;

var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation2;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id);

var NR_UplinkBWP_Type v_NR_UplinkBWP2 ;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);

var boolean v_SI_Rcv := false;

var boolean v_Data_Rcv := false;

var B8_Type v_DeactivateSCell := '00000000'B; //

var NR_CellId_Type    v_CellID_UL :=    p_SPCell;

timer t_Watchdog := 5.0;

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep);

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListUL_Rec:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,v_EncodedRlcPdu4) };

v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) };

v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B)));

v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) };

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) };

v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) };

v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) };

v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit,  cs_NR_MAC_Padding_SubPDU('00000000'O)) };   // 1 byte sub headers +4byte Padding header= 40 bits

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP);

v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case

v_SDU_SubPDUListUL := v_SDU_SubPDUListUL_Rec;

v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) };

if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell

v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2);

v_CellID_UL := p_SCell;

}

t_Watchdog.start ;

//@siclog "Steps 3" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 4" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1));

//@siclog "Steps 6" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation);

//@siclog "Steps 8" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Steps 9" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400);  //

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@sic R5s210066 sic@

//enable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable));

//@siclog "Steps 5, 7" siclog@

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

t_Watchdog.stop;

};

};

//@siclog "Step 10" siclog@

t_Watchdog.start (1.0);

alt{

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR  ");

};

[] t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

}

};

v_SDU_SubPDUListUL_Rec:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,(v_EncodedRlcPdu5)) };

v_SDU_SubPDUListUL := v_SDU_SubPDUListUL_Rec;

v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) };

t_Watchdog.start;

//@siclog "Steps 11" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

…

}; //End of f_TC_7_1_1_7_1_NR_TestBody