3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210906

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1746 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.7 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	6

2.5	Change 5	8

2.6	Change 6	17

3	Branches executed	19

4	Execution Log Files	19

4.1	HiSilicon Balong 5000	19

5	References	20

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.7 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.7.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

In addition to the below, the corrections of R5s210518 and R5s210592 are required and applied.

## Change 1

| Function name | f_TC_11_4_7_NR5GC_IMS1 |
| --- | --- |
| Reason for change | For each IMS MO call the received INVITE will have a different URI due to the different dialled digits |
| Summary of change | Build the expected CalleeUri for each MO call and pass to fl_IMS_NR5GC_MOSpeechCall() |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_7_NR5GC_IMS1()runs on IMS_PTC { // Initial PDU session - for e.g Registration and 'normal' calls var integer v_MaxMOCalls := 5; //5 IMS MO calls to be initiated var integer i; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); for (i:=0; i< v_MaxMOCalls; i:=i+1) { fl_IMS_NR5GC_MOSpeechCall(); } f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_4_7_NR5GC_IMS1()runs on IMS_PTC { // Initial PDU session - for e.g Registration and 'normal' calls var integer v_MaxMOCalls := 5; //5 IMS MO calls to be initiated var integer i; var CharStringList_Type v_IMS_CallNumber := {"120", "10", "1001", "12345", "12345" }; var template SipUrl v_OtherCalleeUri; var integer v_length := lengthof(px_IMS_CalleeUri); var integer v_index_colon := f_StringFindChar(px_IMS_CalleeUri, {":"}); var integer v_index_semicolon := f_StringFindChar(px_IMS_CalleeUri, {";"}); f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); for (i:=0; i< v_MaxMOCalls; i:=i+1) { v_OtherCalleeUri := f_SIP_BuildSipUri_RX(substr(px_IMS_CalleeUri, 0, v_index_colon+1) & v_IMS_CallNumber[i] & substr(px_IMS_CalleeUri, v_index_semicolon, v_length-v_index_semicolon)); fl_IMS_NR5GC_MOSpeechCall(v_OtherCalleeUri); } f_IMS_IpcanReleaseWithOptionalDeregistration(); }

## Change 2

| Function name | fl_IMS_NR5GC_MOSpeechCall |
| --- | --- |
| Reason for change | For each IMS MO call the received INVITE will have a different URI due to the different dialled digits |
| Summary of change | Add input parameter for the expected callee URI and pass to f_IMS_NR5GC_MOSpeechCall_Step1() |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function fl_IMS_NR5GC_MOSpeechCall () runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := f_IMS_NR5GC_MOSpeechCall_Step1(); f_IMS_NR5GC_MOSpeechCall_Step2ToEnd(v_InviteRequestWithSdp); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Call can now be released f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function fl_IMS_NR5GC_MOSpeechCall (template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := f_IMS_NR5GC_MOSpeechCall_Step1(p_OtherCalleeUri); f_IMS_NR5GC_MOSpeechCall_Step2ToEnd(v_InviteRequestWithSdp); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Call can now be released f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

## Change 3

| Function name | f_IMS_NR5GC_MOSpeechCall_Step1 |
| --- | --- |
| Reason for change | For each IMS MO call the received INVITE will have a different URI due to the different dialled digits |
| Summary of change | Add input parameter for the expected callee URI and pass to f_IMS_MOCallSetup_ReceiveINVITE_Common() |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_IMS_NR5GC_MOSpeechCall_Step1() runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { return f_IMS_MOCallSetup_ReceiveINVITE_Common(omit, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, pc_Preconditions); }

After Change

function f_IMS_NR5GC_MOSpeechCall_Step1(template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { return f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, pc_Preconditions); }

## Change 4

| Function name | f_TC_11_4_7_NR5GC |
| --- | --- |
| Reason for change | The test case body ends with the UE camped on NR Cell 3 |
| Summary of change | Perform postamble on NR cell 3 |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_7_NR5GC() runs on NR5GC_PTC { //Handling of Local and Extended emergency numbers / Mobility var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 f_NR5GC_Init(NR_1); ………… fl_TC_11_4_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_11_4_7_NR5GC() runs on NR5GC_PTC { //Handling of Local and Extended emergency numbers / Mobility var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 f_NR5GC_Init(NR_1); ……… fl_TC_11_4_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); }

## Change 5

| Function name | fl_TC_11_4_7_NR5GC_TestBody |
| --- | --- |
| Reason for change | - On mobility to NR cell 1 at steps 1 and 65, IP routing table should be updated - One redundant UT command is sent after step31 |
| Summary of change | - call f_IP_ChangeNRCell() on mobility to NR cell 1 Note : This function is defined in R5s210592. The other NR cell changes in this test case are using the function f_NR5GC_MobilityRegistration(). In these cases the IP routing table update is already covered by change 9 of R5s210592. - remove the redundant f_UT_RequestIMSCall |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_11_4_7_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var TrackingAreaCode v_TAC_1 := f_NR_CellInfo_GetTAC (nr_Cell1); var NAS_PlmnId v_PLMN_1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_1); var template (value) NG_MobileIdentity v_GUTI_1 := f_NG_GutiParameters2MobileIdentity (v_PLMN_1, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1)); var template (value) NG_TrackingAreaIdList v_TAIList_1 := cs_NG_TAIListNonConsecutive(v_PLMN_1, {bit2oct(v_TAC_1)}); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 //@siclog "Steps 1 - 5" siclog@ fl_EmergencyCallAndRelease(nr_Cell12, "120", "Step 2"); //@siclog "Steps 6 - 10" siclog@ fl_EmergencyCallAndRelease(nr_Cell12, "10", "Step 7"); //Cells 11 and 3 are still off f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 11" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (1, {'1001'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'12345'H}, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 12 - 16" siclog@ fl_EmergencyCallAndRelease(nr_Cell1, "1001", "Step 13"); //@siclog "Steps 17 - 21" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 18"); //@siclog "Steps 22 - 26" siclog@ fl_EmergencyCallAndRelease(nr_Cell1, "12345", "Step 23"); //@siclog "Steps 27 - 31" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 28"); f_UT_RequestIMSCall(UT, "10"); //Cells 12 and 3 are still off f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 32" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell11); //@siclog "Steps 33 - 37" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, "1001", "Step 34"); //@siclog "Steps 38 - 42" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, "12345", "Step 39"); //Cells 1 and 12 are still off f_NR_SetCellPower (nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 43" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell3); //@siclog "Steps 44 - 48" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "1001", "Step 45"); //@siclog "Steps 49 - 53" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "12345", "Step 50"); //Cells 1 and 11 are still off f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 54" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell12); //@siclog "Steps 55 - 59" siclog@ fl_MOSpeechCallAndRelease(nr_Cell12, "1001", "Step 56"); //@siclog "Steps 60 - 64" siclog@ fl_MOSpeechCallAndRelease(nr_Cell12, "12345", "Step 61"); //Cells 3 and 11 are still off f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 65" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (1, {'1001'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'12345'H}, '1'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell1); //Cells 11 and 12 are still off f_NR_SetCellPower (nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 66" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell3); //@siclog "Steps 67 - 71" siclog@ fl_MOSpeechCallAndRelease(nr_Cell3, "12345", "Step 67"); //@siclog "Steps 72 - 76" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "1001", "Step 73"); }

After Change

function fl_TC_11_4_7_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var TrackingAreaCode v_TAC_1 := f_NR_CellInfo_GetTAC (nr_Cell1); var NAS_PlmnId v_PLMN_1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_1); var template (value) NG_MobileIdentity v_GUTI_1 := f_NG_GutiParameters2MobileIdentity (v_PLMN_1, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1)); var template (value) NG_TrackingAreaIdList v_TAIList_1 := cs_NG_TAIListNonConsecutive(v_PLMN_1, {bit2oct(v_TAC_1)}); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 //@siclog "Steps 1 - 5" siclog@ fl_EmergencyCallAndRelease(nr_Cell12, "120", "Step 2"); //@siclog "Steps 6 - 10" siclog@ fl_EmergencyCallAndRelease(nr_Cell12, "10", "Step 7"); //Cells 11 and 3 are still off f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 11" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (1, {'1001'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'12345'H}, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); f_IP_ChangeNRCell(IP, nr_Cell1); //Step 6 f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 12 - 16" siclog@ fl_EmergencyCallAndRelease(nr_Cell1, "1001", "Step 13"); //@siclog "Steps 17 - 21" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 18"); //@siclog "Steps 22 - 26" siclog@ fl_EmergencyCallAndRelease(nr_Cell1, "12345", "Step 23"); //@siclog "Steps 27 - 31" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 28"); //REMOVED f_UT_RequestIMSCall(UT, "10"); //Cells 12 and 3 are still off f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 32" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell11); //@siclog "Steps 33 - 37" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, "1001", "Step 34"); //@siclog "Steps 38 - 42" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, "12345", "Step 39"); //Cells 1 and 12 are still off f_NR_SetCellPower (nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 43" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell3); //@siclog "Steps 44 - 48" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "1001", "Step 45"); //@siclog "Steps 49 - 53" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "12345", "Step 50"); //Cells 1 and 11 are still off f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 54" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell12); //@siclog "Steps 55 - 59" siclog@ fl_MOSpeechCallAndRelease(nr_Cell12, "1001", "Step 56"); //@siclog "Steps 60 - 64" siclog@ fl_MOSpeechCallAndRelease(nr_Cell12, "12345", "Step 61"); //Cells 3 and 11 are still off f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 65" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (1, {'1001'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'12345'H}, '1'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); f_IP_ChangeNRCell(IP, nr_Cell1); //Step 6 f_NR_RRCRelease(nr_Cell1); //Cells 11 and 12 are still off f_NR_SetCellPower (nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 66" siclog@ // Default Registration Accept is sent f_NR5GC_MobilityRegistration(nr_Cell3); //@siclog "Steps 67 - 71" siclog@ fl_MOSpeechCallAndRelease(nr_Cell3, "12345", "Step 67"); //@siclog "Steps 72 - 76" siclog@ fl_EmergencyCallAndRelease(nr_Cell3, "1001", "Step 73"); }

## Change 6

| Function name | TC_11_4_7_NR5GC |
| --- | --- |
| Reason for change | Current guard timer is too insufficient to complete this test case |
| Summary of change | Increase guard timer duration to 600s |
| TTCN module | NR5GC_Testsuite |
| MCC160 Comment |  |

Before Change

testcase TC_11_4_7_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Mobility var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_7_NR5GC()); v_IMS1.start(f_TC_11_4_7_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_7_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change

testcase TC_11_4_7_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Mobility var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(600); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_7_NR5GC()); v_IMS1.start(f_TC_11_4_7_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_7_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

# Branches executed

The test case was executed on NR band n41

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_7_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_7_PIXIT.xml

# References

| [1] | R5s210604 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.7 |
| --- | --- |