3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210907

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0705 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of f_IMS_SDP_AudioMediaAttributes_InitialAnswer_TX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The implementation of f_IMS_SDP_AudioMediaAttributes_InitialAnswer_TX is not according to 24.229-5 A.4.1 step 3: For the EVS codec the "bw" parameter is different for "br=13.2" and "br=5.9-13.2" |  |  |  |  |  |  |  |  |
| Summary of change: |  | "bw" parameter corrected in common implementation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect TTCN implementation remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_IMS_SDP_AudioMediaAttributes_InitialAnswer_TX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

function fl_SDP_SelectedEVSCodecParameters_Common(SDP_media_desc p_MediaDescription_InitialOffer) return SemicolonParam_List { var SemicolonParam_List v_FmtpParameterList_InitialOffer := f_SDP_MediaDescr_GetFirstEVSCodecParameters(p_MediaDescription_InitialOffer); var template (value) SemicolonParam_List v_FmtpParameterList; var charstring v_BrVal; var charstring v_BwVal; if (match(v_FmtpParameterList_InitialOffer, cr_EVS_Fmtp_AudioParamsBrBw("13.2", "swb"))) { v_BrVal := "13.2"; // 34.229-5 A.4.1/2 183 Session Progress, NOTE 8 v_BwVal := "swb"; } else { v_BrVal := "5.9-13.2"; // 34.229-5 A.4.1/2 183 Session Progress, NOTE 9 v_BwVal := "nr-swb"; } v_FmtpParameterList := { cs_EVS_Br(v_BwVal), cs_EVS_Bw("swb"), cs_Fmtp_ModesetParam("0,1,2") }; return valueof(v_FmtpParameterList); }

function f_SDP_SelectedEVSCodecParameters_AnswerTX(SDP_media_desc p_MediaDescription_InitialOffer) return template (value) SemicolonParam_List { var template (value) SemicolonParam_List v_FmtpParameterList; v_FmtpParameterList := fl_SDP_SelectedEVSCodecParameters_Common(p_MediaDescription_InitialOffer); v_FmtpParameterList := f_SemicolonParam_List_Add_TX(v_FmtpParameterList, cs_AMR_MaxRed); return v_FmtpParameterList; }

function f_IMS_SDP_AudioMediaAttributes_InitialAnswer_TX(SDP_media_desc p_MediaDescription_Offer, IMS_SDP_AudioCodec_Type p_AudioCodec, charstring p_FmtAudio) runs on IMS_PTC return SDP_attribute_list { var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SDP_attribute v_Attribute; var template (omit) SemicolonParam_List v_FmtpParameterList_Answer := omit; var SemicolonParam_List v_FmtpParameterList_Offer; var CharStringList_Type v_FmtpParametersToBoCopied; var charstring v_BwVal; select (p_AudioCodec) { // @sic R5-151968, R5-153999 sic@ case (EVS_16000){ if (f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5-206382: implementation of TC_7_4 sic@ v_FmtpParameterList_Answer := f_SDP_SelectedEVSCodecParameters_AnswerTX(p_MediaDescription_Offer); } else { // C.44 @sic R5-153999 sic@ v_Attribute := f_SDP_AttributeList_GetAttribute(p_MediaDescription_Offer.attributes, cr_SDP_Attribute_fmtp(p_FmtAudio)); v_BwVal := f_SIP_SemicolonParamList_GetParamValue(v_Attribute.fmtp.params.paramList, "bw"); /* @sic R5s170336/R5-172069 sic@ */ v_FmtpParameterList_Answer := {cs_EVS_Bw(v_BwVal), cs_EVS_BwSend(v_BwVal), cs_EVS_BwRecv(v_BwVal), cs_AMR_MaxRed}; /* @sic R5s160379: Additional Change by MCC160 sic@ @sic R5s170336/R5-172069: v_BwVal sic@ */ v_FmtpParametersToBoCopied := { "br", "br-send", "br-recv" }; v_FmtpParameterList_Offer := f_SDP_MediaDescr_GetCodecParameters(p_MediaDescription_Offer.attributes, p_FmtAudio); v_FmtpParameterList_Answer := f_SIP_SemicolonParamList_CopyParams(v_FmtpParameterList_Offer, v_FmtpParameterList_Answer, v_FmtpParametersToBoCopied); } v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(p_FmtAudio, valueof(v_FmtpParameterList_Answer)); // valueof to avoid compiler warning } case (AMR_16000){ // @sic R5-153950 sic@ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AMR_WB_AudioDef(p_FmtAudio); /* @sic R5s160379 change 2 sic@ */ } case (AMR_8000){ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AudioDef(p_FmtAudio); /* @sic R5-144693: new function f_SDP_MediaAttributes_AudioDef sic@ */ } } return valueof(v_SDP_MediaAttributes); }