3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210909

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1749 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.5.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In steps 13-18 and steps 66-71 a UE that is configured for pc_noOf_PDUsNewConnection > 0 may face an incomplete or incorrectly configured PDU session establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes specified below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE that is configured for pc_noOf_PDUsNewConnection >0 may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_5_1_3_Body |
| --- | --- |
| Reason for change | - UE configured for pc_noOf_PDUsNewConnection > 0 may not establish PDU session in steps 13-18 because theestablishment wont be triggered via UT commands. - Number of expected PDU connections for the f_NR5GC_PDUSessionEstablishment function needs to be set accordingly in case the UE is configured for establishing PDU session in new connection. |
| Summary of change | - PDU session establishment for UE configured for pc_noOf_PDUsNewConnection will now be triggered via UE commands. - Corrected number of expected PDU sessions for such UE. |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_5_1_3_Body() runs on NR5GC_PTC { … //@siclog "Step 13-18" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 19" siclog@

After Change:

import from NAS_5GC_Parameters all; //pc_noOf_PDUsNewConnection import from Parameters all; function fl_TC_9_1_5_1_3_Body() runs on NR5GC_PTC { … var integer v_ExpectedNumberOfPDUSessions := pc_noOf_PDUsSameConnection; … //@siclog "Step 13-18" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } //If the UE is configured to establish PDU session only in new connection then the f_NR5GC_PDUSessionEstablishment will result in no procedure at all. if (pc_noOf_PDUsNewConnection >0 ) { f_UT_RequestPDUSession(UT, 1, pc_APN_ID_Internet); v_ExpectedNumberOfPDUSessions := pc_noOf_PDUsNewConnection; } f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, v_ExpectedNumberOfPDUSessions, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 19" siclog@

## Change 2

| Function name | f_NR5GC_RRC_Idle_Steps16_20 |
| --- | --- |
| Reason for change | Currently the f_NR5GC_RRC_Idle_Extension inside the f_NR5GC_RRC_Idle_Steps16_20 doesn’t reflect any specific NSSAI configuration and always uses default. |
| Summary of change | NSSAI parameter passed in the f_NR5GC_RRC_Idle_Extension. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false) runs on NR5GC_PTC //Anritsu TTCN CR R5s210383 { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, -, -, -, -, -, -, -, -, -, p_ServiceType, p_Include_S_NSSAI); // @sic R5s210189 sic@ //Anritsu TTCN CR R5s210383 } }

After Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false) runs on NR5GC_PTC //Anritsu TTCN CR R5s210383 { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, -, -, -, -, -, -, -, p_S_NSSAI, -, p_ServiceType, p_Include_S_NSSAI); // @sic R5s210189 sic@ //Anritsu TTCN CR R5s210383 } }