3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210912

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1752 | rev | 1 | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 8.2.2.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | StarPoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Step6 is not finished yet according to the ttcn code, but the print shows Step 6 has passed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the step number to 5 after step5 finished. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect tips may mislead the users. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATSiwd-TTCN3-B2020-09_D20wk50related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Step6 is not finished yet according to the ttcn code, but the print shows Step 6 has passed |
| Summary of change | Change the step number to 5 after step5 finished. |
| TTCN module | RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var EUTRA_SecurityParams_Type v_EUTRA_Security_Params := f_EUTRA_Security_Get();

var CipheringAlgorithm_r12 v_EUTRA_CipheringAlg := v_EUTRA_Security_Params.AS_Ciphering.Algorithm;

var IntegrityProtAlgorithm_Type v_EUTRA_IntegrityAlg := v_EUTRA_Security_Params.AS_Integrity.Algorithm;

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

var template (value) octetstring v_NR_RBConfig;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (value) RadioBearerList_Type v_SrbList;

var integer v_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerId_Type v_RbId_DRB := cs_RadioBearerId_DRB(v_DRBId);

var SubFrameTiming_Type v_Timing;

…

// @siclog "Steps 4 - 5 " siclog@

// Modify MCG DRB to NR PDCP

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);  // @sic R5s200650, R5s201070 sic@

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1,

-, -,

{tsc_DRB8},

{cds_508_DRB_ToAddMod_Common(v_DRBId, omit, cs_508_RLC_Config_DRB_AM, cs_508_LogicalChannelConfig_DRB_AM)}); //@sic R5-198930, R5202627, R5-204439 sic@

v_ConfigDrbSS := { cs_RadioBearer_Reconfig_DRBwithNRPDCPonEUTRA(v_DRBId, cs_RLC_Configuration_AM, cs_MAC_Configuration_AM) };

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (omit,

v_NR_RBConfig,

v_Security_Params.SK_Counter);

v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); // @sic R5s200650 sic@

f_EUTRA_508RRC_IntraLTE_HO_IntraCell( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbList,

-, // p_NR_PDCP_HO

-, // p_Drb_ToReleaseList

-, // p_DRB_ToAddModList

v_RadioResourceConfigDedicated);  // @sic R5s200650 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Step 6" siclog@

// Test data on MCG DRB

f_EUTRA_CheckDataPath(eutra_Cell1, NR_PDCP, dedicatedBearer, "Step 6", v_RbId_DRB);

…

}

After change:

function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var EUTRA_SecurityParams_Type v_EUTRA_Security_Params := f_EUTRA_Security_Get();

var CipheringAlgorithm_r12 v_EUTRA_CipheringAlg := v_EUTRA_Security_Params.AS_Ciphering.Algorithm;

var IntegrityProtAlgorithm_Type v_EUTRA_IntegrityAlg := v_EUTRA_Security_Params.AS_Integrity.Algorithm;

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

var template (value) octetstring v_NR_RBConfig;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (value) RadioBearerList_Type v_SrbList;

var integer v_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerId_Type v_RbId_DRB := cs_RadioBearerId_DRB(v_DRBId);

var SubFrameTiming_Type v_Timing;

…

// @siclog "Steps 4 - 5 " siclog@

// Modify MCG DRB to NR PDCP

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);  // @sic R5s200650, R5s201070 sic@

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1,

-, -,

{tsc_DRB8},

{cds_508_DRB_ToAddMod_Common(v_DRBId, omit, cs_508_RLC_Config_DRB_AM, cs_508_LogicalChannelConfig_DRB_AM)}); //@sic R5-198930, R5202627, R5-204439 sic@

v_ConfigDrbSS := { cs_RadioBearer_Reconfig_DRBwithNRPDCPonEUTRA(v_DRBId, cs_RLC_Configuration_AM, cs_MAC_Configuration_AM) };

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (omit,

v_NR_RBConfig,

v_Security_Params.SK_Counter);

v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); // @sic R5s200650 sic@

f_EUTRA_508RRC_IntraLTE_HO_IntraCell( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbList,

-, // p_NR_PDCP_HO

-, // p_Drb_ToReleaseList

-, // p_DRB_ToAddModList

v_RadioResourceConfigDedicated);  // @sic R5s200650 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 6" siclog@

// Test data on MCG DRB

f_EUTRA_CheckDataPath(eutra_Cell1, NR_PDCP, dedicatedBearer, "Step 6", v_RbId_DRB);

…

}