3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210919

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4617 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.2.2.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2021-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per declaration of template cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1: template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1(RRC_TransactionIdentifier p_RRC_TI, ARFCN_ValueEUTRA p_EUTRA_DL_CarrierFreq, AllowedMeasBandwidth p_MeasurementBandwidth, PowerPrefIndicationConfig_r11.setup.powerPrefIndicationTimer_r11 p_PowerPrefIndicationTimer, //@sic R5-171442 sic@ ARFCN_ValueEUTRA_v9e0 p_EUTRA_DL_CarrierFreq_v9e0) According to ASN.1 definition: ARFCN-ValueEUTRA-v9e0 ::= INTEGER (maxEARFCN-Plus1..maxEARFCN2) In addition of ARFCN-ValueEUTRA-v9e0 being an OPTIONAL field, type ARFCN_ValueEUTRA_v9e0 will only allow earfcn values for bands greater than 64. Provoking a possible runtime error in case test case is run with a band < 64. This problem can be solved using an input parameter of type ARFCN_ValueEUTRA_r9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1 tempalte input parameter from type ARFCN_ValueEUTRA_v9e0 to txpe ARFCN_ValueEUTRA_r9. - Changed variable used as 4th input parameter to be of same type as per change 1, type ARFCN_ValueEUTRA_r9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to template cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1	4

1.2	Correction to function f_TC_8_2_2_6_2_EUTRA	5

# Corrections required

## Correction to template cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1

| Template name | cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1 |
| --- | --- |
| Reason for change | In addition of ARFCN-ValueEUTRA-v9e0 being an OPTIONAL field, type ARFCN_ValueEUTRA_v9e0 will only allow earfcn values for bands greater than 64. Provoking a possible runtime error in case test case is run with a band < 64. |
| Summary of change | Changed cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1 tempalte input parameter from type ARFCN_ValueEUTRA_v9e0 to txpe ARFCN_ValueEUTRA_r9. |
| TTCN module | LTE_A_R10_R11\8_2\RRC_ConnReconfig_A.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //TC8.2.2.6.2, Step1 template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1(RRC_TransactionIdentifier p_RRC_TI, ARFCN_ValueEUTRA p_EUTRA_DL_CarrierFreq, AllowedMeasBandwidth p_MeasurementBandwidth, PowerPrefIndicationConfig_r11.setup.powerPrefIndicationTimer_r11 p_PowerPrefIndicationTimer, //@sic R5-171442 sic@ ARFCN_ValueEUTRA_v9e0 p_EUTRA_DL_CarrierFreq_v9e0) := /* @status APPROVED (LTE_A_R10_R11) */ cs_RRCConnectionReconfiguration_v890(p_RRC_TI, cs_MeasConfig(omit, cs_MeasObjectToAddModList_1Entry(tsc_IdMeasObject_f1, cs_508_MeasObjectEUTRA_GENERIC(p_EUTRA_DL_CarrierFreq, p_MeasurementBandwidth)), omit, cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3, cs_508_ReportConfigEUTRA_A3), omit, cs_MeasIdToAddModList_1Entry(1, tsc_IdMeasObject_f1, tsc_IdReportConfig_A3), -,f_EUTRA_SS_InitialiseMeasGapConfig(),-,-,-, //@sic R5s190578 sic@ f_ReturnMeasObjectToAddModList_v9e0(p_EUTRA_DL_CarrierFreq_v9e0)), cs_RRCConnectionReconfiguration_PowerPrefIndicationConfig_Setup(p_PowerPrefIndicationTimer)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //TC8.2.2.6.2, Step1 template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1(RRC_TransactionIdentifier p_RRC_TI, ARFCN_ValueEUTRA p_EUTRA_DL_CarrierFreq, AllowedMeasBandwidth p_MeasurementBandwidth, PowerPrefIndicationConfig_r11.setup.powerPrefIndicationTimer_r11 p_PowerPrefIndicationTimer, //@sic R5-171442 sic@ ARFCN_ValueEUTRA_r9 p_EUTRA_DL_CarrierFreq_r9) := /* @status APPROVED (LTE_A_R10_R11) */ cs_RRCConnectionReconfiguration_v890(p_RRC_TI, cs_MeasConfig(omit, cs_MeasObjectToAddModList_1Entry(tsc_IdMeasObject_f1, cs_508_MeasObjectEUTRA_GENERIC(p_EUTRA_DL_CarrierFreq, p_MeasurementBandwidth)), omit, cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3, cs_508_ReportConfigEUTRA_A3), omit, cs_MeasIdToAddModList_1Entry(1, tsc_IdMeasObject_f1, tsc_IdReportConfig_A3), -,f_EUTRA_SS_InitialiseMeasGapConfig(),-,-,-, //@sic R5s190578 sic@ f_ReturnMeasObjectToAddModList_v9e0(p_EUTRA_DL_CarrierFreq_r9)), cs_RRCConnectionReconfiguration_PowerPrefIndicationConfig_Setup(p_PowerPrefIndicationTimer)); <<SKIPPED CODE>>

## Correction to function f_TC_8_2_2_6_2_EUTRA

| Function name | f_TC_8_2_2_6_2_EUTRA |
| --- | --- |
| Reason for change | In addition of ARFCN-ValueEUTRA-v9e0 being an OPTIONAL field, type ARFCN_ValueEUTRA_v9e0 will only allow earfcn values for bands greater than 64. Provoking a possible runtime error in case test case is run with a band < 64. |
| Summary of change | Changed variable used as 4th input parameter to be of same type as per change 1, type ARFCN_ValueEUTRA_r9. |
| TTCN module | LTE_A_R10_R11\8_2\RRC_ConnReconfig_A.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 1" siclog@ //The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1 & to //configure power preference indication. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1(v_RRC_TI, v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq, v_ChBandDependency_Cell2.AllowedMeasBandwidth, v_PowerPrefIndicationTimer, //@sic R5-171442 sic@ v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq_v9e0)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 1" siclog@ //The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1 & to //configure power preference indication. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_8_2_2_6_2_Step1(v_RRC_TI, v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq, v_ChBandDependency_Cell2.AllowedMeasBandwidth, v_PowerPrefIndicationTimer, //@sic R5-171442 sic@ v_Frequency_IE_Cell2.UL_DL_Earfcn.dl_CarrierFreq_r9)); <<SKIPPED CODE>>