3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210925

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1763 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for ENDC testcase 8.2.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - Adding v_ConfigDrbSS to v_SrbDrbList is not necessary as these two variables will eventually be combined in f_EUTRA_SS_SRBs_DRBs_Config - As in Change 1: In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - During Steps 4A and 4E, SS will resume sending of RLC ACK in EUTRA. There is some delay between resumption of sending RLC ACK and UE retransmission of RLC PDUs. To improve message flow and ensure un-ACKed RLC PDUs are properly ACKed, it is suggested to add some delay after resumption of sending RLC ACK in SS side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add full configuration of PDCP, RLC, and MAC layer for variable v_SrbDrbList. - Remove v_ConfigDrbSS in v_SrbDrbList - Add delay after Steps 4A and 4E. - Add option to include full configuration of RLC bearer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Overview	5

2	Corrections required	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit |
| --- | --- |
| Reason for change | - In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - Adding v_ConfigDrbSS to v_SrbDrbList is not necessary as these two variables will eventually be combined in f_EUTRA_SS_SRBs_DRBs_Config |
| Summary of change | - Add full configuration of PDCP, RLC, and MAC layer for variable v_SrbDrbList. - Remove v_ConfigDrbSS in v_SrbDrbList |
| TTCN module | RRCReconfig_ENDC_EUTRA |
| MCC160 Comment |  |

Before change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348, R5s210128 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

if (p_DuplicatePDU){

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_TestModeAckProhibit(Prohibit)),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, cs_RlcBearerRouting_NR(nr_Cell1)),

v_ConfigDrbSS[0]};

} else {

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB1_Def, cs_RX_AM_RLC_SRB1_Def), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, crs_RlcBearerRouting_None, cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

}

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

if (p_DuplicatePDU)

{

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

}

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}

After change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348, R5s210128 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

if (p_DuplicatePDU){

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_TestModeAckProhibit(Prohibit, cs_RLC_RbConfig_SRB), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

//v_ConfigDrbSS[0]};

} else {

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB1_Def, cs_RX_AM_RLC_SRB1_Def), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, crs_RlcBearerRouting_None, cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

}

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList,

-, -, -,

7

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

if (p_DuplicatePDU)

{

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

}

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}

## 2.2 Change 2

| Function name | cs_RLC_Configuration_TestModeAckProhibit |
| --- | --- |
| Reason for change | As in Change 1: In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. |
| Summary of change | Add option to include full configuration of RLC bearer |
| TTCN module | RRCReconfig_ENDC_EUTRA |
| MCC160 Comment |  |

Before change:

template (value) RLC_Configuration_Type cs_RLC_Configuration_TestModeAckProhibit (RLC_ACK_Prohibit_Type p_RLC_ACK_Prohibit ) :=

{

Rb := omit,

TestMode := {

Info := {

AckProhibit := p_RLC_ACK_Prohibit

}

}

};

After change:

template (value) RLC_Configuration_Type cs_RLC_Configuration_TestModeAckProhibit (RLC_ACK_Prohibit_Type p_RLC_ACK_Prohibit,

template (omit) RLC_RbConfig_Type p_RLC_RbConfig := omit) :=

{

Rb := p_RLC_RbConfig,

TestMode := {

Info := {

AckProhibit := p_RLC_ACK_Prohibit

}

}

};

## 2.3 Change 3

| Function name | New template: cs_RLC_RbConfig_SRB |
| --- | --- |
| Reason for change | As in Change 1: In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. |
| Summary of change | Add option to include full configuration of RLC bearer |
| TTCN module | RRCReconfig_ENDC_EUTRA |
| MCC160 Comment |  |

New template:

template (value) RLC_RbConfig_Type cs_RLC_RbConfig_SRB (template (value) UL_AM_RLC p_SRB_UL_AM_RLC := cs_TX_AM_RLC_SRB1_Def,

template (value) DL_AM_RLC p_SRB_DL_AM_RLC := cs_RX_AM_RLC_SRB1_Def) :=

{

AM := {

Tx := cs_UL_AM_RLC_r8(p_SRB_UL_AM_RLC),

Rx := cs_DL_AM_RLC_r8(p_SRB_DL_AM_RLC),

ExtendedLI := omit

}

};

## 2.4 Change 4

| Function name | fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | During Steps 4A and 4E, SS will resume sending of RLC ACK in EUTRA. There is some delay between resumption of sending RLC ACK and UE retransmission of RLC PDUs. To improve message flow and ensure un-ACKed RLC PDUs are properly ACKed, it is suggested to add some delay after resumption of sending RLC ACK in SS side. |
| Summary of change | Add delay after Steps 4A and 4E. |
| TTCN module | RRCReconfig_ENDC_EUTRA |
| MCC160 Comment |  |

Before change:

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5-191203 R5-201204 sic@

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (omit) octetstring v_NR_SCGConfig := omit;

// @siclog "Step 0-2A" siclog@

// The SS is configured not to send RLC ACK on split SRB1. => done during te HO procedure

// @siclog "Step 1" siclog@

// SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message to configure split SRB1 and split SRB2.

// Change SRB 1/2 with ciphering/integrity algorithms to NR values

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, false);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// SS transmits UECapabilityEnquiry message for NR capabilities on split SRB1 over the SCG path.

//The SS is configured not to send RLC ACK on split SRB1.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Prohibit)});

SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, cs_UE_CapabilityRequest_NR),

-,

cs_RlcBearerRouting_NR(nr_Cell1)));

// @siclog "Step 4" siclog@

// Check: Does the UE send UECapabilityInformation message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_UeCapabilityInformation(tsc_RRC_TI_Def, cr_NRCapInfo),

-,

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_UeCapabilityInformation(tsc_RRC_TI_Def, cr_NRCapInfo),

-,

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4");

// @siclog "Step 4A" siclog@

//After steps in table 8.2.2.2.1.3.2-3 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 4 is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

// @siclog "Step 4B" siclog@

//The SS is configured not to send RLC ACK on split SRB2.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB2, Prohibit)});

// @siclog "Step 4C" siclog@

// The SS sends DLInformationTransfer message (carrying IDENTITY REQUEST message) on split SRB2 over the SCG path.

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_IDENTITY_REQUEST(tsc_IdType_IMSI)),

cs_RlcBearerRouting_NR(nr_Cell1)));

// @siclog "Step 4D" siclog@

// Check: Does the UE send the ULInformationMessage (carrying IDENTITY RESPONSE message) on split SRB2 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_IDENTITY_RESPONSE(?)),

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_IDENTITY_RESPONSE(?)),

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

// @siclog "Step 4E" siclog@

// After steps in table 8.2.2.2.1.3.2-4 are completed the SS is configured to send RLC ACK on split SRB2 and the last message received in step 4D is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB2, Continue)});

// Send trigger to NR

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

// SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message on split SRB1 to release split SRB1 and split SRB2 over the SCG path.

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}

After change:

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5-191203 R5-201204 sic@

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (omit) octetstring v_NR_SCGConfig := omit;

// @siclog "Step 0-2A" siclog@

// The SS is configured not to send RLC ACK on split SRB1. => done during te HO procedure

// @siclog "Step 1" siclog@

// SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message to configure split SRB1 and split SRB2.

// Change SRB 1/2 with ciphering/integrity algorithms to NR values

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, false);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// SS transmits UECapabilityEnquiry message for NR capabilities on split SRB1 over the SCG path.

//The SS is configured not to send RLC ACK on split SRB1.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Prohibit)});

SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, cs_UE_CapabilityRequest_NR),

-,

cs_RlcBearerRouting_NR(nr_Cell1)));

// @siclog "Step 4" siclog@

// Check: Does the UE send UECapabilityInformation message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_UeCapabilityInformation(tsc_RRC_TI_Def, cr_NRCapInfo),

-,

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_UeCapabilityInformation(tsc_RRC_TI_Def, cr_NRCapInfo),

-,

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4");

// @siclog "Step 4A" siclog@

//After steps in table 8.2.2.2.1.3.2-3 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 4 is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

f_Delay(0.1);

// @siclog "Step 4B" siclog@

//The SS is configured not to send RLC ACK on split SRB2.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB2, Prohibit)});

// @siclog "Step 4C" siclog@

// The SS sends DLInformationTransfer message (carrying IDENTITY REQUEST message) on split SRB2 over the SCG path.

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_IDENTITY_REQUEST(tsc_IdType_IMSI)),

cs_RlcBearerRouting_NR(nr_Cell1)));

// @siclog "Step 4D" siclog@

// Check: Does the UE send the ULInformationMessage (carrying IDENTITY RESPONSE message) on split SRB2 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_IDENTITY_RESPONSE(?)),

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_IDENTITY_RESPONSE(?)),

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

// @siclog "Step 4E" siclog@

// After steps in table 8.2.2.2.1.3.2-4 are completed the SS is configured to send RLC ACK on split SRB2 and the last message received in step 4D is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB2, Continue)});

f_Delay(0.1);

// Send trigger to NR

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

// SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message on split SRB1 to release split SRB1 and split SRB2 over the SCG path.

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}