3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210936

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1777 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_1_1_5_5_NR_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current test procedure there is no requirement from the Prose specification to send data on a specific HARQ process, since there is no CRC corruption. The change to remove this requirement was proposed in R5s210582 but implementation in 21wk24 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed parameter from DRB.send to use cs_NR_HarqProcessAssignment_Automatic in Steps 5 and 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose and TTCN are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.5.ENDC, 7.1.1.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for test case 7.1.1.5.5	2

1.1	Correction to function f_TC_7_1_1_5_5_NR_TestBody	2

# Corrections required for test case 7.1.1.5.5

## Correction to function f_TC_7_1_1_5_5_NR_TestBody

| Function name | f_TC_7_1_1_5_5_NR_TestBody |
| --- | --- |
| Reason for change | In the current test procedure there is no requirement from the Prose specification to send data on a specific HARQ process, since there is no CRC corruption. The change to remove this requirement was proposed in R5s210582 but implementation in 21wk24 is missing. |
| Summary of change | Removed parameter from DRB.send to use cs_NR_HarqProcessAssignment_Automatic in Steps 5 and 7. |
| TTCN module | DRX_MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) //@sic R5s201495 sic@ const UInt_Type tsc_DrxInactivityTimer := 10; //in ms const UInt_Type tsc_DrxOnDurationTimer := 40; //in ms var UInt_Type v_ShortDRXCycle := 80; var integer v_LongDRXCycle := 640; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; //var SubFrameTiming_Type v_Timing2 := v_Timing; //var SubFrameTiming_Type v_Timing3 := v_Timing; var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (omit) NR_MAC_CE_SubPDU_DL_List_Type v_CE_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; timer t_Watchdog := 5.0; // stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped: f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset); // The other things in Step 3 automatically taken care by SS //@sic R5s210582 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ // 5 ms later configuere SS for normal CRC error mode // In the Last PDCCH occasion befor on duration timer expiry v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInSubframe * (tsc_DrxOnDurationTimer - 1)); //@sic R5s201495 sic@ v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) }; // v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit, cs_NR_MAC_Padding_SubPDU('000000'O)) }; // 2 byte sub headers +3byte Padding header= 40 bits DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1))); //@siclog "Step 7" siclog@ v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1))); //@siclog "Step 9" siclog@

After Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) //@sic R5s201495 sic@ const UInt_Type tsc_DrxInactivityTimer := 10; //in ms const UInt_Type tsc_DrxOnDurationTimer := 40; //in ms var UInt_Type v_ShortDRXCycle := 80; var integer v_LongDRXCycle := 640; var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; //var SubFrameTiming_Type v_Timing2 := v_Timing; //var SubFrameTiming_Type v_Timing3 := v_Timing; var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (omit) NR_MAC_CE_SubPDU_DL_List_Type v_CE_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2; timer t_Watchdog := 5.0; // stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped: f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset); // The other things in Step 3 automatically taken care by SS //@sic R5s210582 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ // 5 ms later configuere SS for normal CRC error mode // In the Last PDCCH occasion befor on duration timer expiry v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInSubframe * (tsc_DrxOnDurationTimer - 1)); //@sic R5s201495 sic@ v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) }; // v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit, cs_NR_MAC_Padding_SubPDU('000000'O)) }; // 2 byte sub headers +3byte Padding header= 40 bits // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2)); //@siclog "Step 7" siclog@ v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL)); //@siclog "Step 9" siclog@