3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210951

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1894 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC RRC test case 8.1.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.1.3.7 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.1.3.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 2355 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	8

4	Execution Log Files	8

4.1	HiSilicon Balong 5000	8

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.3.7 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.1.3.7.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_1_3_7_NR5GC |
| --- | --- |
| Reason for change | As per Prose CR R5-214767, NR cell2 is no longer needed. Since NR cell 2 has been removed, and UE will come back to NR cell 1 at step 5, the postamble will happen on NR cell 1 |
| Summary of change | Removed NR cell 2 from the TC Changed the postamble cell to NR Cell 1. |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_1_3_7_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / Deprioritisation / T325 expiry f_NR5GC_Init(NR_4); //Update SIB1 according to specific message contents f_NR_CellInfo_SetTAC (nr_Cell2, int2bit(2,24)); f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(3,24)); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Max Power level for FR2 is FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -79, -79); // Max Power level for FR2 is FFS f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_7_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_IDLE_1A); }

After Change

function f_TC_8_1_1_3_7_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / Deprioritisation / T325 expiry f_NR5GC_Init(NR_4); //Update SIB1 according to specific message contents REMOVED //f_NR_CellInfo_SetTAC (nr_Cell2, int2bit(2,24)); //R5-214767 f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(3,24)); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Max Power level for FR2 is FFS REMOVED //f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -79, -79); // Max Power level for FR2 is FFS //R5-214767 f_NR_CellConfig_Def(nr_Cell1); REMOVED //f_NR_CellConfig_Def(nr_Cell2); //R5-214767 f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_7_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

## Change 2

| Function name | fl_TC_8_1_1_3_7_TestBody |
| --- | --- |
| Reason for change | As per Prose CR R5-214767, NR cell2 is no longer needed. Timer T325 should be adjusted for the 10% tolerance |
| Summary of change | Removed NR cell 2 from the TC. Called function f_NR_SetTimerToleranceMin on timer T325 |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_3_7_TestBody() runs on NR5GC_PTC { var template (value) RRCRelease_IEs.deprioritisationReq v_DeprioritisationReq; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var float v_T325 := 300.0; var boolean v_FlagCell3 := false; timer t_WaitTime; v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, -85, -85), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell2, -79, -79), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell3, -91, -91) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message on NR Cell 1 including IE deprioritisationReq. v_DeprioritisationReq := cs_NR_DeprioritisationReq(frequency, min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, -, v_DeprioritisationReq)); //@siclog "Step 2" siclog@ //SS starts timer T325. t_WaitTime.start(v_T325); //@siclog "Step 3" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); alt { //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 3? [not v_FlagCell3] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCSetupRequest()))) { f_NR5GC_MobilityRegistration (nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_FlagCell3 := true; repeat; } //@siclog "Step 5" siclog@ //SS waits for T325 expiry. [] t_WaitTime.timeout { if(v_FlagCell3 == true){ //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 2? f_NR5GC_MobilityRegistration (nr_Cell2); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE did not transmit a RRCSetupRequest message on NR Cell 3 before T325 expiry"); } } } }

After Change

function fl_TC_8_1_1_3_7_TestBody() runs on NR5GC_PTC { var template (value) RRCRelease_IEs.deprioritisationReq v_DeprioritisationReq; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var float v_T325 := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 300.0); var boolean v_FlagCell3 := false; timer t_WaitTime; v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, -85, -85), // Power level for FR2 is FFS REMOVED //cs_NR_CellPower(nr_Cell2, -79, -79), // Power level for FR2 is FFS //R5-214767 cs_NR_CellPower(nr_Cell3, -91, -91) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message on NR Cell 1 including IE deprioritisationReq. v_DeprioritisationReq := cs_NR_DeprioritisationReq(frequency, min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, -, v_DeprioritisationReq)); //@siclog "Step 2" siclog@ //SS starts timer T325. t_WaitTime.start(v_T325); //@siclog "Step 3" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); alt { //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 3? [not v_FlagCell3] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCSetupRequest()))) { f_NR5GC_MobilityRegistration (nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_FlagCell3 := true; repeat; } //@siclog "Step 5" siclog@ //SS waits for T325 expiry. [] t_WaitTime.timeout { if(v_FlagCell3 == true){ //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 1? f_NR5GC_MobilityRegistration (nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE did not transmit a RRCSetupRequest message on NR Cell 3 before T325 expiry"); } } } }

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_1_3_7_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_1_3_7_PIXIT.xml

# References

| [1] | R5s210952 : NR5GC FR1 : Supporting information for addition of NR5GC RRC test case 8.1.1.3.7 |
| --- | --- |