3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210955

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0710 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 6.4 to the IMS/NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 6.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This submission performs changes according to RAN5 agreed prose CR R5-212127. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	MediaTek Tianji 1000	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.4.NR5GC which is part of the iWD-TTCN3-B2020-09_D21wk12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Hans Rohnert
	hans.rohnert@rohde-schwarz.com

## Verification Test Summary

Test Case: 	6.4.NR5GC

ATS Version:	iWD-TTCN3-B2020-09_D21wk12

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	MediaTek Tianji 1000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_6_4_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The REFRESH operation was removed by R5-212127 |
| Summary of change | Removed the code concerning the REFRESH operation |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_6_4_NR5GC_IMS1() runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var SipUrl v_ContactUrl;

var CharStringList_Type v_PublicUserIdList;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

// @siclog "Step 2 - 9" siclog@

v_IMS_REGISTER_REQ := f_IMS_Registration();

f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration);

// @siclog "Step 10" siclog@   trigger UICC to perform REFRESH

f_UT_IMS_Refresh(MMI, px_IMS_HomeDomainName_Refreshed, px_IMS_Private_UserId_Refreshed, px_IMS_PublicUserIdentity1_Refreshed);

// @siclog "Step 11 - 12"

f_IMS_CheckUnexpectedRegisterRequest(5.0);

// @siclog "Step 13 - 14"

f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling);

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl();

f_IMS_Register_Notify(v_ContactUrl, f_IMS_XmlMessageForNotify_Terminated(v_IMS_REGISTER_REQ.Request.Register), cs_SubscriptionState_Terminated);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14");

f_IMS_GlobalRelease();

v_PublicUserIdList := f_IMS_PTC_ImsInfo_GetAllIMPUs();

v_PublicUserIdList[0] := px_IMS_PublicUserIdentity1_Refreshed;

f_IMS_PTC_ImsInfo_Refresh(px_IMS_HomeDomainName_Refreshed, px_IMS_Private_UserId_Refreshed, v_PublicUserIdList);

// @siclog "Step 15 - 22" siclog@

f_IMS_Registration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15 - 22");

f_IMS_CC_ReleaseConnection(IPCAN_MT_IMS_Signalling);

// @siclog "Step 23" siclog@

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

f_UT_IMS_Deregister(MMI);

// @siclog "Step 24 - 29" siclog@

f_IMS_Deregistration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 24 - 29");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling);

}

After change:

function f_TC_6_4_NR5GC_IMS1() runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var SipUrl v_ContactUrl;

//var CharStringList_Type v_PublicUserIdList;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

// @siclog "Step 2 - 9" siclog@

v_IMS_REGISTER_REQ := f_IMS_Registration();

f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration);

// @siclog "Step 10" siclog@   trigger UICC to perform REFRESH

//f_UT_IMS_Refresh(MMI, px_IMS_HomeDomainName_Refreshed, px_IMS_Private_UserId_Refreshed, px_IMS_PublicUserIdentity1_Refreshed);

// @siclog "Step 11 - 12"

//f_IMS_CheckUnexpectedRegisterRequest(5.0); //WI=719523

// @siclog "Step 13 - 14"

f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling);

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl();

f_IMS_Register_Notify(v_ContactUrl, f_IMS_XmlMessageForNotify_Terminated(v_IMS_REGISTER_REQ.Request.Register), cs_SubscriptionState_Terminated);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14");

f_IMS_GlobalRelease();

/*

v_PublicUserIdList := f_IMS_PTC_ImsInfo_GetAllIMPUs();

v_PublicUserIdList[0] := px_IMS_PublicUserIdentity1_Refreshed;

f_IMS_PTC_ImsInfo_Refresh(px_IMS_HomeDomainName_Refreshed, px_IMS_Private_UserId_Refreshed, v_PublicUserIdList);

* */

// @siclog "Step 15 - 22" siclog@

f_IMS_Registration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15 - 22");

f_IMS_CC_ReleaseConnection(IPCAN_MT_IMS_Signalling);

// @siclog "Step 23" siclog@

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

f_UT_IMS_Deregister(MMI);

// @siclog "Step 24 - 29" siclog@

f_IMS_Deregistration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 24 - 29");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling);

}

# Branches executed

n/a

# Execution Log Files

## MediaTek Tianji 1000

The MediaTek Tianji 1200 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Rohde & Schwarz: 6_4_RS_MTK_ttcn_tli

# References

| [1] | R5s210667: Supporting information for agreement of TC 6.4 |
| --- | --- |