3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211243

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2011 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC NAS TC 9.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For TC 9.1.5.1.1, at steps 8, 30, 52 and 74 after UE switch OFF sequence, UE is switched on without handling IMS re-registration in the case when multiple pdu sessions (i.e. internet and ims) are being configured by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify function during switch OFF ON procedure to Handle IMS re-registration in the case of multiple PDU Session (i.e., internet and ims). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE fails the testcase if multiple PDU sessions are configured (i.e., internet and ims) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2021-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | function fl_TC_9_1_5_1_1_Body |
| --- | --- |
| Reason for change | Handle IMS re-registration in the case of multiple PDU Sessions |
| Summary of change | Modify function during switch OFF ON procedure to Handle IMS re-registration in the case of multiple PDU Session (i.e., internet and ims). |
| TTCN module | Initial_Registration_NR5GC |
| MCC160 Comment |  |

Before change:

/*

* @desc      test body of TC_9_1_5_1_1

* @status    APPROVED (NR5GC)

*/

function fl_TC_9_1_5_1_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellH := f_NR5GC_MapNASCell(ngc_CellH, NG_CellsOnDifferentPLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

//@siclog "Step 1" siclog@

//Set NGC Cell A to serving cell power and NGC Cell C and NGC Cell H to Non-suitable cell power

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879 sic@ //@sic R5s191189 sic@

f_NR_SetCellPower (v_NGC_NASCellH, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879 sic@ //@sic R5s191189 sic@

//@siclog "Step 2" siclog@

f_NR5GC_SwitchOnAndResetIMS(); //Anritsu TTCN CR R5s210729

//@siclog "Steps 3-5" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 6" siclog@

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_EmmCause_IllegalUe))))); //@sic R5-194859 sic@

//@siclog "Step 7" siclog@

// Switch off the UE

f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_DEREGISTERED);

//@siclog "Step 8" siclog@

// Switch on UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 9 - 11" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); //@sic R5s191189 sic@

//@siclog "Step 12" siclog@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,

cr_NG_MobileIdentitySUCI(?, ?), //Check the SUCI

-,

-,

-,

-,

-,

-,

- //@sic R5s191189 sic@

)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12");

}

//@siclog "Step 30" siclog@

f_UT_SwitchOnUE(UT);

//@siclog "Steps 31- 33" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellH, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

//@siclog "Step 34" siclog@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,

f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit), //Check the 5G-GUTI

-,

-,

-,

-,

-,

-,

f_LastVisited_TAI (v_NGC_NASCellA) //Check the LastVisitedTAI

)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 34");

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34");

//@siclog "Step 52" siclog@

// Switch on UE

f_UT_SwitchOnUE(UT);

.

.

.

//@siclog "Step 74" siclog@

// Switch on UE

f_UT_SwitchOnUE(UT);

…

}

After change:

/*

* @desc      test body of TC_9_1_5_1_1

* @status    APPROVED (NR5GC)

*/

function fl_TC_9_1_5_1_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellH := f_NR5GC_MapNASCell(ngc_CellH, NG_CellsOnDifferentPLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

//@siclog "Step 1" siclog@

//Set NGC Cell A to serving cell power and NGC Cell C and NGC Cell H to Non-suitable cell power

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879 sic@ //@sic R5s191189 sic@

f_NR_SetCellPower (v_NGC_NASCellH, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879 sic@ //@sic R5s191189 sic@

//@siclog "Step 2" siclog@

f_NR5GC_SwitchOnAndResetIMS(); //Anritsu TTCN CR R5s210729

//@siclog "Steps 3-5" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 6" siclog@

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_EmmCause_IllegalUe))))); //@sic R5-194859 sic@

//@siclog "Step 7" siclog@

// Switch off the UE

f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_DEREGISTERED);

//@siclog "Step 8" siclog@

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS(); //Annritsu TTCN CR Needed

//@siclog "Steps 9 - 11" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); //@sic R5s191189 sic@

//@siclog "Step 12" siclog@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,

cr_NG_MobileIdentitySUCI(?, ?), //Check the SUCI

-,

-,

-,

-,

-,

-,

- //@sic R5s191189 sic@

)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12");

}

//@siclog "Step 30" siclog@

f_NR5GC_SwitchOnAndResetIMS(); //Annritsu TTCN CR Needed

//@siclog "Steps 31- 33" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellH, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

//@siclog "Step 34" siclog@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,

f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA, omit), //Check the 5G-GUTI

-,

-,

-,

-,

-,

-,

f_LastVisited_TAI (v_NGC_NASCellA) //Check the LastVisitedTAI

)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 34");

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34");

//@siclog "Step 52" siclog@

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS(); //Annritsu TTCN CR Needed

.

.

.

//@siclog "Step 74" siclog@

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS(); //Annritsu TTCN CR Needed

…

}