3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211262

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2026 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_UE_DeRegisterOnSwitchOff |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, for the case STATE_DEREGISTERED, the function f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId)))); is called after the local release. This causes issues, as the function f_SubFrameTiming_Duration returns a value that is too high for the type Integer. As a result, the timer value comes out as a junk value. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that instead of calling f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId))));, call function f_Delay(f_SubFrameTimingDuration_Float(f_NR_GetCurrentTiming(p_NR_CellId), v_TimingOfLocalRelease));. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Many other 9.x NR testcases are also affected by this issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_NR_UE_DeRegisterOnSwitchOff	2

# Corrections required

## Correction to function f_NR_UE_DeRegisterOnSwitchOff

| Function name | f_NR_UE_DeRegisterOnSwitchOff |
| --- | --- |
| Reason for change | In the current TTCN implementation, for the case STATE_DEREGISTERED, the function f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId)))); is called after the local release. This causes issues, as the function f_SubFrameTiming_Duration returns a value that is too high for the type Integer. As a result, the timer value comes out as a junk value. This needs to be corrected. |
| Summary of change | It is proposed that instead of calling f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId))));, call function f_Delay(f_SubFrameTimingDuration_Float(f_NR_GetCurrentTiming(p_NR_CellId), v_TimingOfLocalRelease));. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> case(STATE_DEREGISTERED) { // @sic R5-201202 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ // don't need to release the local configuration as RRC Release is always done before this function is called @sic R5s201228 sic@ v_TimingOfLocalRelease := f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); //@sic R5s190984 sic@ f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId)))); //@sic R5s210839 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> case(STATE_DEREGISTERED) { // @sic R5-201202 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ // don't need to release the local configuration as RRC Release is always done before this function is called @sic R5s201228 sic@ v_TimingOfLocalRelease := f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); //@sic R5s190984 sic@ //WA#NAS COMMENTED OUT f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId)))); //@sic R5s210839 sic@ f_Delay(f_SubFrameTimingDuration_Float(f_NR_GetCurrentTiming(p_NR_CellId), v_TimingOfLocalRelease)); //WA#NAS } <<SKIPPED CODE>>