3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211293

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2054 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS Fallback test case 11.1.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EPS Fallback TC 11.1.9 MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to EPS Fallback TC 11.1.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	8

3	Branches Executed	9

4	Execution Log Files	9

4.1	Qualcomm SM 8450 + SDX65	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 11.1.9 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.9

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_1_9_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | - As Expected in Prose Step 14 the re-direct info of EUTRA_CELL1 need to include in RrcRelease and NR5GC PCT includes the re-dirict info in RrcRelease but Eutra PTC doesn’t sends |
| Summary of change | - Sent the Frequency info of eutra cell1 from Eutra PTC to NR5GC PTC |
| TTCN module | fl_TC_11_1_9_NR5GC_EUTRA_TestBody() |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); … //Postamble is in Eutra f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; var EUTRA_NR_CoOrd_SysInfo_Type v_SysInfoNR; // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); //v_SysInfoNR := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); // @siclog "Step 15 - 20" siclog@ … //Postamble is in Eutra f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 2

| Function name | cs_508_SystemInformationBlockType2_Def |
| --- | --- |
| Reason for change | EUTRA Cell 2 SIB2 to include voiceServiceCauseIndication signalled for setting MO-voice call cause #draft proseCR would be raised for RAN5#93-e meeting |
| Summary of change | - Including voiceServiceCauseIndication in SIB2 - New Fucntion(refer change 4) Added to set voiceServiceCauseIndication to true |
| TTCN module | EUTRA_SysInfo_Templates.ttcn |
| MCC160 Comment |  |

Before Change

template (value) SystemInformationBlockType2 cs_508_SystemInformationBlockType2_Def(template (omit) EUTRA_SysinfoUL_Bandwidth_Type p_UL_ChBandwidth, N_RB_CQI_Type p_N_RB_CQI, PUSCH_HoppingOffset_Type p_PuschHoppingOffset, template (value) SoundingRS_UL_ConfigCommon p_SoundingRS_UL_ConfigCommon, template (value) PRACH_ConfigSIB p_PRACH_ConfigSIB, P_b_Type p_P_b, template(omit) PDSCH_ConfigCommon_v1310 p_PDSCH_ConfigCommon_v1310 := omit, template(omit) PUSCH_ConfigCommon_v1310 p_PUSCH_ConfigCommon_v1310 := omit, template(omit) PRACH_ConfigSIB_v1310 p_PRACH_ConfigSIB_v1310 := omit, template(omit) PUCCH_ConfigCommon_v1310 p_PUCCH_ConfigCommon_v1310 := omit, template(omit) PCCH_Config_v1310 p_PCCH_Config_v1310 := omit, template(omit) PreambleTransMax p_PreambleTransMax := omit, template(omit) RACH_CE_LevelInfoList_r13 p_RACH_CE_LevelInfoList_r13 := omit, template(omit) CIOT_EPS_OptimisationInfo_r13 p_CIOT_EPS_OptimisationInfo_r13 := omit, template(omit) FreqHoppingParameters_r13 p_FreqHoppingParameters_r13 := omit, template(omit) HighSpeedConfig_r14 p_HighSpeedConfig := omit, template(omit) PRACH_Config_v1430 p_PRACH_Config_v1430 := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ //@sic R5-167569: added p_PDSCH_ConfigCommon_v1310, p_PUSCH_ConfigCommon_v1310, p_PRACH_ConfigSIB_v1310, p_PUCCH_ConfigCommon_v1310, p_PCCH_Config_v1310 sic@ //@sic R5-172875: added p_CIOT_EPS_OptimisationInfo_r13 sic@ //@sic R5s170550: added p_FreqHoppingParameters_r13 sic@ // Acc. 36.508 Table 4.4.3.3-1: SystemInformationBlockType2 ... ac_BarringPerPLMN_List_r12 := omit, /* @sic BASELINE MOVING 2015 sic@ */ voiceServiceCauseIndication_r12 := omit, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ ... };

After Change

template (value) SystemInformationBlockType2 cs_508_SystemInformationBlockType2_Def(template (omit) EUTRA_SysinfoUL_Bandwidth_Type p_UL_ChBandwidth, N_RB_CQI_Type p_N_RB_CQI, PUSCH_HoppingOffset_Type p_PuschHoppingOffset, template (value) SoundingRS_UL_ConfigCommon p_SoundingRS_UL_ConfigCommon, template (value) PRACH_ConfigSIB p_PRACH_ConfigSIB, P_b_Type p_P_b, template(omit) PDSCH_ConfigCommon_v1310 p_PDSCH_ConfigCommon_v1310 := omit, template(omit) PUSCH_ConfigCommon_v1310 p_PUSCH_ConfigCommon_v1310 := omit, template(omit) PRACH_ConfigSIB_v1310 p_PRACH_ConfigSIB_v1310 := omit, template(omit) PUCCH_ConfigCommon_v1310 p_PUCCH_ConfigCommon_v1310 := omit, template(omit) PCCH_Config_v1310 p_PCCH_Config_v1310 := omit, template(omit) PreambleTransMax p_PreambleTransMax := omit, template(omit) RACH_CE_LevelInfoList_r13 p_RACH_CE_LevelInfoList_r13 := omit, template(omit) CIOT_EPS_OptimisationInfo_r13 p_CIOT_EPS_OptimisationInfo_r13 := omit, template(omit) FreqHoppingParameters_r13 p_FreqHoppingParameters_r13 := omit, template(omit) HighSpeedConfig_r14 p_HighSpeedConfig := omit, template(omit) PRACH_Config_v1430 p_PRACH_Config_v1430 := omit) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ //@sic R5-167569: added p_PDSCH_ConfigCommon_v1310, p_PUSCH_ConfigCommon_v1310, p_PRACH_ConfigSIB_v1310, p_PUCCH_ConfigCommon_v1310, p_PCCH_Config_v1310 sic@ //@sic R5-172875: added p_CIOT_EPS_OptimisationInfo_r13 sic@ //@sic R5s170550: added p_FreqHoppingParameters_r13 sic@ // Acc. 36.508 Table 4.4.3.3-1: SystemInformationBlockType2 ... ac_BarringPerPLMN_List_r12 := omit, /* @sic BASELINE MOVING 2015 sic@ */ voiceServiceCauseIndication_r12 := fl_EUTRA_GetVoiceServiceCauseIndication(), /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ ... };

New Function:

function fl_EUTRA_GetVoiceServiceCauseIndication() return template(omit) VoiceSerCauseInd_r12 { /* @sic R5-198995 sic@ */ var charstring v_TestcaseName := testcasename(); var template(omit) VoiceSerCauseInd_r12 v_voiceSerCauseInd := omit; if (f_GetTestcaseAttrib_EPS(v_TestcaseName)) { v_voiceSerCauseInd := true_; } return v_voiceSerCauseInd; }

## Change 3

| Function name | f_GetTestcaseAttrib_EPS |
| --- | --- |
| Reason for change | Please refer to Change 3 for reason |
| Summary of change | Adding New function in same file at different paths |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

New Function:

Path: NR5GC_IRAT\Common\TestcaseProperties.ttcn function f_GetTestcaseAttrib_EPS(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_1_8_NR5GC") { return true; } } return false; } Path: NR5GC\Common\TestcaseProperties.ttcn function f_GetTestcaseAttrib_EPS(charstring p_Testcase) return boolean { return false; } Path: ENDC\common\TestcaseProperties.ttcn function f_GetTestcaseAttrib_EPS(charstring p_Testcase) return boolean { return false; }

## Change 4

| Function name | EUTRA_CommonDefs.ttcn |
| --- | --- |
| Reason for change | New Variable Added voiceServiceCauseIndication_r12 Refer Change2 |
| Summary of change | Define Varibale VoiceSerCauseInd_r12 |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

New Function:

type SystemInformationBlockType2.freqInfo.ul_Bandwidth EUTRA_SysinfoUL_Bandwidth_Type; type SystemInformationBlockType2.freqInfo.additionalSpectrumEmission AdditionalSpectrumEmission_Type; type SystemInformationBlockType2.ac_BarringInfo.ac_BarringForEmergency AccessBarringForEmergencyCalls_Type; type SystemInformationBlockType2.ac_BarringInfo.ac_BarringForMO_Signalling.ac_BarringFactor AccessProbabilityFactor_Type; type SystemInformationBlockType2.ac_BarringInfo.ac_BarringForMO_Signalling.ac_BarringTime AccessBarringTime_Type; type SystemInformationBlockType2.ac_BarringInfo AccessBarringInformation_Type; type SystemInformationBlockType2.voiceServiceCauseIndication_r12 VoiceSerCauseInd_r12;

# Branches Executed

This test case was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n78 using the Ciphering Algorithm nea1 and Integrity Algorithm nia1.

# Execution Log Files

## Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_11_1_9_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\TC_11_1_9_PICS.txt

# References

| [1] | R5s211265 |
| --- | --- |