3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211309

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2069 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS Fallback test case 11.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EPS Fallback TC 11.1.8 MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to EPS Fallback TC 11.1.8 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches Executed	6

4	Execution Log Files	6

4.1	Qualcomm SM 8450 + SDX65	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 11.1.8 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.1.8

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_1_8_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - We need to set voiceServiceCauseIndication in SIB2 of target EUTRA Cell signalled for setting MO-voice call cause - Incorrect cell ID used in call to EUTRA Postamble |
| Summary of change | - Setting voiceServiceCauseIndication in SIB2 of target EUTRA Cell 2 by adding a call to new function f_EUTRA_CellInfo_SetSysInfo_voiceServiceCauseIndication (see change 2) - Changed cell id to eutra_cell2 in EUTRA postamble |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ///MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell2); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change

function f_TC_11_1_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ///MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set voiceServiceCauseIndication in SIB-2 for eutra cell 2 f_EUTRA_CellInfo_SetSysInfo_voiceServiceCauseIndication(eutra_Cell2); f_EUTRA_CellConfig_Def(eutra_Cell2); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell2, E2_CONNECTED); }

## Change 2

| Function name | New function f_EUTRA_CellInfo_SetSysInfo_voiceServiceCauseIndication() |
| --- | --- |
| Reason for change | Need to set voiceServiceCauseIndication in SIB2 of target EUTRA Cell signalled for setting MO-voice call cause |
| Summary of change | Sets the voiceServiceCauseIndication in SIB2 of target EUTRA Cell |
| TTCN module | EUTRA_CellInfo |
| MCC160 Comment |  |

New function:

function f_EUTRA_CellInfo_SetSysInfo_voiceServiceCauseIndication(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); v_CellInfo.Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib2.voiceServiceCauseIndication_r12 := true_; f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); f_EUTRA_SetSB1_ValueTag(p_CellId,true); }

## Change 3

| Function name | fl_TC_11_1_8_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC |
| --- | --- |
| Reason for change | - The setting of EUTRA cell 2 to T0 level is currently not done until just before step 17, because the function f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA () will block waiting for PDU session info from NR PTC. - The TTCN implementatojn to send ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST piggybanked with RRCConnectionReconfiguration at step 28 and handle RRCConnectionRecinfgurationComplete at step 29 is missing |
| Summary of change | - Move setting of T0 power level to before call to f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA () - Added code to send NAS and RRC message for step 28 |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_8_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; var octetstring v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1)); // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell2, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 10" siclog@ f_EUTRA38_SetCellPower(eutra_Cell2, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 18 -23" siclog@ v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell2, tsc_RRC_TI_Def, -, MAPPED_CXT, noRrcConnectionRelease, mo_VoiceCall_v1280, -, -, -, -, true); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Step 24 - 27" siclog@ // Steps 5-7 of 4.5A.6.3-1 f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell2, tsc_RRC_TI_Def, v_NasCount); // @siclog "Step 28 - 30" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell2); // wait for Trigger for Step 31 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 31" siclog@ f_Delay(1.0); //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell2, tsc_EpsDedicatedBearerId); //Send a trigger to NR for it to release its cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); } }

After Change

function fl_TC_11_1_8_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; f_EUTRA38_SetCellPower(eutra_Cell2, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); var octetstring v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1)); // @siclog "Step 1" siclog@ //MOVED f_EUTRA38_SetCellPower(eutra_Cell2, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 10" siclog@ f_EUTRA38_SetCellPower(eutra_Cell2, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 18 -23" siclog@ v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell2, tsc_RRC_TI_Def, -, MAPPED_CXT, noRrcConnectionRelease, mo_VoiceCall_v1280, -, -, -, -, true); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Step 24 - 27" siclog@ // Steps 5-7 of 4.5A.6.3-1 f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell2, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell2, tsc_RRC_TI_Def); // @siclog "Step 28 - 30" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell2); // wait for Trigger for Step 31 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 31" siclog@ f_Delay(1.0); //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell2, tsc_EpsDedicatedBearerId); //Send a trigger to NR for it to release its cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); } }

# Branches Executed

The test case was executed on NR band n78 and ( EUTRA band 3 on Anritus and EUTRA band 3 on Keysight) using NR ciphering nea1 and integrity algorithm nia1, and EUTRA ciphering algorithm eea1 and integrity algorithm eia1.

# Execution Log Files

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_11_1_8_LOG.xml

Keysight/TC_11_1_8_.html

- PICS/PIXIT parameter file:

Anritsu\TC_11_1_8_PICS.txt

Keysight/TC_11_1_8_PIXIT.xml

# References

| [1] | R5s211267 |
| --- | --- |