3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211321

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2076 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.2 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	HiSilicon Balong 5000	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_4_2_NR5GC |
| --- | --- |
| Reason for change | - Updated PLMN ID is not applied in NR cell 1 system information - Postamble state should be STATE_EMERGENCY_RELEASE_IDLE |
| Summary of change | - Apply system information updates - Correct postamble state |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_2_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-15. f_UT_USIM_Insert(UT, "USIM configuration 15 specified in table 6.4.1-15 TS 38.508-1"); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_TestBody_Set(true); fl_TC_11_4_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_11_4_2_NR5GC() runs on NR5GC_PTC { var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; //@sic R5s210806 sic@ f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-15. f_UT_USIM_Insert(UT, "USIM configuration 15 specified in table 6.4.1-15 TS 38.508-1"); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 //@sic R5s210806 sic@ v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); f_NR_TestBody_Set(true); fl_TC_11_4_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_EMERGENCY_RELEASE_IDLE);//@sic R5s210806 sic@ }

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_2_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_2_PIXIT.xml

# References

| [1] | R5s211322 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.2 |
| --- | --- |