3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211333

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2086 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.4.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR Cell 23 and NR cell 3 are intra-frequency cells and are frame aligned. This may cause the UE to see strong interference from the neighbor cell as the cells have same power level at step 8 when handover is performed from cell 3 to cell 23, due to which the UE may fail the SIB1 read on cell 23 upon handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the cell power level of Cell 23 to -82dBm at T1. Note : An associated prose CR on 38.523-1 will be raised at RAN5#93 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_4_1_7_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | NR Cell 23 and NR cell 3 are intra-frequency cells and are frame aligned. This may cause the UE to see strong interference from the neighbor cell as the cells have same power level at step 8 when handover is performed from cell 3 to cell 23, due to which the UE may fail the SIB1 read on cell 23 upon handover. |
| Summary of change | Changed the cell power level of Cell 23 to -82dBm at T1. Note : An associated prose CR on 38.523-1 will be raised at RAN5#93 |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_SourceSCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -88) //FR2 power levels are FFS @sic R5s210079 sic@ }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_TargetPCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_TargetSCell, -88, -88) //FR2 power levels are FFS }; … … }

After Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_SourceSCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -88) //FR2 power levels are FFS @sic R5s210079 sic@ }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -88), //FR2 power levels are FFS cs_NR_CellPower(p_TargetPCell, -82, -82), //FR2 power levels are FFS cs_NR_CellPower(p_TargetSCell, -88, -88) //FR2 power levels are FFS }; … … }

## Change 2

| Function name | f_TC_8_1_4_1_7_x_NR_Common |
| --- | --- |
| Reason for change | NR Cell 23 and NR cell 3 are intra-frequency cells and are frame aligned. This may cause the UE to see strong interference from the neighbor cell as the cells have same power level at step 8 when handover is performed from cell 3 to cell 23, due to which the UE may fail the SIB1 read on cell 23 upon handover. |
| Summary of change | Update maximum reference power of cell 23 due to change 1. |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_4_1_7_x_NR_Common (NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CA_Tested_Type p_NR_CA_Tested, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { f_NR5GC_Init_CA(NR_4, {p_SourcePCell, p_SourceSCell, p_TargetPCell, p_TargetSCell}, p_NR_CA_Tested); // @sic R5-204446, R5s201249 sic@ //Set TargetPCell with the same PLMN and TAC as the SourcePCell //@sic R5-204446 sic@ f_NR_CellInfo_SetPLMN(p_TargetPCell, f_NR_CellInfo_GetPLMN(p_SourcePCell)); f_NR_CellInfo_SetTAC(p_TargetPCell, f_NR_CellInfo_GetTAC(p_SourcePCell)); … … } //end of f_TC_8_1_5_7_1_x_NR_Common

After Change

function f_TC_8_1_4_1_7_x_NR_Common (NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CA_Tested_Type p_NR_CA_Tested, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { f_NR5GC_Init_CA(NR_4, {p_SourcePCell, p_SourceSCell, p_TargetPCell, p_TargetSCell}, p_NR_CA_Tested); // @sic R5-204446, R5s201249 sic@ f_NR_CellInfo_InitMaxReferencePower (p_TargetPCell, -82, -82); //ADDED //Set TargetPCell with the same PLMN and TAC as the SourcePCell //@sic R5-204446 sic@ f_NR_CellInfo_SetPLMN(p_TargetPCell, f_NR_CellInfo_GetPLMN(p_SourcePCell)); f_NR_CellInfo_SetTAC(p_TargetPCell, f_NR_CellInfo_GetTAC(p_SourcePCell)); … … } //end of f_TC_8_1_5_7_1_x_NR_Common