3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211349

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2099 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The mac-LogicalChannelConfig in RRCreconfiguration in step 6 is not present which is against 38.508-1 Table 4.6.3-148: RLC-BearerConfig. - In line with changes from Prose CR R5-216177, at Step 12B UE is triggered to esatablish a PDU Session. At Step 12C the PDU Session Establishment Procedure is verified, since no specific DNN is specified, UE should follow DNN requested in Step 12B when sending PDU Session Establishment Request |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ommiting the mac_LogicalChannelConfig has been removed. - Modify Step 12C to check PDU Session Establishment Request for DNN according to AT Command sent at Step 12B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | .Content of signalling message not according to 38.508-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The mac-LogicalChannelConfig in RRCreconfiguration in step 6 is not present which is against 38.508-1 Table 4.6.3-148: RLC-BearerConfig. - In line with changes from Prose CR R5-216177, at Step 12B UE is triggered to esatablish a PDU Session. At Step 12C the PDU Session Establishment Procedure is verified, since no specific DNN is specified, UE should follow DNN requested in Step 12B when sending PDU Session Establishment Request |
| Summary of change | - Ommiting the mac_LogicalChannelConfig has been removed. - Modify Step 12C to check PDU Session Establishment Request for DNN according to AT Command sent at Step 12B. |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_CellGroupConfig.rlc_BearerToAddModList[0].mac_LogicalChannelConfig := omit; v_CellGroupConfig.rlc_BearerToAddModList[1].mac_LogicalChannelConfig := omit; v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.1.1: Step 7. SecurityModeComplete Received from UE"); // @siclog "Step 8" siclog@ …

After Change

import from NG_NAS_TypeDefs all; import from NAS_AuxiliaryDefsAndFunctions all; function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; var template (omit) DNN v_DNN; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); // REMOVED v_CellGroupConfig.rlc_BearerToAddModList[0].mac_LogicalChannelConfig := omit; // REMOVED v_CellGroupConfig.rlc_BearerToAddModList[1].mac_LogicalChannelConfig := omit; v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.1.1: Step 7. SecurityModeComplete Received from UE"); // @siclog "Step 8" siclog@ … // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN))), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 sic@ // @siclog "Steps 12B" siclog@ // Cause the UE to request establishment of PDU session. v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN)); f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @siclog "Steps 12C" siclog@ // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell1,-,tsc_NR_RbId_SRB2,-,-,-,-,v_DNN); } …