3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211358

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2106 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_RegistrationReject |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of testcase 9.3.1.2, the function f_NR5GC_PreambleReject f_NR5GC_RegistrationReject f_NR5GC_RRC_Idle_Steps5_9_AKA is called. This function does not store the value of v_GMM_MobilityInfo. Neither does function f_NR5GC_RegistrationReject store this value. This causes an issue when in the function f_TC_9_3_1_2_NR5GC the following check is enforced : v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); if (match(v_GMM_MobilityInfo.GMMCap.s1Cap, '1'B) and match(v_GMM_MobilityInfo.S1NwkCap.cIoT_V2X, v_PatterncIoT_V2X)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called function f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); in function f_NR5GC_RegistrationReject. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function f_NR5GC_RegistrationReject	3

# Corrections required

## Correction to function f_NR5GC_RegistrationReject

| Function name | f_NR5GC_RegistrationReject |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 9.3.1.2, the function f_NR5GC_PreambleReject f_NR5GC_RegistrationReject f_NR5GC_RRC_Idle_Steps5_9_AKA is called. This function does not store the value of v_GMM_MobilityInfo. Neither does function f_NR5GC_RegistrationReject store this value. This causes an issue when in the function f_TC_9_3_1_2_NR5GC the following check is enforced : v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); if (match(v_GMM_MobilityInfo.GMMCap.s1Cap, '1'B) and match(v_GMM_MobilityInfo.S1NwkCap.cIoT_V2X, v_PatterncIoT_V2X)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } This needs to be addressed. |
| Summary of change | Called function f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); in function f_NR5GC_RegistrationReject. Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RegistrationReject(NR_CellId_Type p_NR_CellId, NAS_CauseValue_Type p_Cause := tsc_EmmCause_IllegalMe, TypeOfRegistration_Type p_RegType := Initial_NoSecurity) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 4 - 7 // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, v_PLMN, p_RegType); // Step 8 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, p_Cause))))); // @sic R5s190435 sic@ // Step 9 f_NR_RRCRelease (p_NR_CellId); // Step 10 f_NR_UE_DeRegisterOnSwitchOff(p_NR_CellId, STATE_DEREGISTERED); }

After Change:

function f_NR5GC_RegistrationReject(NR_CellId_Type p_NR_CellId, NAS_CauseValue_Type p_Cause := tsc_EmmCause_IllegalMe, TypeOfRegistration_Type p_RegType := Initial_NoSecurity) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 4 - 7 // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, v_PLMN, p_RegType); // Step 8 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, p_Cause))))); // @sic R5s190435 sic@ // Step 9 f_NR_RRCRelease (p_NR_CellId); // Step 10 f_NR_UE_DeRegisterOnSwitchOff(p_NR_CellId, STATE_DEREGISTERED); f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //WA#9_3_1_2 }