3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211385

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1637 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Addition of NR PDCP test case 7.1.3.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR PDCP test case 7.1.3.5.5 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR PDCP test case 7.1.3.5.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.5.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	5

2.4 Change 4	8

2.5 Change 5	10

3	Branches executed	11

4	Execution Log File	11

4.1	MediaTek MT6885	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.3.5.5 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.3.5.5

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6885

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_7_1_3_5_5_EUTRA_TestBody() |
| --- | --- |
| Reason for change | At step #9, NR PTC shall need to wait for EUTRA PTC to complete stopping of UL Grant Transmission, before sending DL PDCP data at Step #10. |
| Summary of change | Added a CoOrdination message after Step #9, to trigger NR PTC to execute Step #10. |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before

function fl_TC_7_1_3_5_5_EUTRA_TestBody()runs on EUTRA_5GS_PTC

{

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig (-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, SCG_RBConfig, -, -, -, v_RadioResourceConfigDedicated, v_ConfigDrbSS); // @sic R5s191042, R5-198180 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

//@siclog "Step 9" siclog@

//The SS stops allocating any UL grant for the AM RLC secondary entity. @sic R5-204429 sic@

f_EUTRA_StopULGrantTransmission(eutra_Cell1, cs_TimingInfo_Now); // @sic R5-204429 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity.. @sic R5-204429 sic@

f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // @sic R5-204429 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

}

After change

function fl_TC_7_1_3_5_5_EUTRA_TestBody()runs on EUTRA_5GS_PTC

{

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig (-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, SCG_RBConfig, -, -, -, v_RadioResourceConfigDedicated, v_ConfigDrbSS); // @sic R5s191042, R5-198180 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

//@siclog "Step 9" siclog@

//The SS stops allocating any UL grant for the AM RLC secondary entity. @sic R5-204429 sic@

f_EUTRA_StopULGrantTransmission(eutra_Cell1, cs_TimingInfo_Now); // @sic R5-204429 sic@

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity.. @sic R5-204429 sic@

f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // @sic R5-204429 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

}

## 2.2 Change 2

| Function name | f_TC_7_1_3_5_5_ENDC_EUTRA() |
| --- | --- |
| Reason for change | EUTRA side of split bearer configuration is not configured at the SS. |
| Summary of change | Configure SS EUTRA side for DRB2 |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before

function f_TC_7_1_3_5_5_ENDC_EUTRA() runs on EUTRA_5GS_PTC

{ // PDCP Data Duplication

f_EUTRA38_Init(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble to enter UE in state 2

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_Split, RRC_IDLE);

//Set null cipher Algorithm

f_EUTRA_AS_CipheringAlgorithm_Set(eea0);

//Procedure to enter UE in state 2A

f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TEST_LOOPModeA_ON,-, -, -, -, -, MCG_Split);

f_EUTRA_TestBody_Set(true);

}

After change

function f_TC_7_1_3_5_5_ENDC_EUTRA() runs on EUTRA_5GS_PTC

{ // PDCP Data Duplication

var template (omit) RadioBearerList_Type v_DrbConfigAtSS := omit;

f_EUTRA38_Init(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble to enter UE in state 2

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_Split, RRC_IDLE);

//Set null cipher Algorithm

f_EUTRA_AS_CipheringAlgorithm_Set(eea0);

//Procedure to enter UE in state 2A

v_DrbConfigAtSS := {

cs_RadioBearer_Reconfig_SplitDRB2 (nr_Cell1)

};

f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TEST_LOOPModeA_ON,-, -, -, -, v_DrbConfigAtSS, MCG_Split);

f_EUTRA_TestBody_Set(true);

………………………………………..

………………………………………….

}

## 2.3 Change 3

| Function name | f_TC_7_1_3_5_5_NR_TestBody() |
| --- | --- |
| Reason for change | - According to the confirmance requirements: “if the successful delivery of a PDCP Data PDU is confirmed by one of the two associated AM RLC entities: - indicate to the other AM RLC entity to discard the duplicated PDCP Data PDU;” Currently SS RLC will acknowledge the received PDU so the duplicated PDU may not be sent by the UE. - v_ExpectedPdu is not updated after sending the DL PDCP data at step 8B. - At Step 9, wait for CoOrdination message from EUTRA PTC before executing Step 10. (See change 1) - At step 10, index of the SDU is not correct. - At Step 11, Index of expected SDU is not correct - Wait Time need to be stopped after receiving the expected SDU at step #11. - Index of the SDU Need to be corrected at step 13. - Require a PASS verdit on Timeout at step 13 |
| Summary of change | - Modified function f_NR_GetRadioBearerConfig_FullAM with a new parameter to configure RLC on test Mode with NACKNEXT PDU set. This ensures the PDU will be sent over EUTRA RLC as well, for steps 4A and 8D. - Update v_ExpectedPdu after sending DL PDCP data at step 8B - Wait for Coordination message from EUTRA before step #10 - At step 10, index of the PDCP SDU is set as 2 instead of 3 - At Step 11, index of the expected SDU is modified to 2 instead of 3 - Added call to Stop Wait Timer started to receive PDCP data at step 11 - At Step 13, index of the expected SDU is modified to 2 instead of 3 - Added a PASS Verdict at Wait Time out implemented for Step 13 |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before

function f_TC_7_1_3_5_5_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCell, integer p_DRB_Id) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo;

………………………………………………….

//@siclog "Step 8B" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[2], v_TimingInfo);

……………………………………………………………………

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

//@siclog "Step 10" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity..

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[3], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

t_WaitForPdu.start;

//Sujith@71355

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 11");

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 PDCP DATA PDU not received");

}

}

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

//@siclog "Step 13" siclog@

// Does UE transmits a PDCP Data PDU on the AM RLC primary or secondary entity  in next five seconds?

t_WaitForPdu.start;

alt{

#Remove

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

//@siclog "Step 11-12" siclog@

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , v_RlcBearerRoutingPcell, -, {v_ExpectedPdu})) -> value v_Secondary

{ //UE transmits a PDCP Data PDU on the AM RLC secondary entity.

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[] t_WaitForPdu.timeout{

//Do nothing

}

}}

After change

function f_TC_7_1_3_5_5_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCell, integer p_DRB_Id) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo;

………………………………………………….

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

…………………………...................................

//@siclog "Step 8B" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

tsc_NR_DRB2,

crs_RlcBearerRouting_Eutra(eutra_Cell1),

cs_NR_PDCP_RbConfig_TransparentMode,-,-,-,-,true)};

f_NR_SS_CommonRadioBearerConfig(p_PSCell, v_SS_Drb_ConfigList);

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[2], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

……………………………………………………………………

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

}

//@siclog "Step 10" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity..

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[2], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[2],"Step 11");

t_WaitForPdu.stop;

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 PDCP DATA PDU not received");

}

}

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

//@siclog "Step 13" siclog@

// Does UE transmits a PDCP Data PDU on the AM RLC primary or secondary entity  in next five seconds?

t_WaitForPdu.start;

alt{

//@siclog "Step 11-12" siclog@

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , crs_RlcBearerRouting_Eutra(eutra_Cell1), -, {v_ExpectedPdu})) -> value v_Secondary

{ //UE transmits a PDCP Data PDU on the AM RLC seondary entity.

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[2],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[] t_WaitForPdu.timeout{

//Do nothing

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

}

}

## 2.4 Change 4

| Function name | f_NR_GetRadioBearerConfig_FullAM() |
| --- | --- |
| Reason for change | See change 3.1 |
| Summary of change | Add parameter to control the use of cs_NR_RLC_TestModeConfig_NotACK_NextRLC_PDU |
| TTCN module | NR_Common_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_NR_GetRadioBearerConfig_FullAM(NR_CellId_Type p_NR_CellId,

DRB_Identity p_DrbId,

template (value) RlcBearerRouting_Type p_LinkToOtherCellGroup := crs_RlcBearerRouting_None,

template (omit)  NR_PDCP_RbConfig_Type p_PDCP_RbConfig := cs_NR_PDCP_RbConfig_ParamsDRB,

template (omit)  NR_RLC_TestModeConfig_Type p_NR_RLC_TestModeConfig := omit,   // Testmodeconfig

template (omit)  SDAP_Configuration_Type p_Sdap := omit,

template (omit)  NR_LogicalChannelId_Type p_LogicalChannelId := omit,  //when a non default value is applied

template (omit)  T_PollRetransmit p_T_PollRetransmit := omit

)

runs on NR_BASE_PTC return template (value) NR_RadioBearer_Type

{ //@sic R5s200690: added  p_T_PollRetransmit sic@

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId);

var template (omit) NR_LogicalChannelId_Type v_LogicalChannelId := f_NR_LogicalChannelId_DRB(p_DrbId);  //Default value

var template (value) NR_PDCP_Configuration_Type v_PDCP_Configuration := cs_NR_PDCP_Configuration_RBTerminating(p_PDCP_RbConfig, p_LinkToOtherCellGroup);

var template (value) NR_RlcBearerConfig_Type v_RlcBearerConfig;

if (isvalue (p_LogicalChannelId)) { //Use the parameterised value when passed - non default value

v_LogicalChannelId := p_LogicalChannelId;

}

v_RlcBearerConfig := cs_NR_RlcBearerConfig_AM(v_IsFR1, p_NR_RLC_TestModeConfig, v_LogicalChannelId);

if (isvalue(p_T_PollRetransmit)) {

v_RlcBearerConfig.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := p_T_PollRetransmit;

}

return cs_NR_SS_DRB_Config(p_DrbId, v_PDCP_Configuration, v_RlcBearerConfig, p_Sdap);

}

After

function f_NR_GetRadioBearerConfig_FullAM(NR_CellId_Type p_NR_CellId,

DRB_Identity p_DrbId,

template (value) RlcBearerRouting_Type p_LinkToOtherCellGroup := crs_RlcBearerRouting_None,

template (omit)  NR_PDCP_RbConfig_Type p_PDCP_RbConfig := cs_NR_PDCP_RbConfig_ParamsDRB,

template (omit)  NR_RLC_TestModeConfig_Type p_NR_RLC_TestModeConfig := omit,   // Testmodeconfig

template (omit)  SDAP_Configuration_Type p_Sdap := omit,

template (omit)  NR_LogicalChannelId_Type p_LogicalChannelId := omit,  //when a non default value is applied

template (omit)  T_PollRetransmit p_T_PollRetransmit := omit,

boolean p_RLCNack := false

)

runs on NR_BASE_PTC return template (value) NR_RadioBearer_Type

{ //@sic R5s200690: added  p_T_PollRetransmit sic@

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId);

var template (omit) NR_LogicalChannelId_Type v_LogicalChannelId := f_NR_LogicalChannelId_DRB(p_DrbId);  //Default value

var template (value) NR_PDCP_Configuration_Type v_PDCP_Configuration := cs_NR_PDCP_Configuration_RBTerminating(p_PDCP_RbConfig, p_LinkToOtherCellGroup);

var template (value) NR_RlcBearerConfig_Type v_RlcBearerConfig;

if (isvalue (p_LogicalChannelId)) { //Use the parameterised value when passed - non default value

v_LogicalChannelId := p_LogicalChannelId;

}

v_RlcBearerConfig := cs_NR_RlcBearerConfig_AM(v_IsFR1, p_NR_RLC_TestModeConfig, v_LogicalChannelId);

if (isvalue(p_T_PollRetransmit)) {

v_RlcBearerConfig.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := p_T_PollRetransmit;

}

if (p_RLCNack == true)

{

v_RlcBearerConfig.Config.Rlc.TestMode := cs_NR_RLC_TestModeConfig_NotACK_NextRLC_PDU;

}

return cs_NR_SS_DRB_Config(p_DrbId, v_PDCP_Configuration, v_RlcBearerConfig, p_Sdap);

}

## 2.5 Change 5

| Function name | f_TC_7_1_3_5_5_ENDC_NR() |
| --- | --- |
| Reason for change | See change 3.1 |
| Summary of change | Pass new parameter as true in call to f_NR_GetRadioBearerConfig_FullAM(), see change 4 |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before

function f_TC_7_1_3_5_5_ENDC_NR() runs on ENDC_NR_PTC

{//PDCP Duplication

var NR_PDCP_SS_State_Type v_NR_PDCP_State;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Initialize PDCP Parameters

f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18);

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

tsc_NR_DRB2,

crs_RlcBearerRouting_Eutra(eutra_Cell1),

cs_NR_PDCP_RbConfig_TransparentMode)};

v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(tsc_NR_DRB2, cs_NR_PDCP_Config_UlPath(infinity_,len18bits, len18bits))};

……………………………………….

……………………………………….

}

After

function f_TC_7_1_3_5_5_ENDC_NR() runs on ENDC_NR_PTC

{//PDCP Duplication

var NR_PDCP_SS_State_Type v_NR_PDCP_State;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Initialize PDCP Parameters

f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18);

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

tsc_NR_DRB2,

crs_RlcBearerRouting_Eutra(eutra_Cell1),

cs_NR_PDCP_RbConfig_TransparentMode,-,-,-,-,true)};

v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(tsc_NR_DRB2, cs_NR_PDCP_Config_UlPath(infinity_,len18bits, len18bits))};

……………………………………….

……………………………………….

}

# Branches executed

The test case was executed on EN-DC band combination 3_n78.

# Execution Log File

## MediaTek MT6885

The MediaTek MT6885 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_3_5_5_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_3_5_5_PIXIT.html

# References

| [1] | R5s210375 : EN-DC FR1 : Supporting information for addition of NR PDCP test case 7.1.3.5.5 |
| --- | --- |