3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211394

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1870 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 7.1.1.3.2b in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 NR5GC test case 7.1.1.3.2b to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 7.1.1.3.2b required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2b.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	11

2.3	Change 3	14

3	Branches executed	15

4	Execution Log Files	15

4.1	HiSilicon Balong 5000	15

5	References	16

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.3.2b which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	7.1.1.3.2b

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_2b_NR5GC |
| --- | --- |
| Reason for change | - Modify parameters for L2 should be configured before establishing NR cell. - It shall be start UL grant to receive reconfiguration complete. - For checking total number of octets of UL Rx by function f_TC_7_1_1_3_2_Step3 which is calculated with 320 bytes, but not all DRB data is 320 bytes. - PUSCH-TimeDomainResourceAllocationList 0 is set for 4 symbol, and PUSCH-TimeDomainResourceAllocationList 1 is set for 14 symbol, which is not match with resource allocation in function f_TC_7_1_1_3_2b_NR_TestBody_Loop. Note: it supportted with a prose CR. |
| Summary of change | - Move f_NR_CellInfo_SetMAC_CellGroupConfigL2 before f_NR_CellConfig_Def. - Add f_NR_ULGrantConfiguration_Start after f_NR_SendRRCReconfiguration. - Modify the input parameter of function f_TC_7_1_1_3_2b_NR_TestBody_Loop for step6-8. - Modify the input parameter of funciton f_TC_7_1_1_3_2b_NR_TestBody_Loop at step1-3 and step 6-8. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB1; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB2; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB3; var boolean v_IsFR1 := false; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_RRC_MsgRcvd := false; var boolean v_DRB_Rcvd := false; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template(value) SpCellConfig v_SpCellConfig ; var template(value) BWP.locationAndBandwidth v_LocationAndBandwidth; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var integer v_BwpSize ; var integer v_RIV; var SubFrameTiming_Type v_Timing; timer t_Watchdog := 1.0; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_3_2b(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // Now that at least one PDU session has been established, get all DRB ids required v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(3); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; v_DRBId3 := v_ListOfDRBs[2]; v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { v_NR_AllowedSCS_List_DRB1 := cs_NR_AllowedSCS_List_15_30; v_NR_AllowedSCS_List_DRB2 := cs_NR_AllowedSCS_List_60; v_NR_AllowedSCS_List_DRB3 := cs_NR_AllowedSCS_List_15_30_60; } else { v_NR_AllowedSCS_List_DRB1 := cs_NR_AllowedSCS_List_60_120; v_NR_AllowedSCS_List_DRB2 := cs_NR_AllowedSCS_List_60; v_NR_AllowedSCS_List_DRB3 := cs_NR_AllowedSCS_List_120; }; v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3) }; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,v_NR_AllowedSCS_List_DRB1)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,v_NR_AllowedSCS_List_DRB2)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,v_NR_AllowedSCS_List_DRB3)) ; v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; //@sic R5-213455 sic@ v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId3, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit))}; //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 sic@ f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig v_DRBToAddModList, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@sic steps 1-3 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,360,4160,0,16000); //@sic steps 4-5 sic@ v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,ms0p02)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,ms0p5)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,ms0p5)) ; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, v_RLC_BearerConfigList); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 6-8 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 1,25,50,20,420,0,4000,8000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServinCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { case(kHz15) { v_ConfigGrant_Period := sym20x14} case(kHz30) { v_ConfigGrant_Period := sym40x14} case(kHz60) { v_ConfigGrant_Period := sym80x14} case(kHz120) { v_ConfigGrant_Period := sym160x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16))); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,0,4160,0,16000); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB1; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB2; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB3; var boolean v_IsFR1 := false; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_RRC_MsgRcvd := false; var boolean v_DRB_Rcvd := false; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template(value) SpCellConfig v_SpCellConfig ; var template(value) BWP.locationAndBandwidth v_LocationAndBandwidth; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var integer v_BwpSize ; var integer v_RIV; var SubFrameTiming_Type v_Timing; timer t_Watchdog := 1.0; //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_Init_7_1_1_3_2b(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // Now that at least one PDU session has been established, get all DRB ids required v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(3); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; v_DRBId3 := v_ListOfDRBs[2]; v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { v_NR_AllowedSCS_List_DRB1 := cs_NR_AllowedSCS_List_15_30; v_NR_AllowedSCS_List_DRB2 := cs_NR_AllowedSCS_List_60; v_NR_AllowedSCS_List_DRB3 := cs_NR_AllowedSCS_List_15_30_60; } else { v_NR_AllowedSCS_List_DRB1 := cs_NR_AllowedSCS_List_60_120; v_NR_AllowedSCS_List_DRB2 := cs_NR_AllowedSCS_List_60; v_NR_AllowedSCS_List_DRB3 := cs_NR_AllowedSCS_List_120; }; v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3) }; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,v_NR_AllowedSCS_List_DRB1)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,v_NR_AllowedSCS_List_DRB2)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,v_NR_AllowedSCS_List_DRB3)) ; v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; //@sic R5-213455 sic@ v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId3, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit))}; //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig v_DRBToAddModList, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@sic steps 1-3 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,360,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //@sic steps 4-5 sic@ v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,ms0p02)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,ms0p5)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,ms0p5)) ; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, v_RLC_BearerConfigList); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_ULGrantConfiguration_Start(nr_Cell1); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 6-8 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 1,25,50,20,860,0,8000,16000); //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServinCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { case(kHz15) { v_ConfigGrant_Period := sym20x14} case(kHz30) { v_ConfigGrant_Period := sym40x14} case(kHz60) { v_ConfigGrant_Period := sym80x14} case(kHz120) { v_ConfigGrant_Period := sym160x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16))); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,0,4160,0,16000); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 2

| Function name | f_TC_7_1_1_3_2b_NR_TestBody_Loop |
| --- | --- |
| Reason for change | - When p_TimeDomainResourceAssignment is set omit which for 4 symbol but 9224 is used for 14 symbol. - Modify for change1 point 3. |
| Summary of change | - Modify input parameter p_TimeDomainResourceAssignment to cs_NR_DciCommon_TimeDomainResourceAssignment('1'B), and modify the way of configuration for variable v_TimeDomainResourceAssignment and v_NR_ResourceAllocation. - Remove the configuration for 157 bytes PDCP SDU. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3,//@sic R5200634 sic@ template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit) runs on NR_BASE_PTC { var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var template (value) NR_PDCP_SDU_Type v_NR_PDCP_SDU := crs_NR_PDCP_SDU_317B ; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; timer t_WatchDog := 1.0; // Local wait Timer if (isvalue (p_TimeDomainResourceAssignment)) //Loop 2 { v_NR_PDCP_SDU := crs_NR_PDCP_SDU_157B ; v_NR_ResourceAllocation := cs_NR_ResourceAllocation(23,25) ; //PUSCH FR1: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 0, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //PUSCH FR2: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 1, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //--TBS-----L_RBs-----I_MCS- // 4608 23 25 }else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); } f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320/160-octet RLC SDUs on DRB1 // N2 320/160-octet RLC SDUs on DRB2 // N3 320/160-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); //@siclog "Step 2" siclog@ // UL Grant of D[j] every T1ms for a duration of T2ms if (p_T2 != 0) { v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 300); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_ResourceAllocation, p_T1, (p_T2 / p_T1), p_TimeDomainResourceAssignment); }; //@siclog "Step 3" siclog@ // RLC SDUs back to SS/check total number of octets of UL Rx f_TC_7_1_1_3_2_Step3(p_D1, p_D2, p_D3,p_NR_DRB_Id1,p_NR_DRB_Id2,p_NR_DRB_Id3); f_NR_HarqErrorHandling_Set(ignore); // Stop 'Harq Errors not tolerant' f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config t_WatchDog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, (p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3))) { repeat; } [] t_WatchDog.timeout { f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); } }; }

After Change

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3,//@sic R5200634 sic@ template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit) runs on NR_BASE_PTC { var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var template (value) NR_PDCP_SDU_Type v_NR_PDCP_SDU := crs_NR_PDCP_SDU_317B ; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; var template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type v_TimeDomainResourceAssignment; timer t_WatchDog := 1.0; // Local wait Timer if (isvalue (p_TimeDomainResourceAssignment)) //Loop 2 { v_TimeDomainResourceAssignment := p_TimeDomainResourceAssignment; v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); }else { REMOVE//v_NR_PDCP_SDU := crs_NR_PDCP_SDU_157B ; v_NR_ResourceAllocation := cs_NR_ResourceAllocation(23,25) ; v_TimeDomainResourceAssignment := cs_NR_DciCommon_TimeDomainResourceAssignment('0'B); //PUSCH FR1: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 0, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //PUSCH FR2: MCS Index table = 5.1.3.1-1, Mapping type = TypeA, dmrs-AdditionalPosition = 1, number of CDM groups = 1, modulation order <= 6, PUSCH-duration = 4 //--TBS-----L_RBs-----I_MCS- // 4608 23 25 } f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320/160-octet RLC SDUs on DRB1 // N2 320/160-octet RLC SDUs on DRB2 // N3 320/160-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); //@siclog "Step 2" siclog@ // UL Grant of D[j] every T1ms for a duration of T2ms if (p_T2 != 0) { v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 300); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_ResourceAllocation, p_T1, (p_T2 / p_T1), v_TimeDomainResourceAssignment); }; //@siclog "Step 3" siclog@ // RLC SDUs back to SS/check total number of octets of UL Rx f_TC_7_1_1_3_2_Step3(p_D1, p_D2, p_D3,p_NR_DRB_Id1,p_NR_DRB_Id2,p_NR_DRB_Id3); f_NR_HarqErrorHandling_Set(ignore); // Stop 'Harq Errors not tolerant' f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config t_WatchDog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, (p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3))) { repeat; } [] t_WatchDog.timeout { f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); } }; }

## Change 3

| Function name | f_NR_Init_7_1_1_3_2b |
| --- | --- |
| Reason for change | - Change for chagne1 point4. |
| Summary of change | - Repleace typeA with typeB. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR_Init_7_1_1_3_2b(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var integer v_K2 := 4; var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_StartSymbolAndLength := 52;} v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); }

After Change

function f_NR_Init_7_1_1_3_2b(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var integer v_K2 := 4; var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_StartSymbolAndLength := 52;} v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); }

# Branches executed

NR Band 41, nea2, Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610 and Keysight S8704A Protocol Conformance Toolset . The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_7_1_1_3_2b_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(HISI).xml

\ Pixit(TC_7_1_1_3_2b_NR5GC).xml

# References

| [1] | R5s210864: Supporting information for agreement of NR5GC test case 7.1.1.3.2b in FR1 |
| --- | --- |