3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211395

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1873 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.3.1.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The agreed prose CR R5-211389 has not been implemented for this test case. Conseuently a UE which includes SINR measurement results could fail the test case at step 8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove use of cr_NR_MeasQuantityResults_RsrpRsrq at step 8, so UE is allowed to include SINR results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.16.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_1_16_TestBody |
| --- | --- |
| Reason for change | The agreed prose CR R5-211389 has not been implemented for this test case. Conseuently a UE which includes SINR measurement results could fail the test case at step 8. |
| Summary of change | Remove use of cr_NR_MeasQuantityResults_RsrpRsrq at step 8, so UE is allowed to include SINR results. |
| TTCN module | RRC_Measurement_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_3_1_16_TestBody() runs on NR_BASE_PTC { ...... //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2? f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 1, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour2, cr_NR_MeasQuantityResults_Rsrp)}), -, -, -, -, -, cr_NR_MeasQuantityResults_RsrpRsrq //@sic R5-204437, R5s201474 sic@ ); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 8"); //@siclog "Steps 9-10 " siclog@ //SS re-adjusts the cell-specific reference signal level according to row "T2" . //Wait and ignore MeasurementReport messages for 10s to allow change of power levels for the neighbour NR Cell and UE measurement f_NR_ChangeThePower_And_IgnorePeriodicalMeasurmentReport(v_CellPowerLevels[2], 10.0); //@sic R5s201099, R5-203657 sic@ //@siclog "Steps 11 " siclog@ //Check: Does the UE transmit a MeasurementReport message within the next 10s? f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any); //@sic R5s201099, R5-203657 sic@ } // End of f_TC_8_1_3_1_16_TestBody()

After Change

...... //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2? f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 1, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour2, cr_NR_MeasQuantityResults_Rsrp)}), -, -, -, -, -, - //@sic R5-204437, R5s201474 sic@ //KS R5s21xxx ); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 8"); //@siclog "Steps 9-10 " siclog@ //SS re-adjusts the cell-specific reference signal level according to row "T2" . //Wait and ignore MeasurementReport messages for 10s to allow change of power levels for the neighbour NR Cell and UE measurement f_NR_ChangeThePower_And_IgnorePeriodicalMeasurmentReport(v_CellPowerLevels[2], 10.0); //@sic R5s201099, R5-203657 sic@ //@siclog "Steps 11 " siclog@ //Check: Does the UE transmit a MeasurementReport message within the next 10s? f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any); //@sic R5s201099, R5-203657 sic@ } // End of f_TC_8_1_3_1_16_TestBody()