3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211404

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1885 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 7.1.3.5.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR-DC TC 7.1.3.5.2 PDCP Uplink Rouiting / Split DRB |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR-DC test case 7.1.3.5.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	9

2.4	Change 4	11

2.5	Change 5	12

2.6	Change 6	13

2.7	Change 7	15

2.8	Change 8	16

2.9	Change 9	17

3	Branches Executed	21

4	Execution Log Files	21

4.1	Qualcomm SM 8450 + SDX65	21

5	References	21

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.1.3.5.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.3.5.2

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

Change 1,5,6 of R5s210716(Addition of NR-DC test case 8.2.2.4.2 in FR1+FR2 ) are also required in addition to the below changes.

Change 8 of R5s210791 (Addition of NR-DC test case 8.2.5.3.2 in FR1+FR2) is also required in addition to the below changes.

## Change 1

| Function name | f_TC_7_1_3_5_2_NR5GC() |
| --- | --- |
| Reason for change | NR-1 is applied for system information combination #draft proseCR would be raise during RAN5-92-e meeting TS 38.523 Section 7.1.3.0 Indicates UE TEST LOOP MODE A during preamble. NULL Ciphering must be set only for SCG. PDCP Initialization is getting applied to “tsc_NR_DRB2” by default which is not the default DRB id for SCG. The function f_NR5GC_RRC_ConnectedState3N_NRDC has a bug where if bearer type is MCG_Split, the PDCP input parameter is not used. Instead it uses default PDCP parameters. |
| Summary of change | Changed system info to NR_1 when initializing the NR cells. Configured TEST_LOOPModeA_ON during Preamble. Changed Ciphering algorithm of the SCG bearer to NULL Passed SCG DRB id as input parameter to f_NR_PDCP_Init Added section that sends another RRC Reconfiguration message modifying PDCP DiscardTimer and ul-DataSplitThreshold. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var integer v_SCG_DRB_Id; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_4,{nr_Cell1, nr_Cell10}); //=> to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18); //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); //run the preamble as normal and bring back UE to IDLE f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100)); //=> Will bring UE to connected state, The SCG DRB is configured in normal mode v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); //Reconfigure SCG DRB PDCP in Test Mode in SS v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, nr_Cell10, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var integer v_SCG_DRB_Id; var RadioBearerConfig v_RadioBearerConfig2; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //=> to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100)); //=> Will bring UE to connected state, The SCG DRB is configured in normal mode v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18, v_SCG_DRB_Id); //Reconfigure SCG DRB PDCP in Test Mode in SS v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, v_SCG_DRB_Id, -, -, cs_NR_PDCP_Config_UlPathSCG(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,b100))}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, nr_Cell10, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

## Change 2

| Function name | f_TC_7_1_3_5_2_NR_TestBody |
| --- | --- |
| Reason for change | The template “cs_NR_PDCP_Config_UlPath” used configures PDCP Config with LCID of tsc_NR_DRB2 which is not the same as the SCG DRB id At step 5-6, RRC Reconfiguration message is sent on SCG. |
| Summary of change | A new template is created (see Section 2.3) and is used in this function instead. Used nr_Cell1 to send Reconfiguration message |
| TTCN module | RRC_ReconfigFailure_NRDC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_ScellId, integer p_DRB_Id) runs on NR_BASE_PTC { … //@sic R5-204426 sic@ //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes (Note 1). if (v_IsENDC){ //@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b0))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); } else {//@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,b0))}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (p_ScellId, -,-, v_V1530Ext); } … }

After Change:

function f_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_ScellId, integer p_DRB_Id) runs on NR_BASE_PTC { … //@sic R5-204426 sic@ //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes (Note 1). if (v_IsENDC){ //@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPathSCG(ms500, len18bits, len18bits,-,b0))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); } else {//@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,b0))}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); } … } }

## Change 3

| Function name | New template cs_NR_PDCP_Config_UlPathSCG |
| --- | --- |
| Reason for change | The template “cs_NR_PDCP_Config_UlPath” used configures PDCP Config with LCID of tsc_NR_DRB2 which is not the same as the SCG DRB id |
| Summary of change | Created a new template “cs_NR_PDCP_Config_UlPathSCG” based on the “cs_NR_PDCP_ConfigUlPath” but where the LCID is that of tsc_NR_DRB3 (Default SCG Bearer) |
| TTCN module | NR_PDCP_Common.ttcn |
| MCC160 Comment |  |

Before Change:

After Change:

template (value) PDCP_Config cs_NR_PDCP_Config_UlPathSCG(NR_PDCP_DiscardTimer_Type p_DiscardTimer, NR_PDCP_SN_UL_Type p_SN_UL, NR_PDCP_SN_DL_Type p_SN_DL, CellGroupId p_CellGroup := tsc_NR_CellGroupId_SCG, UL_DataSplitThreshold p_UlDataSplitThreshold := infinity_, template (omit) NR_PDCP_CipheringDisabled_Type p_CipheringDisabled := omit, //@sic R5s200701 sic@ boolean p_Pdcp_Duplication := false) := /* @status APPROVED (ENDC) */ cs_38508_NR_PDCP_Config (cs_38508_PDCP_Config_DRB(p_DiscardTimer, p_SN_UL, p_SN_DL), cs_NR_PDCP_MoreThanOneRLC(cs_NR_PDCP_PrimaryPath(p_CellGroup, f_NR_LogicalChannelId_DRB(tsc_NR_DRB3)), p_UlDataSplitThreshold, p_Pdcp_Duplication), -, p_CipheringDisabled);

# Branches Executed

The test case was executed on NR band DC_n79A_n257A.

# Execution Log Files

## Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_3_5_2_LOG.xml

- PICS/PIXIT parameter file:

TC_7_1_3_5_2_PICS.txt

# References

| [1] | R5s210889 |
| --- | --- |