3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211483

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2118 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC test cases 8.1.3.1.12 and 8.1.3.1.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After implementation of R5-216179, UE is not able to measure the SSB for inter-frequency cell as the MeasGapOffset is not correct in wk37 TTCN. In the current TTCN, MeasGapOffset=39. No SSB will fall in this offset. According to 38.508 table 4.6.3-70, When SFNOffset of inter frequency neighbour cell is odd number then MeasGapOffset=9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new templete cs_38508_MeasGapConfig_UE_oddSFN has been added if SFNoffset of inter frequency neighbour cell is odd number. cs_38508_MeasGapConfig_UE_oddSFN has been added to function f_NR_GenerateMeasurementConfigNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.12.NR5GC, 8.1.3.1.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2            Change 2	3

2.3            Change 3	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_1_12_NR5GC |
| --- | --- |
| Reason for change | After implementation of R5-216179, UE is not able to measure the SSB for inter-frequency cell as the MeasGap offset is not correct in TTCN. In the current TTCN, MeasGapOffset=39. No SSB will fall in this offset. According to 38.508 table 4.6.3-70, When SFNOffset of inter frequency neighbour cell is odd number then MeasGapOffset=9. |
| Summary of change | A new templete cs_38508_MeasGapConfig_UE_oddSFN has been added if SFNoffset of inter frequency neighbour cell is odd number. |
| TTCN module | NR_Measurement_Templates.ttcn |
| MCC160 Comment |  |

New Template

template (value) GapConfig cs_38508_GapConfig_oddSFN := cs_NR_GapConfig(9, ms6, ms40, ms0); template (value) MeasGapConfig cs_38508_MeasGapConfig_UE_oddSFN (template (value) GapConfig p_GapConfig := cs_38508_GapConfig_oddSFN):= { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ gapFR2 := omit, gapFR1 := omit, gapUE := { setup := p_GapConfig } };

## Change 2

| Function name | f_TC_8_1_3_1_12_ TestBody |
| --- | --- |
| Reason for change | After implementation of R5-216179, UE is not able to measure the SSB for inter-frequency cell as the MeasGapOffset is not correct in wk37 TTCN. In the current TTCN, MeasGapOffset=39. No SSB will fall in this offset. According to 38.508 table 4.6.3-70, When SFNOffset of inter frequency neighbour cell is odd number then MeasGapOffset=9. |
| Summary of change | cs_38508_MeasGapConfig_UE_oddSFN has been added to function f_NR_GenerateMeasurementConfigNR |
| TTCN module | RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_3_1_12_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerLevels_Type v_CellPowerLevels; var NR_AbsoluteCellPower_Type v_CellPower_ServingInitial_FR1; //@sic R5s200884 sic@ var NR_AbsoluteCellPower_Type v_CellPower_Suitable_FR1; var integer v_Noc; var template(value) NR_CellNoiseLevelList_Type v_CellNoiseList; var SINR_Range v_SinrThres1; var SINR_Range v_SinrThres2; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; v_CellPower_ServingInitial_FR1 := -82; v_CellPower_Suitable_FR1 := -94; v_Noc := -94; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingInitial_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_Suitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerLevels := {v_CellPowerList_AtT0, v_CellPowerList_AtT1, v_CellPowerList_AtT2}; f_NR_SetCellPowerList(v_CellPowerLevels[0]); v_CellNoiseList := {cs_CellNoiseLevel(nr_Cell1, v_Noc),cs_CellNoiseLevel(nr_Cell2, v_Noc)} //@sic R5s201385 sic@ f_NR_ConfigureAndActivateVNG_MultipleCells(v_CellNoiseList); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for intra-frequency event A4 (measId 1). //The UE transmits an RRCReconfigrationComplete message v_SinrThres1 := 68; //@sic R5s200884 sic@ v_SinrThres2 := 47; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, -, 2),cs_MeasObjectId2(nr_Cell3, -, 2)}; v_ReportConfigList := {cs_NR_ReportConfigEventA5_SINR(tsc_NR_IdReportConfigId1, v_SinrThres1, v_SinrThres2, r1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj2_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE); //@sic R5-216179 sic@ f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); fl_TC_8_1_3_1_x_SimultaneousEvents_CommonProcedure(v_CellPowerLevels, true); //@sic R5-204341 sic@ f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG(v_CellNoiseList)); } // End of f_TC_8_1_3_1_12_TestBody() …

After Change

function f_TC_8_1_3_1_12_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerLevels_Type v_CellPowerLevels; var NR_AbsoluteCellPower_Type v_CellPower_ServingInitial_FR1; //@sic R5s200884 sic@ var NR_AbsoluteCellPower_Type v_CellPower_Suitable_FR1; var integer v_Noc; var template(value) NR_CellNoiseLevelList_Type v_CellNoiseList; var SINR_Range v_SinrThres1; var SINR_Range v_SinrThres2; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; v_CellPower_ServingInitial_FR1 := -82; v_CellPower_Suitable_FR1 := -94; v_Noc := -94; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingInitial_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_Suitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerLevels := {v_CellPowerList_AtT0, v_CellPowerList_AtT1, v_CellPowerList_AtT2}; f_NR_SetCellPowerList(v_CellPowerLevels[0]); v_CellNoiseList := {cs_CellNoiseLevel(nr_Cell1, v_Noc),cs_CellNoiseLevel(nr_Cell2, v_Noc)} //@sic R5s201385 sic@ f_NR_ConfigureAndActivateVNG_MultipleCells(v_CellNoiseList); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for intra-frequency event A4 (measId 1). //The UE transmits an RRCReconfigrationComplete message v_SinrThres1 := 68; //@sic R5s200884 sic@ v_SinrThres2 := 47; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, -, 2),cs_MeasObjectId2(nr_Cell3, -, 2)}; v_ReportConfigList := {cs_NR_ReportConfigEventA5_SINR(tsc_NR_IdReportConfigId1, v_SinrThres1, v_SinrThres2, r1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj2_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE_oddSFN); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); fl_TC_8_1_3_1_x_SimultaneousEvents_CommonProcedure(v_CellPowerLevels, true); //@sic R5-204341 sic@ f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG(v_CellNoiseList)); } // End of f_TC_8_1_3_1_12_TestBody()

## Change 3

| Function name | f_TC_8_1_3_1_11_ TestBody |
| --- | --- |
| Reason for change | As describe in R5-216179 ,the MeasGapOffset, mgrp,mgta are not correct in wk37 TTCN. According to 38.508 table 4.6.3-70, When SFNOffset of inter frequency neighbour cell is odd number then MeasGapOffset=9, |
| Summary of change | cs_38508_MeasGapConfig_UE_oddSFN has been added to function f_NR_GenerateMeasurementConfigNR |
| TTCN module | RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_3_1_11_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerLevels_Type v_CellPowerLevels; var NR_AbsoluteCellPower_Type v_CellPower_Suitable_FR1; var integer v_Noc; var template(value) NR_CellNoiseLevelList_Type v_CellListWithVNG; var MeasTriggerQuantityOffset.rsrq v_RsrqThres; //@sic R5-202630, R5s200872 sic@ var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; //var integer v_GapOffset; //var GapConfig.mgl v_Mgl; // var GapConfig.mgrp v_Mgrp; //var GapConfig.mgta v_Mgta; // TCD 8.1.3.1.11_12 Anritsu v_CellPower_Suitable_FR1 := -100; v_Noc := -94; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_Suitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerLevels := {v_CellPowerList_AtT0, v_CellPowerList_AtT1, v_CellPowerList_AtT2}; f_NR_SetCellPowerList(v_CellPowerLevels[0]); v_CellListWithVNG := {cs_CellNoiseLevel(nr_Cell1, v_Noc),cs_CellNoiseLevel(nr_Cell2, v_Noc)} //@sic R5s201385, R5-205718 sic@ f_NR_ConfigureAndActivateVNG_MultipleCells(v_CellListWithVNG); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for intra-frequency event A4 (measId 1). //The UE transmits an RRCReconfigrationComplete message v_RsrqThres := -6; //@sic R5-202630, R5s200872 sic@ v_GapOffset := 9; //@sic R5s201124, R5-204443, R5-205373 sic@ v_Mgl := ms6; v_Mgrp := ms160; if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_Mgta := ms0dot5; } else { v_Mgta := ms0dot25; } v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, -, 2),cs_MeasObjectId2(nr_Cell3, -, 2)}; v_ReportConfigList := {cs_NR_ReportConfigEventA3_RSRQ(tsc_NR_IdReportConfigId1, v_RsrqThres, r1, 0)}; //@sic R5-202630, R5s200872 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj2_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_NR_GapConfig(v_GapOffset, v_Mgl, v_Mgrp, v_Mgta))); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); fl_TC_8_1_3_1_x_SimultaneousEvents_CommonProcedure(v_CellPowerLevels); f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG(v_CellListWithVNG)); } // End of f_TC_8_1_3_1_11_TestBody()

After Change

function f_TC_8_1_3_1_11_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerLevels_Type v_CellPowerLevels; var NR_AbsoluteCellPower_Type v_CellPower_Suitable_FR1; var integer v_Noc; var template(value) NR_CellNoiseLevelList_Type v_CellListWithVNG; var MeasTriggerQuantityOffset.rsrq v_RsrqThres; //@sic R5-202630, R5s200872 sic@ var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; //var integer v_GapOffset; //var GapConfig.mgl v_Mgl; // var GapConfig.mgrp v_Mgrp; //var GapConfig.mgta v_Mgta; // TCD 8.1.3.1.11_12 Anritsu v_CellPower_Suitable_FR1 := -100; v_Noc := -94; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_Suitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerLevels := {v_CellPowerList_AtT0, v_CellPowerList_AtT1, v_CellPowerList_AtT2}; f_NR_SetCellPowerList(v_CellPowerLevels[0]); v_CellListWithVNG := {cs_CellNoiseLevel(nr_Cell1, v_Noc),cs_CellNoiseLevel(nr_Cell2, v_Noc)} //@sic R5s201385, R5-205718 sic@ f_NR_ConfigureAndActivateVNG_MultipleCells(v_CellListWithVNG); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for intra-frequency event A4 (measId 1). //The UE transmits an RRCReconfigrationComplete message v_RsrqThres := -6; //@sic R5-202630, R5s200872 sic@ v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, -, 2),cs_MeasObjectId2(nr_Cell3, -, 2)}; v_ReportConfigList := {cs_NR_ReportConfigEventA3_RSRQ(tsc_NR_IdReportConfigId1, v_RsrqThres, r1, 0)}; //@sic R5-202630, R5s200872 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj2_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE_oddSFN ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); fl_TC_8_1_3_1_x_SimultaneousEvents_CommonProcedure(v_CellPowerLevels); f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG(v_CellListWithVNG)); } // End of f_TC_8_1_3_1_11_TestBody()