3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211489

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2124 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC multilayer test cases 11.4.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the emergency call establishment procedure, there may be race conditions within the UE or SS where consecutive messages on SRB1 and SRB2 can be received out of order at the SRB port. This can occur at steps 7/8 (RRCReconfigurationComplete and PDU SESSION ESTABLISHMETN REQUEST) and steps 12/13 (RRCReconfigurationComplete and PDU SESSION MODIFICATION COMPLETE). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow the uplink signalling messages and steps 7/8 and 12/13 to be received in any order. Note : An associated prose CR on 38.508-1 will be raised at RAN5#93. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_EmergencyCallWithRegistration() |
| --- | --- |
| Reason for change | During emergency call establishment proceudure, the PDU SESSION ESTABLISHMENT REQUEST from the UE (step 8) may arrive at the TTCN before the RRCReconfigurationComplete (step 7) |
| Summary of change | Pass the parameter p_ReceiveForRRCreconfigurationComplete as false in the call to f_NR5GC_RRC_ConnectedState3N_Steps5_8(), so that the PDU SESSION ESTABLISHMENT REQUEST can be handled in f_NR5GC_EmergencyCallPDUSession() if it arrives first (see also change 3). |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 4-7 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active }

After Change

function f_NR5GC_EmergencyCallWithRegistration(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 4-7 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, -, -, -, -, -, -, false); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); // Emergency call now active }

## Change 2

| Function name | f_NR5GC_ModifyPDUSession_AddVoice() |
| --- | --- |
| Reason for change | During emergency call establishment proceudure, the PDU SESSION MODIFICATION COMPLETE from the UE (step 13) may arrive at the TTCN before the RRCReconfigurationComplete (step 12) |
| Summary of change | Add interleave block such that steps 12 and 13 can be handled in either order/ |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_ModifyPDUSession_AddVoice (NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var boolean v_InterworkWithEPS := false; var O1_Type v_EPS_BearerId := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPS_BearerId := hex2oct(int2hex(valueof(f_GetNextEPSBearerId (p_GSM_MobilityInfo, v_GMM_MobilityInfo)), 1) & '0'H); // @sic R5-216161 sic@ } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS, v_EPS_BearerId)})); // @sic R5-216161 sic@ v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, ms80, omit))}); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

After Change

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var boolean v_InterworkWithEPS := false; var O1_Type v_EPS_BearerId := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPS_BearerId := hex2oct(int2hex(valueof(f_GetNextEPSBearerId (p_GSM_MobilityInfo, v_GMM_MobilityInfo)), 1) & '0'H); // @sic R5-216161 sic@ } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS, v_EPS_BearerId)})); // @sic R5-216161 sic@ v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, ms80, omit))}); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, -, -, false); interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp } if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

## Change 3

| Function name | altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq |
| --- | --- |
| Reason for change | During emergency call establishment proceudure, the PDU SESSION ESTABLISHMENT REQUEST from the UE (step 8) may arrive at the TTCN before the RRCReconfigurationComplete (step 7) |
| Summary of change | Allow RRCReconfigurationComplete to be received while waiting for PDU SESION ESTABLISHMENT REQUEST |
| TTCN module | NR5GC_NASSteps.ttc |
| MCC160 Comment |  |

Before Change

altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp); repeat; }

After Change

altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp); repeat; } [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId,tsc_NR_RbId_SRB1 , cr_38508_RRCReconfigurationComplete)) { repeat; } }