3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211506

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2135 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation the function fl_TC_9_3_1_3_NR5GC_TestBody, after transmission of Registration REJ at Step 4 the NR cell power is set to OFF cell immediately. This sometimes result in the last PDUs of Registration REJ being sent out at a low power, nd the UEs are unable to decode thes. This results in the UE discarding the Registration REJ. As a result, the UEs then send a TAU REQ on Eutra cell, instead of an ATTACH. This is an instability and needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed introduce a delay of 100ms after transmission of Registration REJ at Step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_9_3_1_3_NR5GC_TestBody	2

# Corrections required

## Correction to function fl_TC_9_3_1_3_NR5GC_TestBody

| Function name | fl_TC_9_3_1_3_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation the function f_NR5GC_MobileInfo_Init () is called towards the end, because the UE wil be switched OFF and there wont be any DRBs or PDU Sessions existing. It would also make sense to stop all IP handling at this point. This is not yet implemented in TTCN. This results in the Routing tables not being reset, which can cause problems later. This needs to be corrected. |
| Summary of change | It is proposed to call function f_IP_Handling_StopPDN(IP) after the call to function f_NR5GC_MobileInfo_Init(). |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 4" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '00001001'B))))); //@siclog "Step 4A" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5s200539 sic@ f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // @sic R5s201383 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 4" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '00001001'B))))); f_Delay(0.1); //WA#9_3_1_3 //@siclog "Step 4A" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5s200539 sic@ f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // @sic R5s201383 sic@ <<SKIPPED CODE>>