3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211507

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2136 | rev | - | Current version: | 17.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test cases 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 3GPP TS 38.331: "5.3.1.1 RRC connection control RRC connection establishment involves theestablishment of SRB1. The network completes RRC connection establishment priorto completing the establishment of the NG connection, i.e. prior to receivingthe UE context information from the 5GC. Consequently, AS security is notactivated during the initial phase of the RRC connection. During this initialphase of the RRC connection, the network may configure the UE to performmeasurement reporting, but the UE only sends the corresponding measurementreports after successful AS security activation. However, the UE only acceptsare-configuration with sync message when AS security has been activated. Upon receiving the UE contextfrom the 5GC,the RAN activates AS security (both ciphering and integrityprotection) using the initial AS security activation procedure. The RRCmessages to activate AS security (command and successful response) are integrityprotected, while ciphering is started only after completion of the procedure.That is, the response to the message used to activate AS security is notciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs)are both integrity protected and ciphered. After having initiated the initialAS security activation procedure, the network may initiate the establishment ofSRB2 and DRBs, i.e.the network may do this prior to receiving the confirmationof the initial AS security activation from the UE. In any case, the networkwill apply both ciphering and integrity protection for the RRC reconfigurationmessages used to establish SRB2 and DRBs. The network should release the RRCconnection if the initial AS security activation and/ or the radio bearerestablishment fails. Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported. This means that the only existing DRB cannot be released without releasing the RRCConnection in step13 sending the RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configure DRB2 and DRB3 for UE at step11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections requireds	4

2.1	Change 1	4

- Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk37related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

- Corrections requireds

  - Change 1

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | According to 3GPP TS 38.331: "5.3.1.1 RRC connection control RRC connection establishment involves theestablishment of SRB1. The network completes RRC connection establishment priorto completing the establishment of the NG connection, i.e. prior to receivingthe UE context information from the 5GC. Consequently, AS security is notactivated during the initial phase of the RRC connection. During this initialphase of the RRC connection, the network may configure the UE to performmeasurement reporting, but the UE only sends the corresponding measurementreports after successful AS security activation. However, the UE only acceptsare-configuration with sync message when AS security has been activated. Upon receiving the UE contextfrom the 5GC,the RAN activates AS security (both ciphering and integrityprotection) using the initial AS security activation procedure. The RRCmessages to activate AS security (command and successful response) are integrityprotected, while ciphering is started only after completion of the procedure.That is, the response to the message used to activate AS security is notciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs)are both integrity protected and ciphered. After having initiated the initialAS security activation procedure, the network may initiate the establishment ofSRB2 and DRBs, i.e.the network may do this prior to receiving the confirmationof the initial AS security activation from the UE. In any case, the networkwill apply both ciphering and integrity protection for the RRC reconfigurationmessages used to establish SRB2 and DRBs. The network should release the RRCconnection if the initial AS security activation and/ or the radio bearerestablishment fails. Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported. This means that the only existing DRB cannot be released without releasing the RRCConnection in step13 sending the RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND |
| Summary of change | Configure DRB2 and DRB3 for UE at step11. |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5-197057 R5s200044 R5-216177 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get();

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs();

var template (omit) NR_DRBInfo_Type v_DRBInfo;

var integer v_DRBId;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var NR_RadioBearerList_Type v_SS_Config;

…

// @siclog "Steps 9/10 - Void" siclog@

// Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC

f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)});

f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))});

// DRB1 reconfigured during reconfiguration

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config});

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config)});

// Build RLC-BearerConfig for SRB2 and DRB

v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true);

// There should be 2 entries and both of them need to update the re-establishRLC flag

v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_;

v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_;

// @siclog "Steps 11-12" siclog@

// The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration.

// Check: Does the UE transmit an RRCReconfigurationComplete message?

f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1,

v_RadioBearerConfig,

{v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier

v_CellGroupConfig,

omit,

tsc_NR_RbId_SRB1); // @sic R5s190388 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @sic R5s190936 sic@

…

}//end of f_TC_8_1_2_1_1_NR5GC_TestBody

After change

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5-197057 R5s200044 R5-216177 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get();

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs();

var template (omit) NR_DRBInfo_Type v_DRBInfo;

var integer v_DRBId;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var NR_RadioBearerList_Type v_SS_Config;

…

// @siclog "Steps 9/10 - Void" siclog@

// Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC

f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)});

f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))});

// DRB1 reconfigured during reconfiguration

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config});

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config),cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, 3, cs_38508_PDCP_Config)});

// Build RLC-BearerConfig for SRB2 and DRB

v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true);

// There should be 2 entries and both of them need to update the re-establishRLC flag

v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_;

v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_;

// @siclog "Steps 11-12" siclog@

// The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration.

// Check: Does the UE transmit an RRCReconfigurationComplete message?

f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1,

v_RadioBearerConfig,

{v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier

v_CellGroupConfig,

omit,

tsc_NR_RbId_SRB1); // @sic R5s190388 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @sic R5s190936 sic@

…

}//end of f_TC_8_1_2_1_1_NR5GC_TestBody