3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211570

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0752 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Supplementary Services test case 8.37 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To verify IMS 5GS test case 8.37 using IWD 21wk37 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS Supplementary Services test case 8.37 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.37.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2	Branches executed	5

3	Execution Log Files	5

Qualcomm	5

4	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 8.37 which is part of the iwd-TTCN3-B2020-09_D21wk37.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	8.37

ATS Version:	iwd-TTCN3-B2020-09_D21wk37.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:

Verification Status:	PASS

# Corrections required

## Correction to function f_TC_8_37_NR5GC_IMS1

| Function name | f_TC_8_37_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In current TTCN implementation, expected matching BYE template set by SS will set incorrect RouteBody as well as to and from field as no dialog is passed as parameter in function f_IMS_ByeRequest_MessageHeaderRX. |
| Summary of change | Passed the correct dialog as input parameter to function f_IMS_ByeRequest_MessageHeaderRX to match expected BYE message as specified by test case at step 12. |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_TC_8_37_NR5GC_IMS1() runs on IMS_PTC

{ // Communication Waiting and answering the call / 5Gs

// sister test case of 15.27

var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog1;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp_Dialog2;

var MessageHeader v_MessageHeader183;

var template (present) MessageHeader v_MessageHeader_ExpectedBye_Dialog1;

var template (present) SipUrl v_ContactUrl_Dialog1;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall);

v_CallInfo_Dialog1 := f_IMS_MOCallSetup_NR5GC_A_4_1();

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 7" siclog@

f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog);

v_InviteRequestWithSdp_Dialog2 := f_IMS_MTCallSetup_NR5GC_A_5_1_Step1_7(-, -, -, secondDialog);

// @siclog "Step 8 - 10" siclog@

v_MessageHeader183 := f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp_Dialog2);

f_IMS_CommunicationWaiting_Check180Ringing(v_MessageHeader183);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 11" siclog@

f_UT_HangupIMSCall(MMI);

// @siclog "Step 12 - 13" siclog@

v_ContactUrl_Dialog1 := f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri);

v_MessageHeader_ExpectedBye_Dialog1 := f_IMS_ByeRequest_MessageHeaderRX(v_CallInfo_Dialog1.InviteRequestWithSdp.Invite);

a_IMS_ReceiveByeSendOK(firstDialog, v_ContactUrl_Dialog1, v_MessageHeader_ExpectedBye_Dialog1);

// @siclog "Step 14 - 16" siclog@

f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp_Dialog2);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15");

f_IMS_TestBody_Set(false);

f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_8_37_NR5GC_IMS1() runs on IMS_PTC

{ // Communication Waiting and answering the call / 5Gs

// sister test case of 15.27

var IMS_CC_MO_CallInfo_Type v_CallInfo_Dialog1;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp_Dialog2;

var MessageHeader v_MessageHeader183;

var template (present) MessageHeader v_MessageHeader_ExpectedBye_Dialog1;

var template (present) SipUrl v_ContactUrl_Dialog1;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall);

v_CallInfo_Dialog1 := f_IMS_MOCallSetup_NR5GC_A_4_1();

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 7" siclog@

f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog);

v_InviteRequestWithSdp_Dialog2 := f_IMS_MTCallSetup_NR5GC_A_5_1_Step1_7(-, -, -, secondDialog);

// @siclog "Step 8 - 10" siclog@

v_MessageHeader183 := f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp_Dialog2);

f_IMS_CommunicationWaiting_Check180Ringing(v_MessageHeader183);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 11" siclog@

f_UT_HangupIMSCall(MMI);

// @siclog "Step 12 - 13" siclog@

v_ContactUrl_Dialog1 := f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri);

v_MessageHeader_ExpectedBye_Dialog1 := f_IMS_ByeRequest_MessageHeaderRX(v_CallInfo_Dialog1.InviteRequestWithSdp.Invite, -, firstDialog);

a_IMS_ReceiveByeSendOK(firstDialog, v_ContactUrl_Dialog1, v_MessageHeader_ExpectedBye_Dialog1);

// @siclog "Step 14 - 16" siclog@

f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp_Dialog2);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15");

f_IMS_TestBody_Set(false);

f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall);

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n41 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Qualcomm SM8450, SDX65 UE.

# Execution Log Files

## Qualcomm

The Qualcomm SM 8450, SDX65 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

tc_8.37_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s211571: Supporting information for addition of IMS 5GS Supplementary Services test case 8.37 |
| --- | --- |