3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211578

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2169 | rev | - | Current version: | 17.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC 5GMM test case 9.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS38.523-1 Table 9.1.1.6.3.2-1 step 40, The SS configures: - NGC Cell D as the “Serving cell”. - NGC Cell C as a “Non-suitable "Off" cell “. - NGC Cell B as a "Non-suitable "Off" cell ". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate step45 for 9.1.1.3 and step40 for 9.1.1.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections requireds	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk37related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections requireds

## Change 1

| Function name | fl_TC_9_1_1_3_6_Body |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 9.1.1.6.3.2-1 step 40, The SS configures: - NGC Cell D as the “Serving cell”. - NGC Cell C as a “Non-suitable "Off" cell “. - NGC Cell B as a "Non-suitable "Off" cell ". |
| Summary of change | Separate step45 for 9.1.1.3 and step40 for 9.1.1.6 |
| TTCN module | Authentication_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function fl_TC_9_1_1_3_6_Body(boolean p_UseEAP ) runs on NR5GC_PTC

{ /*  test body of TC_9_1_1_3 and 9_1_1_6  The only difference is if including an EAP message or not */

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get();

var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf;

var SecurityHeaderType v_SecurityHeader := tsc_SHT_IntegrityProtected_Ciphered;

var integer i;

var NR_CellId_Type v_CellId := v_NGC_NASCellA; // @sic R5-199098 sic@

var default v_MyDefaultVar := null;

…

if(p_UseEAP){ // @sic R5-200616 sic@

//@siclog "Step 45" siclog@ //This is for Testcase 9_1_1_3

}else{

//@siclog "Step 40" siclog@ //This is for Testcase 9_1_1_6 // @sic R5-199098 sic@

}

f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE);

f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

// Void TP4 @sic R5-204457, R5-204458 sic@

if (p_UseEAP) {

//@siclog "Step 48 - 51" siclog@ //This is for Testcase 9_1_1_3

} else {

//@siclog "Step 43 - 46" siclog@ // This is for Testcase 9_1_1_6

}

…

}

After change

function fl_TC_9_1_1_3_6_Body(boolean p_UseEAP ) runs on NR5GC_PTC

{ /*  test body of TC_9_1_1_3 and 9_1_1_6  The only difference is if including an EAP message or not */

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get();

var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf;

var SecurityHeaderType v_SecurityHeader := tsc_SHT_IntegrityProtected_Ciphered;

var integer i;

var NR_CellId_Type v_CellId := v_NGC_NASCellA; // @sic R5-199098 sic@

var default v_MyDefaultVar := null;

…

if(p_UseEAP){ // @sic R5-200616 sic@

//@siclog "Step 45" siclog@ //This is for Testcase 9_1_1_3

f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE);

f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

}else{

//@siclog "Step 40" siclog@ //This is for Testcase 9_1_1_6 // @sic R5-199098 sic@

f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

}

// Void TP4 @sic R5-204457, R5-204458 sic@

if (p_UseEAP) {

//@siclog "Step 48 - 51" siclog@ //This is for Testcase 9_1_1_3

} else {

//@siclog "Step 43 - 46" siclog@ // This is for Testcase 9_1_1_6

}

…

}