3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211581

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2171 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR bands n38 and n40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From TS 38.508 table 4.3.1.1.1.38-2, the GSCN for band n38 for 10MHz bandwidth is 6439 for low range frequency. As per 38.101 table 5.4.3.3-1, the corresponding SSB SCS for this GSCN is 30kHz and the SSB block pattern is Case C. The TTCN is currently incorrectly setting this as Case A. Similarly in TS 38.508 table 4.3.1.1.1.40-2, the GSCN for band n40 for 10MHz bandwidth is 5764 for low range frequency. As per 38.101 table 5.4.3.3-1, the corresponding SSB SCS for this GSCN is 30kHz and the SSB block pattern is Case C. The TTCN is currently incorrectly setting this as Case A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the block pattern for bands n38 and n40 to Case C |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail test cases on the mentioned NR bands |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR_GetBlockPattern() |
| --- | --- |
| Reason for change | From TS 38.508 table 4.3.1.1.1.38-2, the GSCN for band n38 for 10MHz bandwidth is 6439 for low range frequency. As per 38.101 table 5.4.3.3-1, the corresponding SSB SCS for this GSCN is 30kHz and the SSB block pattern is Case C. The TTCN is currently incorrectly setting this as Case A. Similarly in TS 38.508 table 4.3.1.1.1.40-2, the GSCN for band n40 for 10MHz bandwidth is 5764 for low range frequency. As per 38.101 table 5.4.3.3-1, the corresponding SSB SCS for this GSCN is 30kHz and the SSB block pattern is Case C. The TTCN is currently incorrectly setting this as Case A. |
| Summary of change | Changed the block pattern for bands n38 and n40 to Case C. |
| TTCN module | NR_FrequencyInit_SA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetBlockPattern ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return NR_SS_BlockPattern_Type

{

var NR_SS_BlockPattern_Type v_BlockPattern_Type := caseA; //Acc to 38.101-1 Table 5.4.3.3-1: Applicable SS raster entries per operating band

select(p_FreqBandIndicatorNR) {

case( 5 ) { v_BlockPattern_Type := caseA ;}

case( 8 ) { v_BlockPattern_Type := caseA ;}

case( 12 ) { v_BlockPattern_Type := caseA ;}

case( 14 ) { v_BlockPattern_Type := caseA ;}

case( 20 ) { v_BlockPattern_Type := caseA ;}

case( 24 ) { v_BlockPattern_Type := caseA ;}

case( 26 ) { v_BlockPattern_Type := caseA ;}

case( 29 ) { v_BlockPattern_Type := caseA ;}

case( 30 ) { v_BlockPattern_Type := caseA ;}

case( 70 ) { v_BlockPattern_Type := caseA ;}

case( 71 ) { v_BlockPattern_Type := caseA ;}

case( 76 ) { v_BlockPattern_Type := caseA ;}

case( 1 ) { v_BlockPattern_Type := caseA ;}

case( 2 ) { v_BlockPattern_Type := caseA ;}

case( 3 ) { v_BlockPattern_Type := caseA ;}

case( 7 ) { v_BlockPattern_Type := caseA ;}

case( 25 ) { v_BlockPattern_Type := caseA ;}

case( 28 ) { v_BlockPattern_Type := caseA ;}

case( 65 ) { v_BlockPattern_Type := caseA ;}

case( 66 ) { v_BlockPattern_Type := caseA ;}

case( 74 ) { v_BlockPattern_Type := caseA ;}

case( 75 ) { v_BlockPattern_Type := caseA ;}

case( 34 ) { v_BlockPattern_Type := caseA ;}

case( 51 ) { v_BlockPattern_Type := caseA ;}

case( 53 ) { v_BlockPattern_Type := caseA ;}

case( 38 ) { v_BlockPattern_Type := caseA ;}

case( 39 ) { v_BlockPattern_Type := caseA ;}

case( 40 ) { v_BlockPattern_Type := caseA ;}

case( 48 ) { v_BlockPattern_Type := caseC ;}

case( 50 ) { v_BlockPattern_Type := caseA ;}

case( 41 ) { v_BlockPattern_Type := caseC ;}

case( 77 ) { v_BlockPattern_Type := caseC ;}

case( 78 ) { v_BlockPattern_Type := caseC ;}

case( 79 ) { v_BlockPattern_Type := caseC ;}

case( 257 ) { v_BlockPattern_Type := caseD ;}

case( 258 ) { v_BlockPattern_Type := caseD ;}

case( 259 ) { v_BlockPattern_Type := caseD ;}

case( 260 ) { v_BlockPattern_Type := caseD ;}

case( 261 ) { v_BlockPattern_Type := caseD ;}

}

return v_BlockPattern_Type;

}

After Change:

function f_NR_GetBlockPattern ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return NR_SS_BlockPattern_Type

{

var NR_SS_BlockPattern_Type v_BlockPattern_Type := caseA; //Acc to 38.101-1 Table 5.4.3.3-1: Applicable SS raster entries per operating band

select(p_FreqBandIndicatorNR) {

case( 5 ) { v_BlockPattern_Type := caseA ;}

case( 8 ) { v_BlockPattern_Type := caseA ;}

case( 12 ) { v_BlockPattern_Type := caseA ;}

case( 14 ) { v_BlockPattern_Type := caseA ;}

case( 20 ) { v_BlockPattern_Type := caseA ;}

case( 24 ) { v_BlockPattern_Type := caseA ;}

case( 26 ) { v_BlockPattern_Type := caseA ;}

case( 29 ) { v_BlockPattern_Type := caseA ;}

case( 30 ) { v_BlockPattern_Type := caseA ;}

case( 70 ) { v_BlockPattern_Type := caseA ;}

case( 71 ) { v_BlockPattern_Type := caseA ;}

case( 76 ) { v_BlockPattern_Type := caseA ;}

case( 1 ) { v_BlockPattern_Type := caseA ;}

case( 2 ) { v_BlockPattern_Type := caseA ;}

case( 3 ) { v_BlockPattern_Type := caseA ;}

case( 7 ) { v_BlockPattern_Type := caseA ;}

case( 25 ) { v_BlockPattern_Type := caseA ;}

case( 28 ) { v_BlockPattern_Type := caseA ;}

case( 65 ) { v_BlockPattern_Type := caseA ;}

case( 66 ) { v_BlockPattern_Type := caseA ;}

case( 74 ) { v_BlockPattern_Type := caseA ;}

case( 75 ) { v_BlockPattern_Type := caseA ;}

case( 34 ) { v_BlockPattern_Type := caseA ;}

case( 51 ) { v_BlockPattern_Type := caseA ;}

case( 53 ) { v_BlockPattern_Type := caseA ;}

case( 38 ) { v_BlockPattern_Type := caseC ;}

case( 39 ) { v_BlockPattern_Type := caseA ;}

case( 40 ) { v_BlockPattern_Type := caseC ;}

case( 48 ) { v_BlockPattern_Type := caseC ;}

case( 50 ) { v_BlockPattern_Type := caseA ;}

case( 41 ) { v_BlockPattern_Type := caseC ;}

case( 77 ) { v_BlockPattern_Type := caseC ;}

case( 78 ) { v_BlockPattern_Type := caseC ;}

case( 79 ) { v_BlockPattern_Type := caseC ;}

case( 257 ) { v_BlockPattern_Type := caseD ;}

case( 258 ) { v_BlockPattern_Type := caseD ;}

case( 259 ) { v_BlockPattern_Type := caseD ;}

case( 260 ) { v_BlockPattern_Type := caseD ;}

case( 261 ) { v_BlockPattern_Type := caseD ;}

}

return v_BlockPattern_Type;

}