3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211636

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2022 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.3.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | On test step 1-2 Scell has been added, so MACbearerRouting should be added for further RRC msgs at step 4,5 and 21, Else TC will fail due to mismatch of MACBearerRouting. To disable MACBearerRouting on postamble, need to release SCell. Else TC will fail becuae of Mismatch of MACBearerRouting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MACBearerRouting configuration on step 4,5 and 21. Added Scell release function on postamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will fail conformant and non-conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_8_NR_Common () |
| --- | --- |
| Reason for change | On test step 1-2 Scell has been added, so MACbearerRouting should be added for further RRC msgs at step 4,5 and 21, Else TC will fail due to mismatch of MACBearerRouting. To disable MACBearerRouting on postamble, need to release SCell. Else TC will fail becuae of Mismatch of MACBearerRouting. |
| Summary of change | Added MACBearerRouting configuration on step 4,5 and 21. Added Scell release function on postamble. |
| TTCN module | MAC_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTMode_OFF);//@sic R5s211385 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(p_PCell, -,//@sic R5s211385 sic@ -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true); // NR Cell1, NR Cell3/10 v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 4 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //@siclog "Steps 3" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10); //@siclog "Steps 4" siclog@ f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_MAC_PHR(enable)); // Added MACBearerRouting configuration on below funtion f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,false); //@siclog "Steps 5" siclog@ alt { // Added MACBearerRouting configuration on below template [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } [not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour"); v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } }; //@siclog "Steps 6-20" siclog@ f_TC_7_1_1_3_8_NR_TestBody(p_PCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR(?),tsc_SCellIndex_1); //@siclog "Steps 21-22" siclog@ v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, //@sic R5s200094 sic@ omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); // Added MACBearerRouting configuration on below function f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(p_PCell); // Def UL Grant Config //Anritsu TTCN CR R5s210953 // Added Scell release here f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_3_8_NR_Common

After Change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTMode_OFF);//@sic R5s211385 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(p_PCell, -,//@sic R5s211385 sic@ -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true); // NR Cell1, NR Cell3/10 v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 4 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //@siclog "Steps 3" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10); //@siclog "Steps 4" siclog@ f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_MAC_PHR(enable)); f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,false,-,crs_MacBearerRouting(p_PCell)); //@siclog "Steps 5" siclog@ alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete,-,crs_MacBearerRouting(p_PCell))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } [not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour"); v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } }; //@siclog "Steps 6-20" siclog@ f_TC_7_1_1_3_8_NR_TestBody(p_PCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR(?),tsc_SCellIndex_1); //@siclog "Steps 21-22" siclog@ v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, //@sic R5s200094 sic@ omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,-,-,crs_MacBearerRouting(p_PCell)); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(p_PCell); // Def UL Grant Config //Anritsu TTCN CR R5s210953 f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_3_8_NR_Common