3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211650

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2199 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR2 : Addition of NR5GC test case 8.1.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC test case 8.1.5.2.2 in FR2 to the IWD_21wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.5.2.2 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	HUAWEI Balong 5000	3

5	References	3

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.2.2 in FR2 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry

shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.5.2.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HUAWEI Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_8_1_5_2_2_TestBody |
| --- | --- |
| Reason for change | Currently, numerology mu=3 is used for FR2, so the expect PRACH timing info for step 6 is incorrect |
| Summary of change | Correct the expected timing info based on mu=3 |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var template (present) TimingInfo_Type v_TimingInfo; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //Get the QosFlow ID information from the DRB ID var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); //@sic R5s201136 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_Msg3TBS(v_NR_PhysicalParameters, 640); //@sic R5s201136 R5-206337 sic@ //Set SIB1 content as Table 8.1.5.2.2.3.3-2: SIB1 if ( not v_IsFR1 ) { //FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 4 v_TimingInfo := ( cr_TimingInfo(?, (0), -, cs_SlotTimingInfo_Numerology2(3 mod 4)), //Slot 4 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology2(0 mod 4)), //Slot 9 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology2(1 mod 4)), //Slot 14 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology2(2 mod 4)), //Slot 19 cr_TimingInfo(?, (5), -, cs_SlotTimingInfo_Numerology2(3 mod 4)), //Slot 24 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology2(0 mod 4)), //Slot 29 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology2(1 mod 4)), //Slot 34 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology2(2 mod 4)) //Slot 39 ); } else if (v_IsFR1 and ischosen(v_NR_DuplexMode.TDD)) { //FR1 TDD v_PrachConfigurationIndex := 94; //FR1 TDD - As per 38.211 Table 6.3.3.2-3 v_TimingInfo := (cr_TimingInfo(?, (4)), cr_TimingInfo(?, (9))); } else { //FR1 FDD v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1 v_TimingInfo := (cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9))); } …………….. }

After Change

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var template (present) TimingInfo_Type v_TimingInfo; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //Get the QosFlow ID information from the DRB ID var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); //@sic R5s201136 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_Msg3TBS(v_NR_PhysicalParameters, 640); //@sic R5s201136 R5-206337 sic@ //Set SIB1 content as Table 8.1.5.2.2.3.3-2: SIB1 if ( not v_IsFR1 ) { //FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) in 60kHz - As per 38.211 Table 6.3.3.2-4 4 // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } else if (v_IsFR1 and ischosen(v_NR_DuplexMode.TDD)) { //FR1 TDD v_PrachConfigurationIndex := 94; //FR1 TDD - As per 38.211 Table 6.3.3.2-3 v_TimingInfo := (cr_TimingInfo(?, (4)), cr_TimingInfo(?, (9))); } else { //FR1 FDD v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1 v_TimingInfo := (cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9))); } …… }

# Branches executed

## This test case was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n258 using the Ciphering Algorithm nea2 and Integrity Algorithm nia2.

# Execution Log Files

## HUAWEI Balong 5000

The HUAWEI Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset platform. The documentation below is enclosed as evidence of the successful test case run [1]:

•	Test case execution log file:

TC_8_1_5_2_2_LOG.html

•	PICS/PIXIT parameter file:

TC_8_1_5_2_2_PIXIT.html

# References

| [1] | R5s211651: NR5GC FR2: Supporting information for addition of NR5GC test case 8.1.5.2.2. |
| --- | --- |