3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211654

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2201 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcases 7.1.1.7.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - R5s211302 proposes to use 60ms at Steps 8 and 17. However, it has been observed during actual testing that with 30ms the reliability of sending the RLC STATUS PDU improves and further at Step 17, the RLC STATUS PDU needs to be sent on the correct cell acknowledging reception of RLC PDU in step 16 - According to the Prose, at Step 9, the SS should indicate a new transmission 400ms after Step 4 so that the sCellDeactivationTimer = 320 ms expires at the UE. In TTCN, this is being coded as 300ms after Step 4. - MacBearerRouting should be enabled to ensure the DL Data and RLC STATUS PDU is sent on the correct cell. - At Step 6 and Step 15, the transmission of the UL grant is being done onSR. The coding in TTCN is being done as cs_UL_GrantConfig_OneTimeOnSR, however, the manner in which the timing for the UL grant is calculated based on the timing at Step 4 with no added time (0ms) at Step 6 and Step 15 gives rise to the possibility of scheduling the UL grant before the SR is received and when the UE does send an SR, there is no transmission of the UL grant. Hence, it is recommended to change the UL grant configuration to cs_UL_GrantConfig_OnSR - At Steps 18 - 20 of the Prose, the procedure is done to de-activate the SCell and perform a negative test to ensure that UE does not receive the new transmission on the NR SCell. However, this gives rise to the possibility that during the procedure the release the SCell in function f_NR5GC_Rel_1SCell, the peer signalling message RRCReconfiguration may get sent on the NR SCell, since CA is still active and the SS implementation can choose which cell to send the signalling message on in the absence of a MacBearerRouting parameter. On the UE side, it has de-activated the SCell and would not be able to receive the signalling message. Hence, it is important to re-activate the SCell after the TP4 has been verified at Step 20 - The configuration for the starting of the default UL grant configuration should be done before the function f_NR5GC_Rel_1SCell |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reduced the time to 30ms to transmit the RLC STATUS PDU at Steps 8 and 17 and at Step 17, parameter p_UL_CA is tested to route the RLC STATUS PDU on the correct cell - TTCN corrected to use 400ms at Step 9 - Added MacBearerRouting at the appropriate places to ensure DL Data and RLC STATUS PDU is sent on the correct cell - Used cs_UL_GrantConfig_OnSR at Step 6 and Step 15 - Re-activated SCell with immediate activation time and allowed a delay of 100ms to ensure MAC CE procedure is completed before function f_NR5GC_Rel_1SCell is called. - Moved function call f_NR_ULGrantConfiguration_Start before f_NR5GC_Rel_1SCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.x.ENDC, 7.1.1.7.1.x.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required for MAC testcases 7.1.1.7.1.x	3

1.1	Correction to f_TC_7_1_1_7_1_NR_Common	3

1.2	Correction to f_TC_7_1_1_7_1_NR_TestBody	5

1.3	Correction to f_NR_OneULGrantTransmissionOnSR	13

1.4	Correction to f_NR_RLC_Status_MAC_PDU_Tx	14

1.5	Correction to car_NR_DRB_COMMON_IND	16

1.6	Correction to car_NR_DRB_COMMON_IND_MAC_PDUList	17

# Corrections required for MAC testcases 7.1.1.7.1.x

## Correction to f_TC_7_1_1_7_1_NR_Common

| Function name | f_TC_7_1_1_7_1_NR_Common |
| --- | --- |
| Reason for change | The configuration for the starting of the default UL grant configuration should be done before the function f_NR5GC_Rel_1SCell |
| Summary of change | Moved function call f_NR_ULGrantConfiguration_Start before f_NR5GC_Rel_1SCell. |
| TTCN module | MAC_CA_NR5GC_NRDC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_7_1_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / <<SKIPPED CODE>> //Postamble //@sic R5s210727 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)};//KS R5s21xxx f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList);//KS R5s21xxx f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_ULGrantConfiguration_Start(p_PCell); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(p_PCell); f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_7_1_NR_Common

After Change:

function f_TC_7_1_1_7_1_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / <<SKIPPED CODE>> //Postamble //@sic R5s210727 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)};//KS R5s21xxx f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList);//KS R5s21xxx //MOVED before SCell release f_NR_ULGrantConfiguration_Start(p_PCell); // Def UL Grant Config f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(p_PCell); f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_7_1_NR_Common

## Correction to f_TC_7_1_1_7_1_NR_TestBody

| Function name | f_TC_7_1_1_7_1_NR_TestBody |
| --- | --- |
| Reason for change | - R5s211302 proposes to use 60ms at Steps 8 and 17. However, it has been observed during actual testing that with 30ms the reliability of sending the RLC STATUS PDU improves and further at Step 17, the RLC STATUS PDU needs to be sent on the correct cell acknowledging reception of RLC PDU in step 16 - According to the Prose, at Step 9, the SS should indicate a new transmission 400ms after Step 4 so that the sCellDeactivationTimer = 320 ms expires at the UE. In TTCN, this is being coded as 300ms after Step 4. - MacBearerRouting should be enabled to ensure the DL Data and RLC STATUS PDU is sent on the correct cell. - At Step 6 and Step 15, the transmission of the UL grant is being done onSR. The coding in TTCN is being done as cs_UL_GrantConfig_OneTimeOnSR, however, the manner in which the timing for the UL grant is calculated based on the timing at Step 4 with no added time (0ms) at Step 6 and Step 15 gives rise to the possibility of scheduling the UL grant before the SR is received and when the UE does send an SR, there is no transmission of the UL grant. Hence, it is recommended to change the UL grant configuration to cs_UL_GrantConfig_OnSR - At Steps 18 - 20 of the Prose, the procedure is done to de-activate the SCell and perform a negative test to ensure that UE does not receive the new transmission on the NR SCell. However, this gives rise to the possibility that during the procedure the release the SCell in function f_NR5GC_Rel_1SCell, the peer signalling message RRCReconfiguration may get sent on the NR SCell, since CA is still active and the SS implementation can choose which cell to send the signalling message on in the absence of a MacBearerRouting parameter, on the UE side, it has de-activated the SCell and would not be able to receive the signalling message. Hence, it is important to re-activate the SCell after the TP4 has been verified at Step 20 |
| Summary of change | - Reduced the time to 30ms to transmit the RLC STATUS PDU at Steps 8 and 17 and at Step 17, parameter p_UL_CA is tested to route the RLC STATUS PDU on the correct cell - TTCN corrected to use 400ms at Step 9 - Added MacBearerRouting at the appropriate places to ensure DL Data and RLC STATUS PDU is sent on the correct cell - Used cs_UL_GrantConfig_OnSR at Step 6 and Step 15 - Re-activated SCell with immediate activation time and allowed a delay of 100ms to ensure MAC CE procedure is completed before function f_NR5GC_Rel_1SCell is called. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ <<SKIPPED CODE>> t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . //@sic R5s210727 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SPCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1)); //@siclog "Steps 6" siclog@ //@sic R5s210727 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SPCell, v_TimingDL, 0); // 0 ms later f_NR_OneULGrantTransmissionOnSR(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 140); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 300); // DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ //@sic R5s210727 sic@ alt { [] a_NR_RacingCond_AwaitSysIndAndDrbData(p_SPCell, SysIndBeforeRrcMsg, v_DrbIndDummy, v_SysIndDummy, car_NR_SchedulingReq_IND (p_SPCell, ?), car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL1) //@sic R5s210893 sic@ ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5, 7"); } }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SPCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); //DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4)); //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2)); //@siclog "Steps 15" siclog@ //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 0 ms later f_NR_OneULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 140); //assumed the f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) ); //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); //DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); //@siclog "Steps 14, 16" siclog@ //@sic R5s210668 R5s210727 sic@ alt { [] a_NR_RacingCond_AwaitSysIndAndDrbData(p_SPCell, SysIndBeforeRrcMsg, v_DrbIndDummy, v_SysIndDummy, car_NR_SchedulingReq_IND (p_SPCell, ?), car_NR_DRB_COMMON_IND_MAC_PDUList(v_CellID_UL, p_NR_DRB_Id, -, v_MAC_PDUListUL2)//@sic R5s210893 sic@ ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14, 16"); } }; //@siclog "Step 20" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP4 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); } }; //@sic R5s210066 sic@ //disable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable)); }; //End of f_TC_7_1_1_7_1_NR_TestBody

After Change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ <<SKIPPED CODE>> t_Watchdog.start ; //@siclog "Steps 3" siclog@ //The SS Activates SCell . //@sic R5s210727 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SPCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1,-,-,crs_MacBearerRouting(p_SCell))); //WA#7_1_1_7_1_x: MacBearerRouting //@siclog "Steps 6" siclog@ //@sic R5s210727 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SPCell, v_TimingDL, 0); // 0 ms later f_NR_OneULGrantTransmissionOnSR(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Steps 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 30); //WA#7_1_1_7_1_x f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SPCell)); //WA#7_1_1_7_1_1: RLC Status PDU needs to be sent on SPCell //@siclog "Steps 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400); //WA#7_1_1_7_1_x DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2,-,-,crs_MacBearerRouting(p_SCell))); //WA#7_1_1_7_1_x: MacBearerRouting //@sic R5s210066 sic@ //enable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Steps 5, 7" siclog@ //@sic R5s210727 sic@ alt { [] a_NR_RacingCond_AwaitSysIndAndDrbData(p_SPCell, SysIndBeforeRrcMsg, v_DrbIndDummy, v_SysIndDummy, car_NR_SchedulingReq_IND (p_SPCell, ?), car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL1, crs_MacBearerRouting(p_SPCell)) //@sic R5s210893 sic@ WA#7_1_1_7_1_x: Added MacBearerRouting ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5, 7"); } }; //@siclog "Step 10" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } }; t_Watchdog.start; //@siclog "Steps 11" siclog@ //The SS Activates SCell . v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SPCell, 300); // Timing of DL f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL)); //@siclog "Steps 12" siclog@ //Padding MAC PDU Only v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4,-,-,crs_MacBearerRouting(p_SCell))); //WA#7_1_1_7_1_x: MacBearerRouting //@siclog "Steps 13" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); // 200+200 =400 DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2,-,-, crs_MacBearerRouting(p_SCell))); //WA#7_1_1_7_1_x: MacBearerRouting //@siclog "Steps 15" siclog@ //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 0 ms later f_NR_OneULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@siclog "Steps 17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 30); //WA#7_1_1_7_1_x if (p_UL_CA){ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SCell)); //WA#7_1_1_7_1_x: RLC Status PDU needs to be sent on SCell } else { f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SPCell,cs_TimingInfo_NR(v_TimingDL),crs_MacBearerRouting(p_SPCell)); //WA#7_1_1_7_1_x: RLC Status PDU needs to be sent on SPCell } //@siclog "Step 18" siclog@ //Deactivate SCell v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell))); //@siclog "Step 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200); //DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3)); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SPCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3,-,-,crs_MacBearerRouting(p_SCell))); //WA#7_1_1_7_1_x: MacBearerRouting //@siclog "Steps 14, 16" siclog@ //@sic R5s210668 R5s210727 sic@ v_DrbIndDummy := omit; v_SysIndDummy := omit; alt { [] a_NR_RacingCond_AwaitSysIndAndDrbData(p_SPCell, SysIndBeforeRrcMsg, v_DrbIndDummy, v_SysIndDummy, car_NR_SchedulingReq_IND (p_SPCell, ?), car_NR_DRB_COMMON_IND_MAC_PDUList(v_CellID_UL, p_NR_DRB_Id, -, v_MAC_PDUListUL2,crs_MacBearerRouting(v_CellID_UL))//@sic R5s210893 sic@ WA#7_1_1_7_1_x: MacBearerRouting ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14, 16"); } }; //@siclog "Step 20" siclog@ t_Watchdog.start (1.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP4 failed, UE is sending SR "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); } }; //@sic R5s210066 sic@ //disable reception on sp_Cell f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable)); //The SS Re-Activates the SCell. WA#7_1_1_7_1_x f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex); f_Delay(0.1); }; //End of f_TC_7_1_1_7_1_NR_TestBody

## Correction to f_NR_OneULGrantTransmissionOnSR

| Function name | f_NR_OneULGrantTransmissionOnSR |
| --- | --- |
| Reason for change | At Step 6 and Step 15, the transmission of the UL grant is being done onSR. The coding in TTCN is being done as cs_UL_GrantConfig_OneTimeOnSR, however, the manner in which the timing for the UL grant is calculated based on the timing at Step 4 with 0ms delay gives rise to the possibility of scheduling the UL grant before the SR is received and when the UE does send an SR, there is no transmission of the UL grant. Hence, it is recommended to change the UL grant configuration to cs_UL_GrantConfig_OnSR |
| Summary of change | Used cs_UL_GrantConfig_OnSR at Step 6 and Step 15 |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment |  |

Before Change:

function f_NR_OneULGrantTransmissionOnSR(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ResourceAllocation_Type p_ResourceAllocation := cs_NR_ResourceAllocationDef, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig := omit, //@sic R5s200711 sic@ template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Start ) runs on NR_BASE_PTC { f_NR_ULGrantConfiguration_Start(p_NR_CellId, p_TimingInfo, p_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, p_ResourceAllocation.Lrbs), p_PUCCH_Synch, //@sic R5s200711 sic@ cs_UL_GrantConfig_OneTimeOnSR, p_NR_HarqProcessConfig); }

After Change:

function f_NR_OneULGrantTransmissionOnSR(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ResourceAllocation_Type p_ResourceAllocation := cs_NR_ResourceAllocationDef, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig := omit, //@sic R5s200711 sic@ template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Start ) runs on NR_BASE_PTC { f_NR_ULGrantConfiguration_Start(p_NR_CellId, p_TimingInfo, p_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, p_ResourceAllocation.Lrbs), p_PUCCH_Synch, //@sic R5s200711 sic@ cs_UL_GrantConfig_OnSR, //WA#7_1_1_7_1_x p_NR_HarqProcessConfig); }

## Correction to f_NR_RLC_Status_MAC_PDU_Tx

| Function name | f_NR_RLC_Status_MAC_PDU_Tx |
| --- | --- |
| Reason for change | MacBearerRouting should be enabled to ensure the DL Data and RLC STATUS PDU is sent on the correct cell. |
| Summary of change | Added MacBearerRouting at the appropriate places to ensure DL Data and RLC STATUS PDU is sent on the correct cell |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_NR_RLC_Status_MAC_PDU_Tx(B6_Type p_LchId, integer p_SN, DRB_Identity p_NR_DRB_Id, NR_CellId_Type p_NR_CellId := nr_Cell1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR_BASE_PTC { var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LchId, f_NR_RLC_AMD_StatusPDU_Encvalue(p_SN)) }; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Size made 40 bits 3 Byte RLC Status PDU and 2 Byte MAC Header DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_NR_CellId, p_NR_DRB_Id, p_TimingInfo, v_MAC_PDUListDL));

After Change:

function f_NR_RLC_Status_MAC_PDU_Tx(B6_Type p_LchId, integer p_SN, DRB_Identity p_NR_DRB_Id, NR_CellId_Type p_NR_CellId := nr_Cell1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit /*WA#WI=880634 7_1_1_7_1_x: MacBearerRouting*/) runs on NR_BASE_PTC { var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(p_LchId, f_NR_RLC_AMD_StatusPDU_Encvalue(p_SN)) }; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Size made 40 bits 3 Byte RLC Status PDU and 2 Byte MAC Header DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_NR_CellId, p_NR_DRB_Id, p_TimingInfo, v_MAC_PDUListDL,-,-,p_MacBearerRoutingCA)); /*WA#WI=880634 7_1_1_7_1_x: MacBearerRouting*/ }

## Correction to car_NR_DRB_COMMON_IND

| Function name | car_NR_DRB_COMMON_IND |
| --- | --- |
| Reason for change | MacBearerRouting should be enabled to ensure the DL Data and RLC STATUS PDU is sent on the correct cell. |
| Summary of change | Added MacBearerRouting at the appropriate places to ensure DL Data and RLC STATUS PDU is sent on the correct cell |
| TTCN module | NR_ASP_Templates_DRB_Data |
| MCC160 Comment |  |

Before Change:

template (present) NR_DRB_COMMON_IND car_NR_DRB_COMMON_IND(NR_CellId_Type p_NR_CellId, template (present) DRB_Identity p_DrbId, template (present) TimingInfo_Type p_TimingInfo, template (present) NR_L2DataList_Type p_L2DataList ) := /* @status APPROVED (ENDC, NR5GC) */ car_NR_DRB_COMMON_IND_Common(p_NR_CellId, cr_NR_RoutingInfo_RbId(cr_NR_RadioBearerId_DRB(p_DrbId)), cr_RlcBearerRouting_NR(p_NR_CellId), p_TimingInfo, p_L2DataList);

After Change:

template (present) NR_DRB_COMMON_IND car_NR_DRB_COMMON_IND(NR_CellId_Type p_NR_CellId, template (present) DRB_Identity p_DrbId, template (present) TimingInfo_Type p_TimingInfo, template (present) NR_L2DataList_Type p_L2DataList, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit) := //WA#WI=880634 7_1_1_7_1_x: Added MacBearerRouting /* @status APPROVED (ENDC, NR5GC) */ car_NR_DRB_COMMON_IND_Common(p_NR_CellId, cr_NR_RoutingInfo_RbId(cr_NR_RadioBearerId_DRB(p_DrbId)), cr_RlcBearerRouting_NR(p_NR_CellId), p_TimingInfo, p_L2DataList, p_MacBearerRoutingCA); //WA#7_1_1_7_1_x: Added MacBearerRouting

## Correction to car_NR_DRB_COMMON_IND_MAC_PDUList

| Function name | car_NR_DRB_COMMON_IND_MAC_PDUList |
| --- | --- |
| Reason for change | MacBearerRouting should be enabled to ensure the DL Data and RLC STATUS PDU is sent on the correct cell. |
| Summary of change | Added MacBearerRouting at the appropriate places to ensure DL Data and RLC STATUS PDU is sent on the correct cell |
| TTCN module | NR_ASP_Templates_DRB_Data |
| MCC160 Comment |  |

Before Change:

template (present) NR_DRB_COMMON_IND car_NR_DRB_COMMON_IND_MAC_PDUList(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (present) TimingInfo_Type p_TimingInfo := cr_TimingInfo_Any, template (present) NR_MAC_PDUList_Type p_MAC_PDUList := ? ) := /* @status APPROVED (ENDC, NR5GC) */ car_NR_DRB_COMMON_IND(p_NR_CellId, p_DrbId, p_TimingInfo, cr_NR_L2DataList_MacPdu(p_MAC_PDUList));

After Change:

template (present) NR_DRB_COMMON_IND car_NR_DRB_COMMON_IND_MAC_PDUList(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (present) TimingInfo_Type p_TimingInfo := cr_TimingInfo_Any, template (present) NR_MAC_PDUList_Type p_MAC_PDUList := ?, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit) := //WA#WI=880634 7_1_1_7_1_x: Added MacBearerRouting /* @status APPROVED (ENDC, NR5GC) */ car_NR_DRB_COMMON_IND(p_NR_CellId, p_DrbId, p_TimingInfo, cr_NR_L2DataList_MacPdu(p_MAC_PDUList), p_MacBearerRoutingCA); //WA#WI=880634 7_1_1_7_1_x: Added MacBearerRouting