3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211668

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2208 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_ReconfigureDRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In f_ReconfigureDRBs when the “if condition (isvalue(p_NrofHARQ_ProcessesForPDSCH))” matches then p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig is assigned a value. This results in the runtime error as spCellConfig IEs and spCellConfigDedicated IEs were not initialised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | spCellConfig IEs and spCellConfigDedicated IEs are initialised |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execuction will result in run time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Other TCs where f_ReconfigureDRBs is called with harq information filled |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_ReconfigureDRBs	2

# Corrections required

## Correction to function f_ReconfigureDRBs

| Function name | f_ReconfigureDRBs |
| --- | --- |
| Reason for change | In f_ReconfigureDRBs when the if condition (isvalue(p_NrofHARQ_ProcessesForPDSCH)) matches then p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig is assigned a value. This results in the runtime error as spCellConfig IEs and spCellConfigDedicated IEs were not initialised. |
| Summary of change | spCellConfig IEs and spCellConfigDedicated IEs are initialised |
| TTCN module | \Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_CellGroupConfig, template (omit) NR_RadioBearerList_Type p_SS_Config, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ReceiveForRRCreconfigurationComplete := true, template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit, template (omit) NR_SecurityParams_Type p_SecurityParams := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ // @sic R5-213516: added p_MacBearerRoutingCA sic@ var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, p_TimingInfo, cs_NR_PDSCH_CellLevelConfigDef(p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(p_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, p_SS_Config, p_TimingInfo); // Configure PDCP security if (isvalue(p_SecurityParams)){ // @sic R5s210277 sic@ v_SecurityParams := valueof(p_SecurityParams); } else { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false); // @sic R5s190382, R5s210277 sic@ } f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // Send message to UE f_NR_SendRRCReconfiguration (p_NR_CellId, p_SRB, p_RadioBearerConfig, v_V1530Ext, p_NASMsg, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL, p_MacBearerRoutingCA); }

After Change:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_CellGroupConfig, template (omit) NR_RadioBearerList_Type p_SS_Config, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ReceiveForRRCreconfigurationComplete := true, template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit, template (omit) NR_SecurityParams_Type p_SecurityParams := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ // @sic R5-213516: added p_MacBearerRoutingCA sic@ var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { //(SA 7.1.1.2.1) spCellConfig IEs and spCellConfigDedicated IEs were not initialised and results in runtime error p_CellGroupConfig.spCellConfig.servCellIndex :=omit; p_CellGroupConfig.spCellConfig.reconfigurationWithSync :=omit; p_CellGroupConfig.spCellConfig.rlf_TimersAndConstants :=omit; p_CellGroupConfig.spCellConfig.rlmInSyncOutOfSyncThreshold :=omit; p_CellGroupConfig.spCellConfig.spCellConfigDedicated := cs_NR_ServingCellConfigDef; p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, p_TimingInfo, cs_NR_PDSCH_CellLevelConfigDef(p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(p_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, p_SS_Config, p_TimingInfo); // Configure PDCP security if (isvalue(p_SecurityParams)){ // @sic R5s210277 sic@ v_SecurityParams := valueof(p_SecurityParams); } else { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false); // @sic R5s190382, R5s210277 sic@ } f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // Send message to UE f_NR_SendRRCReconfiguration (p_NR_CellId, p_SRB, p_RadioBearerConfig, v_V1530Ext, p_NASMsg, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL, p_MacBearerRoutingCA); }