3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211680

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2215 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of RRC test case 8.1.1.3.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.1.3.7a to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.1.3.7a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.7a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3.	Branches executed	7

4.	Execution Log Files	8

4.1	Qualcomm SM8450+SDX65	8

5.	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.3.7a which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.1.3.7a.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8450+SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | - Original TTCN implementation keeps waiting for another TAU Update to happen while t_WaitTime is running. This makes the execution to get stuck inside the function f_EUTRA38_TrackingAreaUpdateFromN1(). - Original TTCN did not handle timeout indication of t_WaitTime causing TTCN error when it happens - f_EUTRA38_NR5GC_S1toN1_IdleMode() by default expects native security when UE moves from E-UTRA to NRCell3. |
| Summary of change | - removed “while()” loop as only one TAU procedure is expected. - Added handling of t_WaitTime expiry before sending trigger to NR - Set the 2nd input parameter of f_EUTRA38_NR5GC_S1toN1_IdleMode() to “true” – i.e. expect Mapped Security |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var integer v_EUTRA_CellPower_T1 := -75; var octetstring v_T325_InMillisec; var boolean v_FlagEutra := false; var float v_T325; timer t_WaitTime; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, v_EUTRA_CellPower_T1, v_EUTRA_CellPower_T1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 3" siclog@ //SS starts timer T325. v_T325_InMillisec := f_EUTRA_NR_WaitForCoOrd_Data(NR, "T325"); v_T325 := int2float(oct2int(v_T325_InMillisec)/1000); t_WaitTime.start(v_T325); //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? while(t_WaitTime.running){ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_FlagEutra := true; } if(v_FlagEutra == false){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE did not finish generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 on E-UTRAN Cell 1"); } //@siclog "Step 5" siclog@ //SS waits for T325 expiry. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Timeout")); //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 3? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); }

After Change:

function fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var integer v_EUTRA_CellPower_T1 := -75; var octetstring v_T325_InMillisec; var boolean v_FlagEutra := false; var float v_T325; timer t_WaitTime; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, v_EUTRA_CellPower_T1, v_EUTRA_CellPower_T1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 3" siclog@ //SS starts timer T325. v_T325_InMillisec := f_EUTRA_NR_WaitForCoOrd_Data(NR, "T325"); v_T325 := int2float(oct2int(v_T325_InMillisec)/1000); t_WaitTime.start(v_T325); //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? //while(t_WaitTime.running){ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, omit); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //v_FlagEutra := true; //} //if(v_FlagEutra == false){ // f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE did not finish generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 on E-UTRAN Cell 1"); //} //@siclog "Step 5" siclog@ //SS waits for T325 expiry. []t_WaitTime.timeout{ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Timeout")); } //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 3? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); }

## Change 2

| Function name | f_TC_8_1_1_3_7a_NR5GC() |
| --- | --- |
| Reason for change | - The MaxReferencePower for cell1 is set to -88 but the power is set to -85 during the test case - UE will be on NR cell3 at postamble |
| Summary of change | - Set initialMaxReferencePower of nr_Cell1 to -85. - Use NR cell 3 for postamble |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_1_3_7a_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / Deprioritisation / NR / T325 expiry f_NR5GC_IRAT_Init(NR_7); //Update SIB1 according to specific message contents f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(3,24)); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower (nr_Cell3, -79, -79); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_7a_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

function f_TC_8_1_1_3_7a_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / Deprioritisation / NR / T325 expiry f_NR5GC_IRAT_Init(NR_7); //Update SIB1 according to specific message contents f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(3,24)); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower (nr_Cell1, -85, -85); f_NR_CellInfo_InitMaxReferencePower (nr_Cell3, -79, -79); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_7a_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 3

| Function name | fl_TC_8_1_1_3_7a_TestBody() |
| --- | --- |
| Reason for change | - There is a possibility that a UE would reselect to the EUTRA Cell faster than the function f_NR_RRCReleaseCommon() could finish execution. This causes TTCN Error due to unexpected RRC message received. - The function f_NR5GC_IRAT_S1ToN1_IdleMode() by default uses the current nr_Cell’s TAC when checking lastVisitedRegistered TAI in Registration Request message. Since nr_Cell1 and nr_Cell3 TACs are different, this causes a TTCN error. |
| Summary of change | - Send trigger to EUTRA PTC before calling f_NR_RRCReleaseCommon(), so cell access on EUTAR can be immediately handled. - Use TAC of nr_Cell1 as input to function f_NR5GC_IRAT_S1ToN1_IdleMode(). No need for new input parameter since one already exists. |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_3_7a_TestBody() runs on NR5GC_PTC { var template (value) RRCRelease_IEs.deprioritisationReq v_DeprioritisationReq; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var float v_T325 := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 300.0); var integer v_T325_InMillisec := float2int(v_T325 * 1000.0); v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, -85, -85), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell3, -79, -79) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message on NR Cell 1 including IE deprioritisationReq. v_DeprioritisationReq := cs_NR_DeprioritisationReq(nr, min5); //@siclog "Step 3" siclog@ //SS starts timer T325. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(int2oct(v_T325_InMillisec, 4), "T325")); //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Timeout"); //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 3? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

After Change:

import from NG_NAS_TypeDefs all; import from NR_ASP_Templates_SRB all; function fl_TC_8_1_1_3_7a_TestBody() runs on NR5GC_PTC { var template (value) RRCRelease_IEs.deprioritisationReq v_DeprioritisationReq; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var float v_T325 := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 300.0); var integer v_T325_InMillisec := float2int(v_T325 * 1000.0); var template NG_TrackingAreaId v_TAI; v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, -85, -85), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell3, -79, -79) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7a.3.2-1/2 f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message on NR Cell 1 including IE deprioritisationReq. v_DeprioritisationReq := cs_NR_DeprioritisationReq(nr, min5); //f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, -, v_DeprioritisationReq)); //MOVED f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, -, v_DeprioritisationReq)); //@siclog "Step 3" siclog@ //SS starts timer T325. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(int2oct(v_T325_InMillisec, 4), "T325")); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, -, v_DeprioritisationReq)); //@siclog "Step 4" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Timeout"); //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 3? v_TAI := f_LastVisited_TAI (nr_Cell1); f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell3,-,-,-,v_TAI); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm SM8450+SDX65

The Qualcomm SM8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_1_3_7a_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_1_3_7a_PIXIT.xml

The Qualcomm SM8450+SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_8_1_1_3_7a_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_8_1_1_3_7_PICS.txt

# References

| [1] | R5s211681 : NR5GC FR1 : Supporting inforamtion for addition of RRC test case 8.1.1.3.7a |
| --- | --- |