3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211692

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1879 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.5 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	8

2.5	Change 5	10

5.1	Change 6	16

6	Branches executed	17

7	Execution Log Files	17

7.1	HiSilicon Balong 5000	17

8	References	17

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.5 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.5.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_4_5_NR5GC_IMS2 |
| --- | --- |
| Reason for change | In step 8, the UE is in Limited Service state, so, the procedure for IMS emergency call without IMS registration should be called |
| Summary of change | Replace call to f_IMS_EmergencyCallAndRelease with f_IMS_EmergencyCallAndReleaseWithoutRegistration |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_5_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndRelease(); }

After Change

function f_TC_11_4_5_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndReleaseWithoutRegistration(); }

C

## Change 2

| Function name | f_IMS_EmergencyCallAndReleaseWithoutRegistration |
| --- | --- |
| Reason for change | Currently this function will send the trigger to NR PTC immediately, which will be handled by NR PTC as soon as RRCReconfigurationComplete is received following emergency PDU session establishment (after step 15 of 38.508 9.4.12). This means SS may send PDU SESSION MODIFICATION COMMAND (step 16) before UE has finished IP address allocation (step 1a1 of parallel behaviour 9.4.122.2-2). The EXCEPTION statements in 38.508 4.9.12.2.2-1 state that the IPv6 address allocation should take place before PDU SESSION MODIFICATIONCOMMAND is sent at step 16. |
| Summary of change | Remove f_IMS_IPCAN_SendCoOrdMsg(IPCAN), to be sent later in f_IMS_EmergencyCallSetup (see change 3) |
| TTCN module | IMS_NR5GC_Functions |
| MCC160 Comment |  |

Before Change

function f_IMS_EmergencyCallAndReleaseWithoutRegistration (INVITE_A_2_1_Context_Type p_Context := A_2_1_A6) runs on IMS_PTC { var INVITE_Request v_InviteRequest; // Tell NR to add voice now @sic R5s210515 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup(p_Context); // @sic R5s210518 sic@ // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function f_IMS_EmergencyCallAndReleaseWithoutRegistration (INVITE_A_2_1_Context_Type p_Context := A_2_1_A6) runs on IMS_PTC { var INVITE_Request v_InviteRequest; // Tell NR to add voice now @sic R5s210515 sic@ //REMOVE f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup(p_Context); // @sic R5s210518 sic@ // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

## Change 3

| Function name | f_IMS_EmergencyCallSetup |
| --- | --- |
| Reason for change | Currently this function will send the trigger to NR PTC immediately, which will be handled by NR PTC as soon as RRCReconfigurationComplete is received following emergency PDU session establishment (after step 15 of 38.508 9.4.12). This means SS may send PDU SESSION MODIFICATION COMMAND (step 16) before UE has finished IP address allocation (step 1a1 of parallel behaviour 9.4.122.2-2). The EXCEPTION statements in 38.508 4.9.12.2.2-1 state that the IPv6 address allocation should take place before PDU SESSION MODIFICATIONCOMMAND is sent at step 16. |
| Summary of change | Send trigger to NR PTC after receiving the INVITE, to make sure step 1a1 (IP address allocation) has been completed before step 16 (PDU SESSION MODICAITION COMMAND) is triggered. |
| TTCN module | IMS_Procedures_CallControl4G5G |
| MCC160 Comment |  |

Before Change

function f_IMS_EmergencyCallSetup(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request { /* p_Context .. A_2_1_A6 (INVITE for creating an emergency session in case of no registration) or * A_2_1_A7 (INVITE for creating an emergency session within an emergency registration) */ /* @sic R5s130988 change 6/7: sic@ * in case of A_2_1_A6 there has been no registration beforehand and therefore we don't know whether the UE uses IPv4 or IPv6 * => we cannot setup any address information before we have received the INVITE * => as the SDP message at step 1 contains address information we cannot build up the template in advance but need to match after the INVITE has been received */ /* @sic R5-140920: additional parameter p_OtherCalleeUri sic@ */ /* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */ var IMS_DATA_REQ v_IMS_DATA_REQ; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (present) SDP_Message v_SDP_Message_Step1; var template (value) SDP_Message v_SDP_Message_Step4; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; // Step 1. Receive INVITE for an Emergency call v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest(p_Context, -, p_OtherCalleeUri, -, p_GeolocationInfoAvailable); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); /* @sic R5s130988 change 6/7: has to be done after the INVITE has been received and the address information has been initialised even for A_2_1_A6 sic@ */ v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1, p_GeolocationInfoAvailable); /* @sic R5w140112 sic@ sic@ */ if (f_IMS_PTC_RanTypeIsNR5GC()) { // 34.229-5 A.6 v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfoEVS(v_SDP_Message); } else { // 34.229-5 C.22 v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfoAMR(v_SDP_Message); } ….. }

After Change

function f_IMS_EmergencyCallSetup(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request { /* p_Context .. A_2_1_A6 (INVITE for creating an emergency session in case of no registration) or * A_2_1_A7 (INVITE for creating an emergency session within an emergency registration) */ /* @sic R5s130988 change 6/7: sic@ * in case of A_2_1_A6 there has been no registration beforehand and therefore we don't know whether the UE uses IPv4 or IPv6 * => we cannot setup any address information before we have received the INVITE * => as the SDP message at step 1 contains address information we cannot build up the template in advance but need to match after the INVITE has been received */ /* @sic R5-140920: additional parameter p_OtherCalleeUri sic@ */ /* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */ var IMS_DATA_REQ v_IMS_DATA_REQ; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (present) SDP_Message v_SDP_Message_Step1; var template (value) SDP_Message v_SDP_Message_Step4; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; // Step 1. Receive INVITE for an Emergency call v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest(p_Context, -, p_OtherCalleeUri, -, p_GeolocationInfoAvailable); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); /* @sic R5s130988 change 6/7: has to be done after the INVITE has been received and the address information has been initialised even for A_2_1_A6 sic@ */ v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1, p_GeolocationInfoAvailable); /* @sic R5w140112 sic@ sic@ */ if (f_IMS_PTC_RanTypeIsNR5GC()) { // 34.229-5 A.6 v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfoEVS(v_SDP_Message); } else { // 34.229-5 C.22 v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfoAMR(v_SDP_Message); } ….. }

## Change 4

| Function name | f_TC_11_4_5_NR5GC |
| --- | --- |
| Reason for change | - According to TS38.508-1, NR Cell 2 and NR Cell 11 can’t be used at the same time. NR Cell 1 is the cell with another TAC, but this cell is only used in preamble, so there is no need to add a new cell in step 13, turn on NR Cell 1 can fullfil this requirement. - Following agreed prose CR R5-216201, the UE shall perform initial registration at the last step, so T3540 is not started |
| Summary of change | - Remove NR CELL 2 Note : An associated prose CR on 38.523-1 will be raised at RAN5#93 - Replace STATE_CONNECTED_3A_T3540 with STATE_CONNECTED_3A for postamble |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_5_NR5GC() runs on NR5GC_PTC { //5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell11); //Set power for the cells f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_4_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_CONNECTED_3A_T3540); }

After Change

function f_TC_11_4_5_NR5GC() runs on NR5GC_PTC { //5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //REMOVED f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell11); //Set power for the cells f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //REMOVED f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_4_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 5

| Function name | fl_TC_11_4_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | - According to TS38.508-1, NR Cell 2 and NR Cell 11 can’t be used at the same time. NR Cell 1 is the cell with another TAC, but this cell is only used in preamble, so there is no need to add a new cell in step 13, turn on NR Cell 1 can fullfil this requirement. - From agreed prose CR R5-216201: - In step 8, the UE is in Limited Service state, therefore, the Emergency call without IMS registration shall be called, instead of the procedure with IMS registration. - Some RRC message is wrong. - When UE is in 5GMM-REGISTERED.LIMITED-SERVICE state and initiate an emergency call, the UE will perform emergency registration, and then after the emergency call is released, it’s likely that the UE will either stay in 5GMM-REGISTERED state or enter 5GMM-DEREGISTERED state. If the UE tries to enter 5GMM-DEREGISTERED, it’s possible that the UE will send a DEREGISTRATION REQ or perform locally release. But current steps couldn’t cover the condition when the UE perform locally deregistration. |
| Summary of change | - Remove NR cell 2 Note : An associated prose CR on 38.523-1 will be raised at RAN5#93 - Re-implement steps 7-14 according to agreed prose CR R5-216201 |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

- Before Change

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //The SS configures:NR Cell 11 as "Serving cell" NR Cell 1 as "Non-Suitable "Off" cell" NR Cell 2 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 2" siclog@ //The UE transmits an RRCConnectionRequest message. //@siclog "Step 3" siclog@ //SS transmit an RRCConnectionSetup message. } //@siclog "Step 5" siclog@ //SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA))))); //@siclog "Step 6" siclog@ //The SS transmits an RRCConnectionRelease message. f_NR_RRCRelease(nr_Cell11); //@siclog "Steps 7-11" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? //Make the UE release the emergency call. (NOTE 1) //The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place //The SS transmits an RRCConnectionRelease message. fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber,"step 4");//Defaut value is px_EmergencyCallNumber f_NR_PreliminaryPass(__FILE__, __LINE__, "step 4"); //@siclog "Step 12" siclog@ //Check: Does the UE respond to paging (the Generic Test Procedure for no response to paging as specified in TS 38.508-1 [4], subclause 4.9.13 is performed)? f_NR5GC_PageNoResponse(nr_Cell11); //@siclog "Step 13" siclog@ //The SS configures:NR Cell 2 as "Serving cell" NR Cell 11 as "Non-Suitable "Off" cell" NR Cell 1 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 14" siclog@ //Check: Does the UE perform on NR Cell 2 the Registration procedure for mobility registration update as specified in TS 38.508-1 [4] subclause 4.9.5, 'connected without release'? f_NR5GC_MobilityRegistration (nr_Cell2, -, -, -, -, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); }

- After Change

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_Detach:= 10.0; var NR_SecurityParams_Type v_RRC_SecurityParams; //@siclog "Step 1" siclog@ //The SS configures:NR Cell 11 as "Serving cell" NR Cell 1 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //REMOVE f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 2" siclog@ //The UE transmits an RRCConnectionRequest message. //@siclog "Step 3" siclog@ //SS transmit an RRCConnectionSetup message. //@siclog "Step 4" siclog@ //The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. //Prose CR needed to merge step2-4 as "Steps 2 to 4 of the mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed. " v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Req failed"); } //@siclog "Step 5" siclog@ //SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA))))); //@siclog "Step 6" siclog@ //The SS transmits an RRCConnectionRelease message. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 7" siclog@ //Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 8" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? f_NR5GC_EmergencyCallLimitedService(nr_Cell11); //@siclog "Step 9-10" siclog@ //Make the UE release the emergency call. f_NR5GC_EmergencyCallRelease(nr_Cell11); f_NR_PreliminaryPass(__FILE__, __LINE__, "step 10"); //@siclog "Step 11" siclog@ //void //@siclog "Step 12" siclog@ t_Detach.start; //EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 12a1" siclog@ //The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) { //@siclog "Step 12a2" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); //@siclog "Step 12a3" siclog@ //Stop Timer. t_Detach.stop; } //@siclog "Step 12b1" siclog@ //Timer expires. [] t_Detach.timeout { } } //@siclog "Step 12A" siclog@ //SS releases the RRC connection f_NR_RRCRelease(nr_Cell11); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //@siclog "Step 12B" siclog@ //Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT, "12345"); //@sic R5-216198 sic@ //@siclog "Step 12C" siclog@ //Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NR Cell 11 within the next 10 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell11, 10.0); //@siclog "Step 13" siclog@ //The SS configures:NR Cell 1 as "Suitable cell" NR Cell 11 as "Serving cell". f_NR_SetCellPower (nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //@siclog "Step 14" siclog@ //Check: Does the UE perform on NR Cell 1 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? f_NR5GC_MobileInfo_Init(); f_NR5GC_Security_Init(); v_RRC_SecurityParams := f_NR_Security_Get(); v_RRC_SecurityParams.AS_Integrity.Algorithm := px_NR_IntegrityProtAlgorithm; v_RRC_SecurityParams.AS_Ciphering.Algorithm := px_NR_CipheringAlgorithm; f_NR_Security_Set(v_RRC_SecurityParams); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1,-,noRrcConnectionRelease, Initial_NoSecurity); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); }

## Change 6

| Function name | f_TC_11_4_5_NR5GC_IMS2 |
| --- | --- |
| Reason for change | In step 8, the UE is in Limited Service state, therefore, the Emergency call without IMS registration shall be called, instead of the procedure with IMS registration. As a result, NullIntegrityAlgorithm shall be used. |
| Summary of change | Add case name into f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed() |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { //@sic R5s210518 sic@ case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } } return false; }

After Change

function f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { //@sic R5s210518 sic@ case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } } return false; }

# Branches executed

The test case was executed on NR band n79 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_5_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_5_PIXIT.xml

# References

| [1] | R5s210876 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.5 |
| --- | --- |