3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211702

# Online, ,7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4655 | rev | - | Current version: | 17.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA_InitAS_KeyChaining_KeyRefresh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_AWS_EXT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When run test case with LTE band66, we found the ARFCN is 66886 which is larger than 65535, this will lead to ttcn error . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use ARFCN_ValueEUTRA_r9 instand of ARFCN_ValueEUTRA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execuction will result in run time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_EUTRA_InitAS_KeyChaining_KeyRefresh |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_EUTRA_InitAS_KeyChaining_KeyRefresh	2

- Corrections required

  - Correction to function f_EUTRA_InitAS_KeyChaining_KeyRefresh

| Functionname | f_EUTRA_InitAS_KeyChaining_KeyRefresh |
| --- | --- |
| Reason for change | When run test case with LTE band66, we found the ARFCN is 66886 which is larger than 65535, this will lead to ttcn error . |
| Summary of change | Use ARFCN_ValueEUTRA_r9 instand of ARFCN_ValueEUTRA |
| TTCN module | EUTRA_SecurityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_InitAS_KeyChaining_KeyRefresh(PhysCellId p_TargetPhysicalCellIdentity, ARFCN_ValueEUTRA p_EARFCN_DL, // @sic R5s100138 sic@ EUTRA_SecurityParams_Type p_Auth_Params, NextHopChainingCount p_NCC) return EUTRA_SecurityParams_Type { // @sic R5s100138 sic@ var EUTRA_SecurityParams_Type v_Auth_Params := p_Auth_Params; // initialise to received as parameter if (v_Auth_Params.NCC == p_NCC) { // Refresh of KeNB; Horizontal key derivation v_Auth_Params.KENB := f_EUTRA_NB_Authentication_S13(v_Auth_Params.KDF, v_Auth_Params.KENB, v_Auth_Params.NH, p_TargetPhysicalCellIdentity, p_EARFCN_DL); // @sic R5s100138, R5s160711 sic@ } else { //Chaining of KeNB; Vertical Key derivation while (v_Auth_Params.NCC != p_NCC) { if (v_Auth_Params.NCC != 0) { // NH for NCC =1 is already derived in initial AS Key derivation v_Auth_Params.NH := f_EUTRA_NB_Authentication_S12(v_Auth_Params.KDF, v_Auth_Params.KASME, v_Auth_Params.NH); //@sic R5s100496, R5s160711 sic@ // Derive NH and store in v_Auth_Params.KENB } v_Auth_Params.NCC := (v_Auth_Params.NCC + 1) mod 4; } v_Auth_Params.KENB := f_EUTRA_NB_Authentication_S13(v_Auth_Params.KDF, v_Auth_Params.KENB, v_Auth_Params.NH, p_TargetPhysicalCellIdentity, p_EARFCN_DL, true); // @sic R5s100138 R5s100496 R5s160711 sic@ //Derive KENB* which will be KENB from NH stored in v_Auth_Params.KENB } v_Auth_Params.AS_Ciphering.KRRCenc := f_EUTRA_NB_Authentication_S15(tsc_RRC_Enc_Alg, f_EUTRA_NB_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Integrity.KRRCint := f_EUTRA_NB_Authentication_S15(tsc_RRC_Int_Alg, fl_AS_IntegrityInfo_GetIntAlg(v_Auth_Params.AS_Integrity), // @sic R5-126020 sic@ v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Ciphering.KUPenc := f_EUTRA_NB_Authentication_S15 (tsc_UP_Enc_Alg, f_EUTRA_NB_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key; // Ciphering algorithm for UP is same as RRC return v_Auth_Params; };

After Change:

function f_EUTRA_InitAS_KeyChaining_KeyRefresh(PhysCellId p_TargetPhysicalCellIdentity, ARFCN_ValueEUTRA_r9 p_EARFCN_DL, // @sic R5s100138 sic@ EUTRA_SecurityParams_Type p_Auth_Params, NextHopChainingCount p_NCC) return EUTRA_SecurityParams_Type { // @sic R5s100138 sic@ var EUTRA_SecurityParams_Type v_Auth_Params := p_Auth_Params; // initialise to received as parameter if (v_Auth_Params.NCC == p_NCC) { // Refresh of KeNB; Horizontal key derivation v_Auth_Params.KENB := f_EUTRA_NB_Authentication_S13(v_Auth_Params.KDF, v_Auth_Params.KENB, v_Auth_Params.NH, p_TargetPhysicalCellIdentity, p_EARFCN_DL); // @sic R5s100138, R5s160711 sic@ } else { //Chaining of KeNB; Vertical Key derivation while (v_Auth_Params.NCC != p_NCC) { if (v_Auth_Params.NCC != 0) { // NH for NCC =1 is already derived in initial AS Key derivation v_Auth_Params.NH := f_EUTRA_NB_Authentication_S12(v_Auth_Params.KDF, v_Auth_Params.KASME, v_Auth_Params.NH); //@sic R5s100496, R5s160711 sic@ // Derive NH and store in v_Auth_Params.KENB } v_Auth_Params.NCC := (v_Auth_Params.NCC + 1) mod 4; } v_Auth_Params.KENB := f_EUTRA_NB_Authentication_S13(v_Auth_Params.KDF, v_Auth_Params.KENB, v_Auth_Params.NH, p_TargetPhysicalCellIdentity, p_EARFCN_DL, true); // @sic R5s100138 R5s100496 R5s160711 sic@ //Derive KENB* which will be KENB from NH stored in v_Auth_Params.KENB } v_Auth_Params.AS_Ciphering.KRRCenc := f_EUTRA_NB_Authentication_S15(tsc_RRC_Enc_Alg, f_EUTRA_NB_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Integrity.KRRCint := f_EUTRA_NB_Authentication_S15(tsc_RRC_Int_Alg, fl_AS_IntegrityInfo_GetIntAlg(v_Auth_Params.AS_Integrity), // @sic R5-126020 sic@ v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Ciphering.KUPenc := f_EUTRA_NB_Authentication_S15 (tsc_UP_Enc_Alg, f_EUTRA_NB_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.KENB, v_Auth_Params.KDF); // Derives KRRCenc key; // Ciphering algorithm for UP is same as RRC return v_Auth_Params; };