3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211705

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2221 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 14, TAC for NR Cell 3 is updated in the dedicatedSIB1-Delivery to 2 and in thye TTCN cell info store, but the broadcast SIB1 for NR Cell3 still has the TAC as 1. This can cause the UE to reject the REGISTRATION ACCEPT message as the TAC will not match the value broadcast on the cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update broadcast system information with the changed TAC for NR cell 3 before transmitting the handover command. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_1_2_TestBody |
| --- | --- |
| Reason for change | At step 14, TAC for NR Cell 3 is updated in the dedicatedSIB1-Delivery to 2 and in thye TTCN cell info store, but the broadcast SIB1 for NR Cell3 still has the TAC as 1. This can cause the UE to reject the REGISTRATION ACCEPT message as the TAC will not match the value broadcast on the cell. |
| Summary of change | Update broadcast system information with the changes TAC for cell 3 before transmitting the handover command. |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_4_1_2_TestBody () runs on NR5GC_PTC

{

……………………………

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 R5s210135 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

……………………………

}

After Change

…

import from NR5GC_SysinfoPaging all;

…

function fl_TC_8_1_4_1_2_TestBody () runs on NR5GC_PTC

{

…………………………

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

f_NR_ModifySysinfo(nr_Cell3, false);

f_NR_WaitModificationPeriods(nr_Cell3);

//@sic R5s201303 R5s210135 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

…………………….

}