3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211710

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2224 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR MAC test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case shall send MAC PDU DATA marked as retransmission in order to verify the test purpose. Therefore, to indicate data as a retransmission, TTCN shall configure the next DL assigment setting toggleNDI to false, then SS shall not toggle NDI in the next DL transmission. Additionally, SS shall not use automatically the HARQ id process used by test case procedure. Then, the HARQ id list of permited HARQ id is preconfigure in the test case preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MAC PDU sent in DL market as retransmission. HARQ id process used by the SS in automatic mode are preconfigured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A non-conformant UE may pass the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.ENDC, 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Corrections required to test case 7.1.1.9.1

  - Correction to function f_TC_7_1_1_9_1_NR5GC

| Function name | f_TC_7_1_1_9_1_NR5GC |
| --- | --- |
| Reason for change | Configure the SS to not use the HARQ id 5 and 1used in the test case body at steps 1/6 and 17/25. |
| Summary of change | List of allowed HARQ id process in the test case sent to the SS. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_9_1_NR5GC() runs on NR5GC_PTC { // MAC Reset var DRB_Identity v_DRBId ; var template (value) NR_RadioBearerList_Type v_SS_DRB_ConfigList; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //Init Cell parameters f_NR5GC_Init(NR_1); //Create and configure NR cell1 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //@sic R5-204418 sic@ v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); f_NR_AS_CipheringAlgorithm_Set(nea0); v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode v_SS_DRB_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; v_DRB_ToAddMod := cs_38508_NRDRB_ToAddMod( f_NR_GetSDAP_InDRBList(v_DRBId), v_DRBId, // DRB id cs_38508_PDCP_Config(ms60, len18bits, len18bits, omit)); // default is 18 bits from 38.508-1 v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)), -, -, -, -, f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1))); //Bring UE to State 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig {v_DRB_ToAddMod}, v_SS_DRB_ConfigList); f_NR_TestBody_Set(true); fl_TC_7_1_1_9_1_NR_TestBody(v_DRBId,v_SS_DRB_ConfigList);//@sic R5s200855 sic@ f_NR_TestBody_Set(false); //Postamble f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); //@sic R5s210138 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_7_1_1_9_1_NR5GC() runs on NR5GC_PTC { // MAC Reset var DRB_Identity v_DRBId ; var template (value) NR_RadioBearerList_Type v_SS_DRB_ConfigList; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }}; //Init Cell parameters f_NR5GC_Init(NR_1); ... f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig {v_DRB_ToAddMod}, v_SS_DRB_ConfigList); //Restrict the SS to use of the HARQ proccess id used by the test cases f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -))); f_NR_TestBody_Set(true); fl_TC_7_1_1_9_1_NR_TestBody(v_DRBId,v_SS_DRB_ConfigList);//@sic R5s200855 sic@ f_NR_TestBody_Set(false); //Postamble f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); //@sic R5s210138 sic@ f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

  - Correction to function f_TC_7_1_1_9_1_NR5GC

| Function name | f_TC_7_1_1_9_1_ENDC_NR |
| --- | --- |
| Reason for change | Same as 1.1. |
| Summary of change | Same as 1.1. |
| TTCN module | MAC_NR_ENDC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_9_1_ENDC_NR() runs on ENDC_NR_PTC { // MAC Reconfiguration and Reset var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (omit) DRB_ToAddModList v_DRB_ToAddModList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; Cell parameters f_ENDC_NR_Init(); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //@sic R5-204418 sic@ v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //set PDCP parameter discardTimer to 60ms v_DRB_ToAddModList[0] := cs_38508_DRB_ToAddMod(tsc_NR_DRB2, // DRB id cs_38508_PDCP_Config (-, len18bits, len18bits, omit)); // default is 18 bits from 38.508-1 //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList, omit, omit, v_DRB_ToAddModList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); fl_TC_7_1_1_9_1_ENDC_TestBody(tsc_NR_DRB2); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_9_1_ENDC_NR() runs on ENDC_NR_PTC { // MAC Reconfiguration and Reset var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (omit) DRB_ToAddModList v_DRB_ToAddModList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }}; //Init Cell parameters f_ENDC_NR_Init(); ... //Restrict the SS to use of the HARQ proccess id used by the test cases f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -))); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); fl_TC_7_1_1_9_1_ENDC_TestBody(tsc_NR_DRB2); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

  - New function f_NR_SS_CellConfig_DciDlInfo_Retransmission

| Function name | f_NR_SS_CellConfig_DciDlInfo_Retransmission |
| --- | --- |
| Reason for change | New function to reconfigure DL DCI in SS to not toggleNDI in order to signal that the next DL data is a retransmission |
| Summary of change | Set toggleNDI flag to false in the configuration of the next TBS sent in DL. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_NR_SS_CellConfig_DciDlInfo_Retransmission(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList, template (omit) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList2 := omit, template (omit) NR_HarqProcessConfig_Type p_HarqProcessConfig := omit, template (omit) boolean p_CnfFlag := omit) runs on NR_BASE_PTC { var template (value) NR_DciDlInfo_Type v_DciDlInfo := cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-, cs_NR_TransportBlockSchedulingDL_Explicit(p_TransportBlockRetransmissionList, p_TransportBlockRetransmissionList2), p_HarqProcessConfig)); f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ(p_NR_CellId, p_TimingInfo, v_DciDlInfo, p_CnfFlag)); };

  - New template cs_NR_TransportBlockRetransmission

| Function name | cs_NR_TransportBlockRetransmission |
| --- | --- |
| Reason for change | Same as 1.3 |
| Summary of change | New template adding toggleNDI as a parameter created. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

template (value) NR_TransportBlockRetransmissionList_Type cs_NR_TransportBlockRetransmission(template (value) NR_ImcsValue_Type p_ImcsValue, boolean p_ToggleNDI := false) := { cs_NR_TransportBlockSingleTransmission(p_ImcsValue,-,p_ToggleNDI) }; //-----

  - Correction to function f_TC_7_1_1_9_1_Step5to9A

| Function name | f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | In the test case before step 6, reconfigure DL DCI in SS to not toggle NDI when sending the next DL Data. |
| Summary of change | DL DCI configured not toggling NDI before sending DL Data. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); ... //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(5))); //@sic R5s200855 sic@ ...

After Change:

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var integer v_N_PRB := 12; // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ f_NR_SS_CellConfig_DciDlInfo_Retransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs)); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(5))); //@sic R5s200855 sic@

  - Correction to function f_TC_7_1_1_9_1_Step24to29

| Function name | f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | Same as 1.5 in step 25 |
| Summary of change | DL DCI configured not toggling NDI before sending DL Data. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@ //@siclog "Step 24A" siclog@ //The SS ignores scheduling requests and does not allocate any uplink grant. //@sic R5s201457 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 25" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 17. //PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 sic@ v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(3,crs_NR_PDCP_SDU_37B))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 26 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now); //Default Grant configuration. }

After Change:

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ var integer v_N_PRB := 12; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@ //@siclog "Step 24A" siclog@ //The SS ignores scheduling requests and does not allocate any uplink grant. //@sic R5s201457 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 25" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 17. //PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 sic@ f_NR_SS_CellConfig_DciDlInfo_Retransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs)); v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(3,crs_NR_PDCP_SDU_37B))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 26 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 27" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits @sic R5s201457 sic@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); // @siclog "Step 28" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now); //Default Grant configuration. }