3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211716

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2229 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. At step 18-28 there can be a mismatch in the INVITE contents because the expected CalleeUri is build from px_IMS_CalleeUri, but the voice call is initiated using the digit string "01234567890" 2. The UE will not initiate any voice/SMS procedure at step 17-28 and step 36-43 if the AT command is sent before the UE has read the updated system information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Pass px_IMS_CalleeUri into f_UT_RequestIMSCall at steps 13 and 16 2. uncomment the call to f_NR_WaitModificationPeriods before step 16 and 35 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

Note : Below changes are applied in addition to the changes of R5s211658

## Change 1

| Function name | fl_TC_11_3_1_TestBody() |
| --- | --- |
| Reason for change | - At step 18-28 there can be a mismatch in the INVITE contents because the expected CalleeUri is build from px_IMS_CalleeUri, but the voice call is initiated using the digit string "01234567890" - The UE will not initiate any voice/SMS procedure at step 17-28 and step 36-43 if the AT command is sent before the UE has read the updated system information. |
| Summary of change | - Pass px_IMS_CalleeUri into f_UT_RequestIMSCall at steps 13 and 16 - uncomment the call to f_NR_WaitModificationPeriods before step 16 and 35 |
| TTCN module | UAC_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 6.0; timer t_WaitTime; //@sic R5s211658 sic@ //@siclog "Step 12C" siclog@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT); //@siclog "Steps 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 6 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { } } //@siclog "Steps 15" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(6,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @sic R5-216192 sic@ //f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 16" siclog@ //Make the UE attempt another MO MTSI MMTEL Voice session. f_UT_RequestIMSCall(UT); //@siclog "Steps 17 -28" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall'? //Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 29-31" siclog@ //Step1 to step3 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place, f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall); //@siclog "Steps 31A" siclog@ //The SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); //@sic R5-216192 sic@ //@siclog "Steps 32" siclog@ //AT command to make the UE attempt to send SMS over IP f_UT_IMS_TriggerSMS(UT); //@siclog "Step 33" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-Data'? // @sic R5-210356 sic@ t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Data))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); } [] t_WaitTime.timeout {} } //@siclog "Steps 34" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); // @sic R5-216192 sic@ // f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 35" siclog@ //AT command to make the UE attempt to send another SMS over IP f_UT_IMS_TriggerSMS(UT); //@siclog "Steps 36-43" siclog@ ……. }

After Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 6.0; timer t_WaitTime; //@sic R5s211658 sic@ //@siclog "Step 12C" siclog@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 6 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } } //@siclog "Steps 15" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(6,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @sic R5-216192 sic@ f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 16" siclog@ //Make the UE attempt another MO MTSI MMTEL Voice session. f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 17 -28" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall'? //Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); // done on IMS PTC // Wait until need to add voice QoS f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); } [] t_WaitTime.timeout {} } //@siclog "Steps 34" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); // @sic R5-216192 sic@ f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 35" siclog@ //AT command to make the UE attempt to send another SMS over IP f_UT_IMS_TriggerSMS(UT); //@siclog "Steps 36-43" siclog@ ……. }