3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211718

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2230 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eNS NR5GC test case 9.1.10.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add eNS NR5GC test case 9.1.10.1 NSSAA / EAP message transport / Success |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to eNS NR5GC test case 9.1.10.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches Executed	6

4	Execution Log Files	6

4.1	Qualcomm SM 8450 + SDX65	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 9.1.10.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	9.1.10.1

ATS Version:	iWD-TTCN3-B2020-09_D21wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_9_1_10_1_TestBody |
| --- | --- |
| Reason for change | - As per the Prose, the REGISTRATION ACCEPT at step 12 “T3512” IE and allowedNSSAI should be provided #draft proseCR would be raised in RAN5#94-e meeting - Security parameters should be initialized with function f_NG_EAP_AuthenticationInit() in order to initialize Auth_K, without which a runtime error will be observed - The Configuration Update Indication in Configuration Update Command message should contain the Acknowledgement Requested and Registration Required IEs set to 1. - The REGISTRATION REQUEST at step 22 will be “mobility registration” not “periodic registration” # R5-216827 has been raised in RAN5#93-e meeting |
| Summary of change | - Removed T3512 from REGISTRATION ACCEPT at step 12. - Initialized the security parameters with function f_NG_EAP_AuthenticationInit() - Added Ack Requested and RED in Configuration Update Command - Removed the timer T3512 start and expiry from step 21 and changed the registration type to Mobility in step22. - Updated step 12 message contents following agreed prose CR R5-217939 |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_10_1_TestBody() runs on NR5GC_PTC { … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; … … //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, cs_GprsTimer3('37'O,'101'B,'00001'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI(omit, '01'O, '01'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI(omit, '01'O, '01'O, omit),?)))) -> value v_ReceivedAsp; if (not(f_NG_EAP_SuccessfulAuthenticationResponse(v_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.network_Slice_Specific_Authentication_Complete.eapMessage.eapMsg))){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EAP Authentication Failed"); } //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI(omit, '01'O, '01'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)), omit))))); //@siclog "Step 18" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, v_AllowedNSSAI)))); //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 21" siclog@ t_T3512.start; //WAit for the periodic timer to expire t_T3512.stop; //@siclog "Step 22" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic,-,-,-,-,cdr_NG_GMM_Cap_NSSAA,-,v_RequestedNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } //@siclog "Step 23" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00000011'B))))); //SS releases the RRC connection. //@siclog "Step 24" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); } //End of fl_TC_9_1_10_1_Body

After Change

function fl_TC_9_1_10_1_TestBody() runs on NR5GC_PTC { … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit)} }; … … //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_SecurityParams := f_NG_EAP_AuthenticationInit(v_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI(omit, '01'O, '01'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI(omit, '01'O, '01'O, omit),?)))) -> value v_ReceivedAsp; if (not(f_NG_EAP_SuccessfulAuthenticationResponse(v_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.network_Slice_Specific_Authentication_Complete.eapMessage.eapMsg))){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EAP Authentication Failed"); } v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI(omit, '01'O, '01'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)), omit))))); //@siclog "Step 18" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, v_AllowedNSSAI)))); //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 21" siclog@ //REMOVED t_T3512.start; //WAit for the periodic timer to expire //REMOVED t_T3512.stop; //@siclog "Step 22" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility,-,-,-,-,cdr_NG_GMM_Cap_NSSAA,-,v_RequestedNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } //@siclog "Step 23" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause (omit, '00000011'B))))); //SS releases the RRC connection. //@siclog "Step 24" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); } //End of fl_TC_9_1_10_1_Body

## Change 2

| Function name | cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT |
| --- | --- |
| Reason for change | The message type for Network Slice Specific Authentication Result is incorrect in the template |
| Summary of change | Corrected the message type for Network Slice Specific Authentication Result |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

template (value) NG_NAS_DL_Message_Type cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT (template (value) S_NSSAI_Type p_S_NSSAI, template (value) EAP_Message p_EAP) := { /* 24.501 cl. 8.2.33 */ network_Slice_Specific_Authentication_Result := { protocolDiscriminator := tsc_EPD_GMM, /* cl. 9.2 M V 1 */ spareHalfOctet := tsc_SpareHalfOctet, /* cl. 9.5 M V 1 */ securityHeaderType := tsc_SHT_NoSecurityProtection, messageType := tsc_MT_NG_NetworkSliceSpecificAuthenticationComplete, /* cl. 9.7 M V 1 */ sNSSAI := p_S_NSSAI, eapMessage := p_EAP } };

After Change

template (value) NG_NAS_DL_Message_Type cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT (template (value) S_NSSAI_Type p_S_NSSAI, template (value) EAP_Message p_EAP) := { /* 24.501 cl. 8.2.33 */ network_Slice_Specific_Authentication_Result := { protocolDiscriminator := tsc_EPD_GMM, /* cl. 9.2 M V 1 */ spareHalfOctet := tsc_SpareHalfOctet, /* cl. 9.5 M V 1 */ securityHeaderType := tsc_SHT_NoSecurityProtection, messageType := tsc_MT_NG_NetworkSliceSpecificAuthenticationResult, /* cl. 9.7 M V 1 */ sNSSAI := p_S_NSSAI, eapMessage := p_EAP } };

# Branches Executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

# Execution Log Files

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_9_1_10_1_LOG.xml

Keysight\TC_9_1_10_1_LOG.html

- PICS/PIXIT parameter file:

Anritsu\ TC_9_1_10_1_PIXIT.txt

Keysight\ TC_9_1_10_1_PIXIT.xml

# References

| [1] | R5s211719 - Supporting information for addition of eNS NR5GC test case 9.1.10.1 in FR1 |
| --- | --- |