3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210042

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1484 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The msg3 at steps 3-6 is not received because no UL grant is configured in NR cell 12 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Start UL Grant allocation for NR CELL 12 at the beginning of Step3-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | The msg3 at steps 3-6 is not received because no UL grant is configured in NR cell 12 |
| Summary of change | start UL Grant allocation for NR CELL 12 at the beginning of Step3-6 |
| TTCN module | Inactive_PLMNSelection_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; // Power levels on FR2 are FFS var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_TimerValue1 := 60.0; timer t_WaitTime; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell13, -78, -78) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12? f_NR_RRC_ResumeRequest_Def(nr_Cell12,-,mo_Signalling); // @sic R5s200892 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Steps 3-6" siclog@ // Steps 2 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 with condition MOBILITY are performed // Note 1: The REGISTRATION REQUEST is accepted with the PLMN of NR Cell 1 listed as an Equivalent PLMN f_NR_RRCSetup_Def (nr_Cell12); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)))); // Table 6.4.1.2.3.3-4: REGISTRATION ACCEPT for NR Cell 12 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // step 15 of the Registration procedure f_NR5GC_Registration_Complete(nr_Cell12); // @siclog "Step 7" siclog@ // The SS transmits an RRCRelease message with suspend. f_NR_RRC_InactiveDef(nr_Cell12, f_NR_CellInfo_GetPLMN (nr_Cell12)); // @siclog "Step 8" siclog@ // Check: Does the UE send an RRCResumeRequest on NR Cell 13 and NR Cell 1 within 60s? t_WaitTime.start(v_TimerValue1); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND((nr_Cell1, nr_Cell13), cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.4.1.2 Step 8"); } [] t_WaitTime.timeout{} } // @siclog "Step 9" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 10" siclog@ // Set UE to Automatic PLMN selection mode // Note 2: Step 10 is to ensure UE is set back to automatic PLMN selection mode for the next test case f_UT_AutomaticPLMN_Select(UT); }

After Change

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; // Power levels on FR2 are FFS var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_TimerValue1 := 60.0; timer t_WaitTime; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell13, -78, -78) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12? f_NR_RRC_ResumeRequest_Def(nr_Cell12,-,mo_Signalling); // @sic R5s200892 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Steps 3-6" siclog@ // Steps 2 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 with condition MOBILITY are performed // Note 1: The REGISTRATION REQUEST is accepted with the PLMN of NR Cell 1 listed as an Equivalent PLMN f_NR_ULGrantConfiguration_Start(nr_Cell12); f_NR_RRCSetup_Def (nr_Cell12); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)))); // Table 6.4.1.2.3.3-4: REGISTRATION ACCEPT for NR Cell 12 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // step 15 of the Registration procedure f_NR5GC_Registration_Complete(nr_Cell12); // @siclog "Step 7" siclog@ // The SS transmits an RRCRelease message with suspend. f_NR_RRC_InactiveDef(nr_Cell12, f_NR_CellInfo_GetPLMN (nr_Cell12)); // @siclog "Step 8" siclog@ // Check: Does the UE send an RRCResumeRequest on NR Cell 13 and NR Cell 1 within 60s? t_WaitTime.start(v_TimerValue1); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND((nr_Cell1, nr_Cell13), cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.4.1.2 Step 8"); } [] t_WaitTime.timeout{} } // @siclog "Step 9" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 10" siclog@ // Set UE to Automatic PLMN selection mode // Note 2: Step 10 is to ensure UE is set back to automatic PLMN selection mode for the next test case f_UT_AutomaticPLMN_Select(UT); }