3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210044

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1486 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.3.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Expected MeasurementReport at step7 does not align with R5-204437 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the expected measurement quantity results in MeasurementReport at step7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_3_2_TestBody |
| --- | --- |
| Reason for change | Expected MeasurementReport at step7 does not align with R5-204437 |
| Summary of change | Correct the expected measuremtn quantity reuslts in MeasurementReport at step7 |
| TTCN module | RRC_Measurement_NR5GC_NR |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_3_3_2_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -85; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 3; //@sic R5s200713 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 77; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 56; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var PLMN_Identity v_PLMN_Identity; var B16_Type v_TrackingAreaCode_Eutra_Epc; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) CellGroupConfig v_CellGroupConfig; //var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (present) CGI_InfoEUTRA v_NR_MeasResultEUTRA_CGI_InfoEUTRA; var template (value) CellAccessRelatedInfo_EUTRA_EPC v_CellAccessRelatedInfo_EUTRA_EPC; var template (value) FreqBandIndicatorEUTRA v_FreqBandIndicatorEUTRA; ……. //@siclog "Step 7" siclog@ //Check: Does the UE transmit a MeasurementReport message with CGI of E-UTRA Cell 1 within 1 sec? f_NR_CheckReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, 1.0, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_None, v_NR_MeasResultEUTRA_CGI_InfoEUTRA)})); }//End of f_TC_8_1_3_3_2_TestBody()

After Change

function f_TC_8_1_3_3_2_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -85; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 3; //@sic R5s200713 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 77; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 56; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var PLMN_Identity v_PLMN_Identity; var B16_Type v_TrackingAreaCode_Eutra_Epc; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) CellGroupConfig v_CellGroupConfig; //var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (present) CGI_InfoEUTRA v_NR_MeasResultEUTRA_CGI_InfoEUTRA; var template (value) CellAccessRelatedInfo_EUTRA_EPC v_CellAccessRelatedInfo_EUTRA_EPC; var template (value) FreqBandIndicatorEUTRA v_FreqBandIndicatorEUTRA; ……. //@siclog "Step 7" siclog@ //Check: Does the UE transmit a MeasurementReport message with CGI of E-UTRA Cell 1 within 1 sec? f_NR_CheckReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, 1.0, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_None, v_NR_MeasResultEUTRA_CGI_InfoEUTRA , cr_NR_MeasQuantityResults_RsrpRsrq); //@sic R5-204437sic@ }//